3GPP TSG-RAN WG2 Meeting #131	R2-2505895

Bengaluru, India, 25 - 29 August 2025

Title: 	Discussion on MBS broadcast over NTN

Source: 	Huawei, HiSilicon

Agenda Item:	8.8.4

Document for:	Discussion and decision

# 1	Introduction

One of the objectives for Rel-19 NTN WI [1] is the support of broadcast service over NTN:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this contribution, we provide our views on the remaining issues, regarding the provision of ISAs by different means and the MCCH acquisition skipping.

# 2	Discussion

## 2.1	The ISA or ISAs

In the draft TS 38.300 CR, there is the following:

### 16.14.Y Support of ISA for MBS Broadcast in NTN

An ISA can be signaled for location-dependent services. The ISA is a geographical information that can cover an area larger than one NTN cell (i.e. serving and neighbour cells) or portions thereof. If ISAs are defined for an MBS broadcast service, they are broadcasted as a list in SIBXX and also indicated as part of the MCCH with the ISA identifier.

The NTN-capable UE supporting the MBS broadcast may initiate the broadcast MRB establishment procedure when it is inside the ISA. The UE may initiate the broadcast MRB release procedure when UE leaves the ISA (see clause 5.9.3 of TS 38.331 [12]).

NOTE 1: When using ISA(s) for MBS broadcast service reception or MBS service continuity, it is up to UE implementation how to determine if it is in the ISA(s) of MBS broadcast service or not.

According to the green highlighted sentence, there can be multiple ISAs for an MBMS broadcast service i.e., the ISA is not "the area in which an MBS broadcast service is intended to be received", it is "one area in which an MBS broadcast service is intended to be received".

In TS 23.247, there is

Broadcast service area: The area within which data of one or multiple Broadcast MBS session(s) are sent.

In the TS 38.331 running CR, there is:

### 5.9.3	Broadcast MRB configuration

#### 5.9.3.1	General

The broadcast MRB configuration procedure is used by the UE to configure PDCP, RLC, MAC and the physical layer upon starting and/or stopping to receive a broadcast MRB transmitted on MTCH, or upon modification of a configuration of a broadcast MRB received by the UE. The procedure applies to MBS capable UEs that are interested to receive or that are receiving an MBS broadcast service that are in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED with an active BWP with common search space configured by searchSpaceMTCH or searchSpaceMCCH.

NOTE:	How to perform a modification of a broadcast MRB which is already configured in the UE is left to UE implementation.

#### 5.9.3.2	Initiation

The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated e.g. upon start of the MBS session, upon entering a cell providing an MBS broadcast service which is not associated with an ISA and the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in, upon entering the ISA associated with a MBS broadcast service the UE is interested in.

The UE applies the broadcast MRB release procedure to stop receiving a session of an MBS broadcast service. The procedure may be initiated e.g. upon stop of the MBS session, upon leaving the cell broadcasting the MBS service the UE is interested in, upon losing interest in the MBS service, when capability limitations start inhibiting reception of the concerned service, upon leaving the ISA associated with a MBS broadcast service the UE is interested in.

In the above, "entering/leaving the ISA associated with a MBS broadcast service" seems to mean "entering/leaving the area in which an MBS broadcast service is intended to be received". However, this is not consistent with how ISA is defined in the TS 38.300 CR.

In the 38.331 CR, there is the following definition:

-- ASN1START

-- TAG-SIBXX-START

SIBXX-r19 ::=                     SEQUENCE {

intendedServiceAreaList-r19     IntendedServiceAreaList-r19      OPTIONAL,  -- Need R

lateNonCriticalExtension        OCTET STRING                     OPTIONAL,

...

}

IntendedServiceAreaList-r19 ::=   SEQUENCE (SIZE (1.. maxNrofMBS-Area-r19)) OF IntendedServiceAreaInfo-r19

IntendedServiceAreaInfo-r19 ::=   SEQUENCE {

intendedServiceAreaId-r19       MBS-IntendedAreaID-r19,

areaCoordinates-r19             CHOICE {

polygonArea-r19                   OCTET STRING,

circleArea-r19                    SEQUENCE {

referenceLocation-r19           ReferenceLocation-r17,

distanceRadius-r19              INTEGER(0..65535)

}

}

}

MBS-IntendedAreaID-r19  ::= INTEGER (1.. maxNrofMBS-Area-r19)

-- TAG-SIBXX-STOP

-- ASN1STOP

SIBXX field descriptions

intendedServiceAreaList Contains a list of intended service areas, each associated to one or more MBS broadcast services provided in an NTN cell.

polygonArea Parameter type Polygon defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

From this, it can be understood that "an ISA" is what is defined by a polygonArea or a circleArea. This is somehow compatible with the definition in the TS 38.300 CR.

This further raises the question of the meaning of "upon entering a cell providing an MBS broadcast service which is not associated with an ISA". Is the association with the cell (e.g., does the cell not broadcast SIBxx) or is the association with the MBS service?

Observation 1: In some places of the TS 38.331 running CR, "ISA" is used to mean "the area in which an MBS broadcast service is intended to be received", which consists of a list of cells, TAIs, polygons and circles, and in other places, it means one polygon or circle as defined in one IntendedServiceAreaInfo-r19 in SIBxx.

Proposal 1: Clarify whether "ISA" refers to the whole area in which an MBS broadcast service is to be received or to one polygon/circle defined in SIBxx of one cell, and update the TS 38.331 CR accordingly.

## 2.2	MBS interest indication and ISAs

The MBMS interest indication initiation procedure is defined as:

#### 5.9.4.2	Initiation

An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

At RAN23129bis, it was agreed that

R2-2503057	Report of [AT129bis][304][R19 NR NTN] Service continuity	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1:

WA: UE may prioritize the frequency for an interested service when UE can receive the service on the frequency and the UE is in intended service area associated with that frequency/service. Otherwise, UE may no longer prioritize the frequency.

FFS whether and how the association between intended service area and frequencies is acquired.

-	Nokia and Ericsson could be ok if we remove frequency

-	ZTE suggests to change to “the service provided in the frequency”

UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency.

- FFS whether and how the association between intended service area and frequencies is acquired.

This agreement formally is only about prioritization of an MBS frequency i.e., in RRC_IDLE.

However, the purpose of the MBS interest indication is exactly the same like the purpose of prioritization: steer the UE to a particular frequency, so that the UE can receive the MBS service of interest.

The motivation to "deprioritize a frequency" if the UE is not in the area in which the service is intended to be received, while the service is still broadcast, is to avoid gathering many UEs on the same frequency unnecessarily, which could affect RACH load and setup delay. For UEs in RRC_CONNECTED, the network is in control of which frequency the UE uses, and having too many UEs on the same frequency, because e.g., they need this in order to receive an MBS service of interest, is even more problematic than having many UEs in RRC_IDLE on the same frequency.

Therefore, there is even a stronger motivation to allow the UE to indicate interest in a frequency only when the UE enters the area in which the service is intended to be received and to indicate no more interest when the UE leaves the area in which the service is intended to be received.

Proposal 2: The UE may also initiate the MBS interest indication procedure when entering the area in which an MBS broadcast service of interest is intended to be received, and when leaving the area in which an MBS broadcast service of interest is intended to be received.

## 2.3	ISAs in new SIB and service announcement

The service announcement defined in TS 23.247 may include the MBS service area. The MBS service area used by AF can be Cell ID list, TAI list, geographical area information or civic address information.

For local MBS service, the Service Announcement may include the MBS service area. The MBS service area used by AF can be Cell ID list, TAI list, geographical area information or civic address information. <omitted> If the MBS Session is for broadcast, the Service Announcement may include the MBS FSA ID(s) and optional frequency information associated with the broadcast MBS session and may also include an indication that the MBS Session is intended for RedCap UEs, for non-RedCap UEs or both for RedCap UEs and non-RedCap UEs as defined in TS 26.502 [18].

In accordance with TS 26.502, the MBS service area currently only support the TAI list and/or cell list, as indicated as below:

Table 4.5.9-1: Mapping of baseline parameters to MBS Session Context parameters MBS Session Context parameter; Source; Clause; Source parameter. State; MBS Distribution Session.; - 4.5.6; State.. Source-Specific Multicast (SSM)IP address; MBS Distribution Session.; - 4.5.6; MBS Session Identifier. TMGI. MBS Service Area; MBS Distribution Session.; - 4.5.6; Target service area (see NOTE 2). Area Session Identifier; Assigned by MB-SMF.; - 4.5.6; Location-dependent service flag. MBS Frequency Selection Area (FSA) ID (see NOTE 1); MBS Distribution Session.; - 4.5.6; MBS Frequency Selection Area. <omitted>. NOTE 1: Applicable to Broadcast MBS Session only. NOTE 2: Target service area is signalled to the MBSF at reference point Nmb10/Nmb5 in the form of a Tracking Area Identifier (TAI) list and/or Cell ID list. External-facing target service area identification at reference point N33 is translated by the NEF into the appropriate form(s) as required..

An incoming LS from SA4[2] confirms to support the geographical information, i.e. in form of a set of polygon(s) and circle(s) as defined by RAN2, leveraging the Service announcement message. The previous RAN2 progress also has supported the provision of ISAs via new SIBs for UE interested in receiving MBS broadcast service. Therefore, the UE may be able to receive the ISAs for a given MBS session not only from new SIB but also from service announcement. The conflict may occur when the UE receives different ISAs by means of the MBSBroadcastConfiguration message in combination with the new SIB, and the service announcement.

Proposal 3: Discuss whether ISAs can be provided in MCCH for an MBS service for which ISAs are provided in the service announcement.

## 2.4	Skip MCCH acquisition

The RAN2 #129bis meeting agreed that:

If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

The UE gets to know the ISAs applicable for an MBS session through the MCCH information (MBSBroadcastConfiguration message provided via MCCH), so this seems to imply that the UE needs to first acquire MCCH and information for all the MBS broadcast services of interest, and then, if the UE is not in any of the applicable ISA, the UE can skip monitoring MCCH.

However, an NTN cell can be large and it is possible that all the ISAs of all the MBS broadcast service only cover a subset of the cell. In this case, if the UE reads SIBx and sees that it is not in any of the ISAs, it does not need to acquire MCCH. However, if the cell provides an MBS broadcast service that is relevant for the whole cell coverage, the UE should still acquire MCCH to determine whether ISAs are applicable to the service (and which ones). This means that there should be an indication in SIBx whether UEs that do not know the applicable ISA(s) for their MBS service(s) of interest can skip MCCH acquisition is they are not in any ISA in SIBx.

Proposal 4a: SIBx indicates (e.g., 1-bit) whether the cell may broadcast MBS broadcast services relevant in the whole cell coverage.

Proposal 4b: If SIBx does not indicate that the cell may transmit MBS broadcast services relevant in the whole cell coverage, a UE interested in receiving one or more MBS broadcast service but not in any ISA of the cell may skip reading MCCH.

If the UE has read MCCH and acquired the list of ISAs applicable to the MBS broadcast services of interest, and the UE is not located in any of the ISAs, the UE may skip reading MCCH.

The MCCH change notification message can be used to indicate to the UE to perform MCCH information re-acquisition i.e., acquire the updated MBSBroadcastConfiguration message. One possible case is that the ISAs applicable to an ongoing MBS broadcast service have changed, so the UE that was not in any ISA of an MBS service of interest might now be in such an ISA. Such a modification may require updating SIBx but it is also possible that no such update occurs, only MCCH information is updated. So it is necessary that the UE monitors MCCH change notification and re-acquire MCCH if needed.

Proposal 5a: The UE that has acquired MCCH and is out of the ISAs of the MBS service of interest shall monitor MCCH change notification and re-acquire MCCH again.

The MCCH change notification can indicate whether a new MBS service has started or whether there is a modification of MCCH information other than the change caused by start of new MBS service(s). If the bit indicating "modification of MCCH information other than the change caused by start of new MBS service(s)" is used when the ISAs of a service are modified, for any modification of MCCH information, the UEs currently out of any ISA of the MBS services of interest will re-acquire MCCH at any modification of MCCH information other than the change caused by start of new MBS service(s), which such modifications are normally only useful for UEs currently receiving MBS services. Such modifications may be rather frequent, so that the UE power consumption may be unnecessarily increased.

In order to mitigate such issue, we can consider alternatives such as:

-	upon MCCH change notification, the UE only re-acquires MCCH when the bit indicating "service start" is set ;

-	upon MCCH change notification, the UE only re-acquires MCCH when a new bit indicating "change of ISAs of ongoing MBS service" is set;

-	the UE only monitors MCCH change notification every x MCCH modification periods, where x is provided to the UE e.g., in SIBx;

Proposal 5b: Discuss how to reduce unnecessary MCCH re-acquisition for the UE that has acquired MCCH and is out of the ISAs of the MBS services of interest, such as:

alt. 1)		upon MCCH change notification, the UE only re-acquire MCCH when the bit indicating "service start" is set ;

alt. 2)	upon MCCH change notification, the UE only re-acquires MCCH when a new bit indicating "change of ISAs of ongoing MBS service" is set;

alt. 3)	the UE only monitors MCCH change notification every x MCCH modification periods, where x is provided to the UE e.g., in SIBx.

# 3	Conclusion

In this document, we discussed the support of broadcast service via NR NTN, and proposed the following:

The ISA or ISAs

Proposal 1: Clarify whether "ISA" refers to the whole area in which an MBS broadcast service is to be received or to one polygon/circle defined in SIBxx of one cell, and update the TS 38.331 CR accordingly.

MBS interest indication and ISAs

Proposal 2: The UE may also initiate the MBS interest indication procedure when entering the area in which an MBS broadcast service of interest is intended to be received, and when leaving the area in which an MBS broadcast service of interest is intended to be received.

ISAs in MCCH and service announcement

Proposal 3: Discuss whether ISAs can be provided in MCCH for an MBS service for which ISAs are provided in the service announcement.

Skip MCCH acquisition

Proposal 4a: SIBx indicates (e.g., 1-bit) whether the cell may broadcast MBS broadcast services relevant in the whole cell coverage.

Proposal 4b: If SIBx does not indicate that the cell may transmit MBS broadcast services relevant in the whole cell coverage, a UE interested in receiving one or more MBS broadcast service but not in any ISA of the cell may skip reading MCCH.

Proposal 5a: The UE that has acquired MCCH and is out of the ISAs of the MBS service of interest shall monitor MCCH change notification and re-acquire MCCH again.

Proposal 5b: Discuss how to reduce unnecessary MCCH re-acquisition for the UE that has acquired MCCH and is out of the ISAs of the MBS services of interest, such as:

alt. 1)		upon MCCH change notification, the UE only re-acquire MCCH when the bit indicating "service start" is set ;

alt. 2)	upon MCCH change notification, the UE only re-acquires MCCH when a new bit indicating "change of ISAs of ongoing MBS service" is set;

alt. 3)	the UE only monitors MCCH change notification every x MCCH modification periods, where x is provided to the UE e.g., in SIBx.

# 4	References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- S4-251099, Reply on Inclusion of NTN intended service area in the Service Announcement, SA4