3GPP TSG-RAN WG2 Meeting #131	R2-2505917

Bangalore, India, 25th – 29th Aug, 2025

Agenda item:	8.9.3

Source:	Samsung

Title:	On open issues for CB-Msg3-EDT

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#107 the latest WID for IoT NTN Phase 3 was agreed [1], where Capacity enhancements for uplink:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In this contribution, we provide some discussions on the procedures and open issues on the objective of Support of Capacity enhancements for the uplink.

# Discussion

## CB-RNTI formula

WA: the formula for RNTI for mMsg4 monitoring is:

- RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id.

- •	X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT,

- •	Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19,

- •	Y is ceil (Msg4_WS/Msg3_WP),

- •	CE_level is the CE level, 0 <= CE_level < 3

- •	carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0,

- 0 <= carrier_id < 16

- Can come back to check if the NW can also simply configure RNTI = X

The value of X is 4097 for NB-IoT and 2401 for eMTC

the value of Msg4_WS is the maximum Msg4 window size

the value of Msg3_WP is the minimum Msg3 window periodicity

Proposal 1: RAN2 to confirm the working assumption on CB-RNTI formula [10/11].

Proposal 2: For the agreed CB-RNTI formula from P1, RAN2 to discuss

- Whether to define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity].

- Whether to define Y = 16.

- Whether starting value X is optionally configurable.

First of all, we do not think that it is reasonable to use a single RNTI. This may work in the case of only a single CE level, but if there are multiple CE levels on a single carrier, then there will very likely be unintended collisions.

Observation 1: Single RNTI is not workable solution as there may be multiple CE levels on a carrier, leading to RNTI collisions for different CE levels.

The purpose of Y is to ensure that if the Msg4 windows overlap, then there will be a different CB Msg3 RNTI for the Msg4 windows.

The original proposal on defining Y was that Y is defined as the ceiling of (Msg4_WS/Msg3_WP), where the Msg4_WS is the size of the msg4 window and Msg3_WP is the Msg3 window periodicity. It was found that the main issue with this approach is that the Msg4 window and Msg3 transmission window may vary with different CE levels, which means that the Y will be different for the CE levels. This can result in RNTI being the same for different CE levels, which is clearly not the intention.

The second proposal was that Y is that MSG4_WS is the maximum size of the Msg4 window and Msg3_WP is the minimum size of the Msg3_WP. However, the issue here is that the maximum size of the Msg4_WS can be very long, while Msg3_WP can be very short. This can cause Y to be very large, in range of 100, even up 1000. The maximum size of the CB-RNTI is thus (for NB-IoT) max_RNTI = 4097 + Y-1 + Y*3 + 3*Y*16 = 4097 + 52*Y – 1, which for Y=100 equals 9296 and for Y=400 equals 24896, which means that an incredible amount of RNTIs would be used.

Instead we think it would be more reasonable to define a Y which is based on a reasonable amount of overlap of Msg4 for a single CE level. For instance, values such as 4, 8, 16 and 32 are much more reasonable to assume.

Proposal 1: RAN2 to confirm the working assumption on CB-RNTI formula.

- With Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity].

- With Y set to a value for reasonable amount of Msg4 window overlap, such as Y = 32, 16, 8 or 4. Y is not configurable

- Starting value X is not configurable.

## TA report in CB-Msg3 EDT

In the RRC e-mail discussion, one issue on reporting the TA report (TAR MAC CE) was discussed.

The field description indicates the following:

ta-Report When this field is included in SystemInformationBlockType2, it indicates reporting of timing advance is enabled during Random Access due to RRC connection establishment, RRC connection resume or RRC connection reestablishment. When this field is included in MobilityControlInfo, it indicates TA reporting is enabled during Random Access due to handover, see TS 36.321 [6], clause 5.4.9.

However, according to the RRC procedures, the TA report is always included as long as ta-Report is configured, as seen in 36.331. Important to note that 5.3.3.2, which triggers the TA report, is initiated for all RRC establishment, RRC resume, EDT etc.

----------------- 36.331 V18.6.0 -----------------

5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment or resume of an RRC connection while the UE is in RRC_IDLE or when upper layers request resume of an RRC connection or RRC layer requests resume of an RRC connection for, e.g. RNAU or reception of RAN paging while the UE is in RRC_INACTIVE.

. . .

Except for NB-IoT, upon initiating the procedure, if connected to EPC or 5GC, the UE shall:

. . .

1>	if UE supports timing advance reporting and ta-Report is included in SystemInformationBlockType2:

2>	instruct the associated MAC entity to trigger Timing Advance reporting;

----------------- 36.331 V18.6.0 -----------------

So clearly there is a misalignment on whether TAR MAC CE shall be sent for CB-Msg3-EDT or not.

Therefore, RAN2 needs to discuss what should be the behaviour of TA report for CB-Msg3-EDT:

Proposal 2: RAN2 to discuss options on TA report for CB-Msg3 EDT.

- TA report is not included for CB-Msg3-EDT – RAN2 clarifies the RRC procedures.

- TA report is included in the CB-Msg3-EDT according to the RRC procedures – RAN2 clarifies the ta-Report field description

- New configuration to configure TA report for CB-Msg3-EDT

## Capturing ending CB-Msg3 EDT procedure with CRID

In the RRC open issue list, it has been discussed whether the following agreement needs to be captured in  RRC:

3.	A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution.

It should be self-evident that if an RRC procedure shall be finished, then there needs to be RRC procedures capturing the RRC procedure being finished. Otherwise, upper layers will not be indicated of the RRC procedures being finished, timers such as T300 will continue and expire.

Proposal 3: RRC procedures need to capture CRID as complete response for the CB-Msg3 EDT in CP solution.

It was also discussed whether any of the CB-Msg3 EDT features can be introduced to normal EDT or normal RACH procedures. We think that probably the only feature that can be reasonable to introduce would be the feature to have CRID finish the CP EDT procedures.

Proposal 4: For introducing enhancements from CB-Msg3 EDT to 4-step EDT, the only viable feature would be receiving CRID as complete response.

## CQI reporting in Msg3

Proposal 12: RAN2 to discuss whether to support CQI reporting in anchor carrier and/or non-anchor in CB-Msg3-EDT procedure. If yes, RAN2 to discuss how does the UE prepare DL channel quality measurement result in CB-Msg3 without Msg1/Msg2.

CQI reporting in Msg3 during RRC resume and EDT is supported for both eMTC and NB-IoT. This is useful in order to schedule Msg4 and to assess the coverage situation if the UE is moved to RRC_CONNECTED. In NB-IoT it is supported by including the cqi-NPDCCH in RRCConnectionResumeRequest and in RRCEarlyDataRequest if cqi-Reporting is configured in SIB2. For eMTC it is supported by including the DCQR and AS RAI MAC CE which is configured by the mpdcch-CQI-Reporting in SIB2. For these cases, the CQI is measured on Msg2 or before the random access procedure.

CQI reporting may also be useful during the contention-based Msg3, similar to how it is already useful during EDT. In particular it may be useful for scheduling Msg4 to multiple UEs, as it can indicate whether two UEs sending Msg4 during the same CB-Msg3 opportunity operate with different CQI. For NB-IoT it will not occupy further bits as the bits are already occupied in the RRCConnectionResumeRequest and RRCEarlyDataRequest, however for eMTC it may occupy further bits as it is included in a MAC CE.

As explained in 36.133, the CQI can be measured on either before the random access procedure (T1) or on based on Msg2 (T2). T1 is supported only for the anchor carrier while T2 is supported on all carriers. Obviously Msg2 is not sent during the CB-Msg3 EDT procedures, so T2 cannot be used for CB-Msg3 EDT.

Proposal 5: RAN2 to confirm CQI reporting (in RRCEarlyData and DCQR and AS RAI MAC CE) in CB-Msg3 for CB-Msg3-EDT (for CP/UP eMTC and NB-IoT) is supported for anchor carrier.

## Other

One open issue is the following:

Proposal 4: RAN2 to choose between 3ms or 0ms processing time when determine the Msg4 monitoring starts.

It is a waste of UE energy if the UEs start monitoring without taking eNB processing into account. Furthermore, current RACH procedures already have processing time taken into account, so it would be odd to not have such for a procedure that surely requires more eNB processing. Therefore we propose:

Proposal 6: 3 ms processing is introduced for Msg4 monitoring.

Another open issue is the following:

Proposal 7: RAN2 to further discuss whether to support multiple TBS.

- Option 1 [4/11] – Support multiple TBS per CE level

- Option 2 [7/11] – One TBS per CE level is enough

  - Note that 3 companies think R1 input is needed

Multiple TBS per CE level would very much complicate the design, so we propose to go with one TBS per CE level.

Proposal 7: Multiple TBS per CE level is not supported.

# Conclusion

In this contribution we discussion further issues on contention-based Msg3:

Observation 1: Single RNTI is not workable solution as there may be multiple CE levels on a carrier, leading to RNTI collisions for different CE levels.

Proposal 1: RAN2 to confirm the working assumption on CB-RNTI formula.

- With Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity].

- With Y set to a value for reasonable amount of Msg4 window overlap, such as Y = 32, 16, 8 or 4. Y is not configurable

- Starting value X is not configurable.

Proposal 2: RAN2 to discuss options on TA report for CB-Msg3 EDT.

- TA report is not included for CB-Msg3-EDT – RAN2 clarifies the RRC procedures.

- TA report is included in the CB-Msg3-EDT according to the RRC procedures – RAN2 clarifies the ta-Report field description

- New configuration to configure TA report for CB-Msg3-EDT

Proposal 3: RRC procedures need to capture CRID as complete response for the CB-Msg3 EDT in CP solution.

Proposal 4: For introducing enhancements from CB-Msg3 EDT to 4-step EDT, the only viable feature would be receiving CRID as complete response.

Proposal 5: RAN2 to confirm CQI reporting (in RRCEarlyData and DCQR and AS RAI MAC CE) in CB-Msg3 for CB-Msg3-EDT (for CP/UP eMTC and NB-IoT) is supported for anchor carrier.

Proposal 6: 3 ms processing is introduced for Msg4 monitoring.

Proposal 7: Multiple TBS per CE level is not supported.

# Reference

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#107, Incheon, ROK, March 2025.

R2-2502773, Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4 (Mediatek), Mediatek, RAN2#129bis, Wuhan, China, April 2025.