3GPP TSG-RAN WG2 Meeting #131	R2-2505926

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:		8.6.3

Source:		Lenovo

Title:		Leftover for L1 measurement and report

Document for:		Discussion and Decision

# Introduction

Some agreements have been made in the previous meetings [1]. This contribution aims to discuss the leftover issues for the event-based measurement report.

# Discussion

SP CSI-RS resource

It was agreed that UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. Whether to deactivate SP CSI-RS resource of the target cell after cell switch is not concluded. All the CSI-RS resources related to one LTM-CSI-ResourceConfigId cannot be applied to both RRM and CSI acquisition. Namely, the CSI-RS resources from multiple candidate cells related to one LTM-CSI-ResourceConfigId is applied to either measurement or CSI acquisition. So, we need to analyse considering the different purpose i.e., measurement or CSI acquisition.

Observation 1: The activated SP CSI-RS resource of candidate cells is applied to either measurement or CSI acquisition. Namely, the activated SP CSI-RS resource cannot be used for both measurement and CSI acquisition.

It is obvious that there are two options i.e., deactivation or activation after cell switch. There is no significant difference between two options. If the state of deactivation and activation is not suitable, the target cell can change it after cell switch.

It was agreed in RAN1 meeting that for PUSCH to convey the early CSI report,

- For RACH-less LTM, the first CG or DG PUSCH after CSC is used

- For RACH-based LTM with CFRA, PUSCH scheduled by RAR or Msg.A

- For RACH-based LTM with CBRA, the first CG or DG PUSCH after HARQ-ACK transmission for Msg.4 or Msg.B

If the activated SP CSI-RS resource for CSI acquisition should be deactivated after cell switch, we need to discuss the exact time point of deactivating SP CSI-RS resource since UE may continue measurement for CSI acquisition before report (after cell swich is triggered). In general, UE can deactivate SP CSI-RS resource once UE transmits early CSI report because it is possible that UE may update CSI report after triggering cell switch. If the activated SP CSI-RS resource is for measurement purpose, UE can deactivate it after cell switch is triggered.

If UE can keep the activated SP CSI-RS resource as activation after cell switch, it will impact specification less. Therefore, we suggest to keep activation after cell switch.

Proposal 1: UE can keep the activated SP CSI-RS resource of the target cell as activation after cell switch.

MR MAC CE

About the Type 4 beam type, the current definition captured in MAC CR is:

“…it is set to 11 to indicate the RS(s) not satisfying the event for the entire TTT, if configured by network by allowReportAnyBeam specified in TS 38.331 [5], i.e. the RS(s) with Type set to neither 00, 01, nor 11”

Not satisfying the even for "entire" TTT can possibly mean two things:

- The RS has entered reporting range, but TTT is still running

- RS has not even entered the reporting range and therefore the "not satisfying the event for the entire TTT" can be said to be true (below “Trigger Condition” in Figure 1)

There can be too many beams with b). It is just some RSRP/ RSSI (energy detection) and so, a UE implementation can (almost) never report all of these and so ends up creating a truncated report...which tells the network that there's something remaining to be reported. Additional grant is provided but if all the Type1/ 2 and 3 were already reported then the new UL grant is useless!

Figure 1: Any beam with quality lower than the Trigger Condition (i.e., the beam has not entered reporting range) has not started TTT

Beam type is set to 11 (= Type 4) to indicate the RS(s) not satisfying the event for the entire TTT but it is still unclear if this is an RS for which the TTT has been started, or even any RS (without any reference to TTT) can be termed as Type 4?

If the latter, then there will be many RS for which a measurement value can be reported. If so, what is the definition of (or the difference between) an 'event triggered L1 measurement report' and 'truncated event triggered L1 measurement report'?

Is the latter (=truncated) format used when the maxNumberOfReportedBeams is not reached or when the beams available for reporting can't fit in the available grant - which might often be the case depending on how we define Type 4 beams, as discussed above.

Proposal 2: Type 4 beams are the RS for which TTT has been started but it is still running at the time of reporting.

Proposal 3: If all Type0, 1 and 2 RSs corresponding to a MR MAC CE are included in a MAC TB, UE uses 'event triggered L1 measurement report'; otherwise, the 'truncated event triggered L1 measurement report' is used.

With respect to the Note X:

NOTE X: After sending a Truncated L1 measurement report MAC CE, if the subsequent UL grant is still not big enough to fit all the remaining beams, it is up to UE implementation to select the beam(s) which were not included in the previous MAC CE.

Even here the priority and quality order must be followed and so, we do not think this note is needed.

Proposal 4: RAN2 agrees to remove NOTE X.

With respect to the Note Y:

NOTE Y: If more than one triggering events for L1 measurement report are pending and the UL grant is not sufficient for all regular L1 measurement report MAC CEs, it is up to UE implementation how to handle/include the L1 measurement report MAC CEs and/or Truncated L1 measurement report MAC CE.

It's not of much use to report Type 4 beams. UE should maximize reporting of Type1/ 2/ 3 beams.

NOTE Y: UE maximizes transmission of Type1/ 2/ 3 beams when the UL grant is not sufficient to transmit all regular L1 measurement report MAC CEs.

Proposal 5: RAN2 agree to maximize transmission of Type1/ 2/ 3 beams when the UL grant is not sufficient to transmit all regular L1 measurement report MAC CEs.

It was agreed in RAN2 that the L1 MR MAC CEs can be multiplexed in one MAC PDU if multiple MR MAC CEs are triggered. That means some beams will be included in different MAC CEs. The UL resource can be limited and therefore it seems useful that UE includes a beam(s) only once and only in the latest triggering measurement report in a MAC CE if there are potentially multiple MRs with the same beam. Same applies to the current beam of the serving cell i.e., the current serving beam is included only once in the MAC TB.

Proposal 6: UE includes a beam(s) once and only in the latest triggering measurement report in a MAC PDU if there are potentially multiple MRs with the same beam. Same applies to the current beam of the serving cell i.e., the current serving beam is included only once in the MAC PDU.

# Conclusion

In this contribution, the following proposals are given based on the discussion:

Observation 1: The activated SP CSI-RS resource of candidate cells is applied to either measurement or CSI acquisition. Namely, the activated SP CSI-RS resource cannot be used for both measurement and CSI acquisition.

Proposal 1: UE can keep the activated SP CSI-RS resource of the target cell as activation after cell switch.

Proposal 2: Type 4 beams are the RS for which TTT has been started but it is still running at the time of reporting.

Proposal 3: If all Type0, 1 and 2 RSs corresponding to a MR MAC CE are included in a MAC TB, UE uses 'event triggered L1 measurement report'; otherwise, the 'truncated event triggered L1 measurement report' is used.

Proposal 4: RAN2 agrees to remove NOTE X.

Proposal 5: RAN2 agree to maximize transmission of Type1/ 2/ 3 beams when the UL grant is not sufficient to transmit all regular L1 measurement report MAC CEs.

Proposal 6: UE includes a beam(s) once and only in the latest triggering measurement report in a MAC PDU if there are potentially multiple MRs with the same beam. Same applies to the current beam of the serving cell i.e., the current serving beam is included only once in the MAC PDU.

# Text Proposal

-	Typei: This field indicates the type of the RS i of LTM candidate cell included in the event triggered L1 measurement report. The field is set to 00 to indicate the RS(s) that has satisfied the entry condition of the event associated with the report ID for the entire TTT, triggers this measurement report MAC CE, and included in the BEAM_TRIGGERED_LIST; it is set to 01 to indicate the RS(s) that has satisfied the leaving condition of the event associated with the report ID for the entire TTT, triggers this measurement report MAC CE, and exit the BEAM_TRIGGERED_LIST; it is set to 10 to indicate the RS(s) in the BEAM_TRIGGERED_LIST associated with the report ID as specified in clause 5.x.3 other than the RS(s) with Type set to 00; it is set to 11 to indicate the RS(s) for which TTT is still running, if configured by network by allowReportAnyBeam specified in TS 38.331 [5], i.e. the RS(s) with Type set to neither 00, 01, nor 11. The RS(s) not satisfying the event for the entire TTT is selected based on the decending order of measured quantity. The RS(s) included in the truncated event triggered L1 measurement report MAC CE are selected based on a decreasing order of the priority for the type of beam: 00, 01, 10, 11. If the (truncated) event triggered L1 measurement report MAC CE cannot accommodate all RS(s) with the same priority, the RS(s) are selected based on a decreasing order of measured quality. The length of the field is 2 bits;

# References

[1] RAN2#130 chair notes for Rel-19 mobility