3GPP TSG-RAN WG2 Meeting #131										R2-2505992

Bangalore, India Aug 25th – 29th, 2025                         		Revision of R2-2503763

Agenda item:		8.4.2

Title: 	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this contribution, we will further discuss the RAN2 related issues on procedure and configuration of LP-WUS for IDLE/INACTIVE modes and give our proposals.

- Discussion

2.1 About LP-WUS monitoring configuration

In RAN2#129bis meeting [1], there is an FFS as follows:

- All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations.

In RAN2#129bis meeting [1], there are some discussion on whether/how reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading, and there is the following agreements:

- Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). - It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

And some further discussion is suggested for direction 3 as follows [1]:

- Direction 3a. merge LPWUS monitoring and serving cell offloading - Direction 3b. merge LPWUS monitoring and Rel-19 RRM relaxation

During the LP_WUS study phase, it is indicated that LP-WUS monitoring without RRM Relaxation/Offloading would not bring much gain [2].

Meanwhile, if only RRM Relaxation/Offloading is enabled, UE need monitoring PDCCH for paging frequently, which is also not beneficial for UE power saving.

Observation 1: More power saving gain can be achieved if LP-WUS monitoring and RRM Relaxation/Offloading are performed simultaneously.

Therefore, it is suggested to merge the RRM Relaxation/Offloading condition and LP-WUS monitoring condition. Based on previous RAN2 agreements, network might configure Relaxation, Offloading, or both, and UE will work only in one state (e.g. either in RRC Relaxation, in offloading state, or none), UE can monitoring LP-WUS only when it is in serving cell offloading state or in Rel-19 RRM relaxation state.

Proposal 1: UE monitors LP-WUS only when it is in serving cell offloading state or in Rel-19 RRM relaxation state, i.e. the entry/exit condition for serving cell offloading and relaxation can be reused for LP-WUS monitoring, not configuring separate thresholds.

## 2.2 Activation/deactivation of LP-WUS

Since it has been agreed in RAN2#125bis that LP-WUS related configuration for IDLE/INACTIVE state is provided via system information, the provision of LP-WUS configuration via SIB can be seen as an implicit activation to apply LP-WUS configuration for all the UEs supporting this function. Meanwhile, if the LP-WUS configuration is no longer broadcast via SIB, it can be assumed that application of LP-WUS configuration (and also the LP-WUS monitoring) is deactivated.

However, it’s also possible that the UE would not apply LP-WUS configuration immediately on reception of LP-WUS configuration and the application of LP-WUS configuration will be activated later, e.g., per-UE via a dedicated signaling, e.g., RRC connection release message. During SI stage, such process has been discussed but no decision has been made.

The main motivation to activate/deactivate LP-WUS for a specific UE or part of UEs via dedicated signaling is that it may be not suitable to always activate the LP-WUS monitoring for all the UEs as soon as the LP-WUS configuration is broadcast. For example, LP-WUS can be applied to diverse UE types, e.g., IIoT UE, (e) Redcap UE, NR UE with XR traffic and so on, the device type or traffic type of UE may be another consideration for activating or deactivating LP-WUS for a UE. For example, the UEs with delay-sensitive services may not be suitable to monitor LP-WUS considering the possible large wake up latency caused by LP-WUS, which may have unexpected impacts on the delay-sensitive service. The exact latency that may be increased by LP-WUS is still under evaluation in RAN1, e.g., to take into account the LP-WUS processing time, MR transition time for ramp up, Time/frequency synchronization of MR etc. Per our understanding, if the additional latency caused by LP-WUS is far larger than the duration that random access procedure may use, e.g. if the increased latency is several seconds or more, the LP-WUS in IDLE/INACTIVE modes should be activated/deactivated by UE dedicated signaling, e.g., to activate LP-WUS only for the UE without delay-sensitive services or to deactivate LP-WUS when the UE starts to perform delay-sensitive services.

In a summary, from RAN2 perspective, we can see the need to activate/deactivate the application of LP-WUS configuration via dedicated signaling.

Observation 2: LP-WUS monitoring may result in additional latency compared to legacy paging procedure and hence, it is useful to support per UE activation of LP-WUS monitoring in addition to the per cell configuration.

In the last meeting, some companies propose and/or support that “CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signaling; the absence of indication means UE is allowed to use LP-WUS functionality”. But considering that CN does not know the radio resource configuration status (e.g. DRX cycle) and UE’s capability (e.g. time Offset between LP_WUS and PDCCH), it is difficult for CN to decide whether the LP-WUS is suitable for the UE. And SA2 has deleted the description related to PEI not used for emergency session, it is impossible that SA2 specifies something that LP-WUS does not apply some UEs.

Observation 3: CN does not know the radio resource configuration status and UE’s capability, it is difficult for CN to decide whether the LP-WUS is suitable for the UE.

Proposal 2: In addition to the SIB signaling for LP-WUS configuration, dedicated activation/deactivation of LP-WUS per UE is supported.

2.3 Time offset between LP-WUS and PO

As agreed in RAN1, UE reports one wake-up delay (e.g. one of candidate values) via UE capability reporting, and wake-up delay is minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. If UE reports, it can implicitly indicate that UE supports LP-WUS in RRC_IDLE/RRC_INACTIVE state.

Proposal 3: Introduce UE reporting wake-up delay as part of the UE capability, which is also an implicit indication of UE supporting LP-WUS in RRC_IDLE/RRC_INACTIVE state.

Considering that the LP-WUS time offset capability will be used for paging gNB to determine the LP-WUS occasion, the LP-WUS wake-up delay capability should be included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, since the ue-RadioPagingInfo-r17 IE is already included in UERadioPagingInformation container(e.g. which is included in RAN3 IE of UE Radio Capability for Paging IE) and is transparently passed from gNB to AMF, from AMF to gNB and from one gNB to another gNB, the LP-WUS capability will not be lost if the gNB is rel-17 or later.

Proposal 4: The UE reported wake-up delay is included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message so that it can be delivered to paging gNB transparently.

2.3 LP-WUS and eDRX

In RAN2#129bis meeting[3], it has been agreed that：

- LP-WUS is supported with eDRX, FFS on exact impact if any

Considering that eDRX includes CN eDRX and RAN eDRX, if eDRX is configured:

- UE in RRC_IDLE state need only monitor paging in CN PTW.

- UE in RRC_INACTIVATE state need monitor paging both in CN PTW and outside of CN PTW, if RAN eDRX is not configured.

- UE in RRC_INACTIVATE state need monitor paging in CN PTW and in RANPTW if RAN eDRX is configured.

Considering that eDRX is used for high latency communication case, UE need only monitor LP-WUS during the PTW and/or shortly before the start of the PTW when UE monitors paging only in PTW. For simplicity, UE can decide the LP-WUS monitoring occasion based on the reference PDCCH monitoring occasion, e.g. UE monitors LP-WUS with a time offset before the reference PO as already agreed, whichever the eDRX is configured or not. When determining the reference PO and the LP-WUS subgroup, the same mechanism as already agreed can be used, no matter whether the eDRX is configured or not.

Proposal 5: When UE monitors paging only in PTW, UE only monitors LP-WUS during the PTW and/or shortly before the start of the PTW, e.g. with a time offset before the reference PO.

Proposal 6a: UE monitors LP-WUS with a time offset before the reference PO as already agreed, no matter whether the eDRX is configured or not.

Proposal 6b: When determining the reference PO and the LP-WUS subgroup, the same mechanism as already agreed can be used, no matter whether the eDRX is configured or not.

2.5 Whether LP-WUS is only used in the last used cell or in any cell

Although LP-WUS false wake-up case is less than PEI false wakeup case (e.g. there is at most 31 LP_WUS subgroups, and UE may monitor PEI after the UE detects it’s LP-WUS), if the LP-WUS is sent in Tracking Area, and one Tracking Area includes many cells and/or there are large amount of UEs in one tracking area, LP-WUS false wake-up may also occur. Similar as PEI, a network indication (e.g. lastUsedCellOnly_LPWUS) can be used to indicate whether UE monitors LP-WUS only in the cell where the latest RRCRelease without noLastCellUpdate is received, or in all the cells, which can be configured by network based on implementation (e.g. how many cells included in one Tracking Area and/or the paging probability statistics in one tracking area).

Proposal 7: An indication (e.g. lastUsedCellOnly_LPWUS) is introduced to control whether UE only monitors LP-WUS in the cell where the latest RRCRelease without noLastCellUpdate is received.

## 2.6 Whether/how LP-WUS with SDT is supported

There is the following option issue that is still not concluded yet:

| 38304-10 | FFS whether/how LP-WUS with SDT is supported Note: the open issue was discussed online in RAN2#130 (R2-2504264) without conclusion. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

Since LP-WUS is used for UE to trigger paging monitoring, and MT-SDT is indicated in paging message. LP-WUS can be used with SDT, and SDT does not impact the LP-WUS mechanism i.e. when triggering MT-SDT, LP_WUS is sent firstly, and a legacy MT-SDT procedure (i.e. paging with MT-SDT indication) is followed.

Proposal 8a: LP-WUS can be used with SDT.

Proposal 8b (38304-10): When triggering MT-SDT, LP-WUS is sent firstly, and a legacy MT-SDT procedure (i.e., paging with MT-SDT indication in the paging payload) is followed. It is up to NW implementation and no spec impact is foreseen.

2.7 About the value range of LP-WUS subgroup

Based on the TS 38.304 running CR [2] as follows:

### 7.y.2 UE_ID based subgrouping LP-WUS with UE_ID based subgrouping is used in the cell which supports UE_ID based subgrouping for LP-WUS, as described in clause 7.y.0. If the UE is not configured with a CN assigned subgroup ID for LP-WUS, or if the UE configured with a CN assigned subgroup ID for LP-WUS is in a cell supporting only UE_ID based subgrouping for LP-WUS, the subgroup ID of the UE for LP-WUS is determined by the formula below: SubgroupID = (floor(UE_ID/(N*Ns*Np)) mod lp-SubgroupsNumForUEID) + (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID), ...

- If the UE is not configured with a CN assigned subgroup ID for LP-WUS, and the UE is in a cell supporting only UE_ID based subgrouping for LP-WUS(i.e. if lp-SubgroupsNumPerPO = lp-SubgroupsNumForUEID): since (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID)=0, the SubgroupID is from 0 to lp-SubgroupsNumForUEID - 1( e.g. SubgroupID = (floor(UE_ID/(N*Ns*Np)) mod lp-SubgroupsNumForUEID) in this case); And since lp-SubgroupsNumPerPO = lp-SubgroupsNumForUEID, the SubgroupID is from 0 to lp-SubgroupsNumPerPO - 1.

- If the UE is not configured with a CN assigned subgroup ID for LP-WUS and the UE is in a cell supporting CN assigned subgrouping for LP-WUS, the SubgroupID is from (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID) -1 to lp-SubgroupsNumPerPO - 1(e.g. (lp-SubgroupsNumForUEID -1) + (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID) = SubgroupsNumPerPO -1.

Observation 4a: For UE ID based LP_WUS subgrouping, the minimal LP-WUS subgroup ID is 0 or (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID), the maximal LP-WUS subgroup ID is lp-SubgroupsNumPerPO - 1.

And based on the TS 38.331 running CR [3], the maximal value of lp-SubgroupsNumPerPO is 31, thus, the maximal LP-WUS subgroup ID is 31 - 1 = 30.

LP-SubgroupConfig-r19 ::= SEQUENCE { lp-SubgroupsNumPerPO-r19 INTEGER (1.. maxNrofPagingSubgroupsLP-r19), lp-SubgroupsNumForUEID-r19 INTEGER (1.. maxNrofPagingSubgroupsLP-r19) OPTIONAL, -- Need S ... } ... maxNrofPagingSubgroupsLP-r19 INTEGER ::= 31 -- Maximum number of paging subgroups per paging occasion for LP-WUS

Observation 4b: For UE ID based LP_WUS subgrouping, the value range for LP-WUS subgroup ID is 0~30.

We think the value range of CN assigned LP-WUS subgroup ID should be the same as the value range of the UE ID based LP_WUS subgroup ID. Then the value range for CN assigned LP-WUS subgroup ID should also be 0 to 30.

And in the LS R2-2505028_R1-2505070[4], it is mentioned that “In addition to monitoring a codepoint associated with its subgroup ID, a UE configured with LP-WUS monitoring also monitors a common codepoint associated with all subgroups in a PO, if applicable.”

In TS 38.213[5], it only defined the calculation for code point associated with all subgroups in a PO, i.e. the code point for all subgroups in the PO is (iPO+1)*NSGPO+1-1, but which LP-WUS code point that associated with the UE refers to TS 38.304 as follows:

<TS 38.213> 10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE ... A paging occasion associated with a WUS occasion has index iPO=UE_ID mod N⋅NS+i_s mod NPOWO where NPOWO is a number of paging occasions associated with a WUS occasion, N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. If a number of NSGPO subgroups per paging occasion, provided by subgroupNumber-PO-WUS, is NSGPO>1, the codepoint for the subgroup index iSG in a PO iPO is iPO*NSGPO+1+iSG, and the codepoint for all subgroups in the PO is (iPO+1)*NSGPO+1-1; otherwise, the codepoint for the PO iPO is iPO. If, in a WUS monitoring occasion, a UE determines a codepoint associated with the UE [17, TS 38.304], the UE performs PDCCH monitoring according to Type2-PDCCH CSS sets for the paging occasion associated with the WUS monitoring occasion when a time from the end of the WUS reception to the start of the PDCCH monitoring occasion is not smaller than the value of XYZ; otherwise, the UE is not required to perform the PDCCH monitoring.

So, It should captured in TS38.304 that UE should monitor a LP-WUS code point associated with its subgroup ID, and a common code point associated with all subgroups in it’s PO, such as:

<TP for 38.304> 7.x.0 General The UE may monitor LP-WUS in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If LP-WUS configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting LP-WUS (except for the UEs expecting MBS group notification) may start LP-WUS monitoring using LP-WUS parameters in system information according to the procedure described below if the entry condition in clause 7.x.1 is fulfilled. The UE may stop LP-WUS monitoring if the exit condition in clause 7.x.1 is fulfilled. If the UE detects LP-WUS and the LP-WUS is associated with the UEor associated with all the subgroups in it’s PO as specified in clause 10.xx in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1 or monitors PEI as specified in clause 7.2 if PEI is supported and PEI configuration is provided in system information, which is up to UE implementation. If the UE does not detect an LP-WUS on the monitored LP-WUS occasion (LO) or the LP-WUS is not associated with the UE as specified in clause 10.xx in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1.

Proposal 9: Captured in TS 38.304 that UE monitors both a LP-WUS code point associated with its subgroup ID and a common code point associated with all subgroups in it’s PO.

Based on the TS38.213 [5], code point 31 will be used for common subgroup (i.e. code point associated with all subgroups in a PO), and would not occupy the value range of UE specific subgroup ID (either UE ID based or CN assigned).

Observation 5: The CN assigned LP-WUS subgroup ID should also be 0 to 30.

But in the TS 38.304 running CR [3], it has been captured that the UE can be assigned an LP_WUS subgroup ID between 1 to 31 as follows, it is not correct:

### 7.y.1 CN assigned subgrouping LP-WUS with CN assigned subgrouping is used in the cell which supports CN assigned subgrouping for LP-WUS, as described in clause 7.y.0. A UE supporting CN assigned subgrouping for LP-WUS in RRC_IDLE or RRC_INACTIVE state can be assigned a subgroup ID (between 1 to 31) by AMF through NAS signalling. The UE belonging to the assigned subgroup ID monitors its associated LP-WUS as specified in clause 7.x.

Proposal 10a: The value range for UE ID based and CN assigned LP-WUS subgroup ID is 0~30.

Proposal 10b: In TS 38.304 running CR, change the CN assigned LP-WUS subgroup ID range from “between 1 to 31” to “between 0 to 30”.

- Conclusion

The following proposals are given based on the above analysis:

Provision of LP-WUS related configuration

Observation 1: More power saving gain can be achieved if LP-WUS monitoring and RRM Relaxation/Offloading are performed simultaneously.

Proposal 1: UE monitors LP-WUS only when it is in serving cell offloading state or in Rel-19 RRM relaxation state, i.e. the entry/exit condition for serving cell offloading and relaxation can be reused for LP-WUS monitoring, not configuring separate thresholds for LP-WUS monitoring.

Activation/deactivation of LP-WUS

Observation 2: LP-WUS monitoring may result in additional latency compared to legacy paging procedure and hence, it is useful to support per UE activation of LP-WUS monitoring in addition to the per cell configuration.

Observation 3: CN does not know the radio resource configuration status and UE’s capability, it is difficult for CN to decide whether the LP-WUS is suitable for the UE.

Proposal 2(RRC-12): In addition to the SIB signaling for LP-WUS configuration, dedicated activation/deactivation of LP-WUS per UE is supported.

Time offset between LP-WUS and PO

Proposal 3: Introduce UE reporting wake-up delay as part of the UE capability, which is also an implicit indication of UE supporting LP-WUS in RRC_IDLE/RRC_INACTIVE state.

Proposal 4: The UE reported wake-up delay is included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, so that it can be delivered to paging gNB transparently.

LP-WUS and eDRX

Proposal 5(38304-9): When UE monitors paging only in PTW, UE only monitors LP-WUS during the PTW and/or shortly before the start of the PTW, e.g. with a time offset before the reference PO.

Proposal 6a(38304-9): UE monitors LP-WUS with a time offset before the reference PO as already agreed, no matter whether the eDRX is configured or not.

Proposal 6b(38304-9): When determining the reference PO and the LP-WUS subgroup, the same mechanism as already agreed canbe used, no matter whether the eDRX is configured or not.

Whether LP-WUS is only used in the last used cell or in any cell

Proposal 7(38304-8): An indication (e.g. lastUsedCellOnly_LPWUS) is introduced to control whether UE only monitors LP-WUS in the cell where the latest RRCRelease without noLastCellUpdate is received.

Whether and how LP-WUS with SDT is supported

Proposal 8a (38304-10): LP_WUS can be used with SDT.

Proposal 8b (38304-10): When triggering MT-SDT, LP-WUS is sent firstly, and a legacy MT-SDT procedure (i.e., paging with MT-SDT indication in the paging payload) is followed. It is up to NW implemetation and no spec impact is foreseen.

Value range of LP-WUS subgroup

Observation 4a: For UE ID based LP_WUS subgrouping, the minimal LP-WUS subgroup ID is 0 or (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID), the maximal LP-WUS subgroup ID is lp-SubgroupsNumPerPO - 1.

Observation 4b: For UE ID based LP_WUS subgrouping, the value range for LP-WUS subgroup ID is 0~30.

Observation 5: The CN assigned LP-WUS subgroup ID should also be 0 to 30.

Proposal 9: Captured in TS 38.304 that UE monitors both a LP-WUS codepoint associated with its subgroup ID and a common codepoint associated with all subgroups in it’s PO.

Proposal 10a: The value range for UE ID based and CN assigned LP-WUS subgroup ID is 0~30.

Proposal 10b: In TS 38.304 running CR, change the CN assigned LP-WUS subgroup ID range from “between 1 to 31” to “between 0 to 30”.

- References

- Draft_RAN2_129bis_Meeting_Report_v2

- R2-250xxxx 38.304 Running CR for LP-WUS_after post email.docx

- R2-250xxxx_RRC Running CR for LP-WUS WUR_v17_Rapp3.docx

- R2-2505028 LS on TP to TS38.300 for Rel-19 LP-WUSWUR (R1-2505070; contact vivo)

- TS 38.213, V19.0.0 (2025-06), NR; Physical layer procedures for control (Release 19)