3GPP TSG-RAN WG2 Meeting #131	 R2-2506006

Bangalore, India, August 25-29, 2025

Agenda Item:	8.3.2.2

Source: 	NEC

Title: 	Discussion on Functionality management for RRM measurement event prediction

Document for:	Discussion and Decision

# 1 Introduction

In last RAN2#130 meeting, the following agreements are made:

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations).FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

It has been highlighted in the agenda for RAN2#131 meeting that the functionality management for RRM measurement event prediction may include applicability procedure, inference configuration/reporting and performance monitoring for UE sided model for RRM measurement event predictions. Thus, in this contribution, we address few FFS issues related to applicability of associated IDs and inference configuration and reporting.

# Discussion

2.1 Applicability of Associated IDs

In AI/ML-based mobility use cases such as RRM measurement prediction, mobility event prediction, and RLF prediction, the gNB may configure multiple data collection tasks for a single UE through RRC signaling. These configurations may vary in purpose (e.g., training, inference, monitoring), periodicity, scope (e.g., per cell or frequency), or handover applicability. To manage this complexity, an Associated ID can serve as an identifier that links each configuration to its intended use case or model. It may include a use case ID, model/configuration ID, or be implicitly associated with periodicity and data logging behavior. This helps the UE correctly interpret and apply configurations when multiple models or functionalities are active.
Proposal-1: RAN2 may consider defining the Associated ID as a generic identifier to support flexible data collection configuration, logging, and reporting across multiple AI/ML-based mobility use cases.

2.2 Absence of Associated ID (FFS)

In AI/ML-based mobility procedures (e.g., RRM measurement prediction, measurement event prediction, RLF prediction), the Associated ID serves as a key identifier to distinguish between various configurations. This includes identifying use cases, sub-use cases, specific model configurations, and the intended purpose of data logging (e.g., training, inference, or monitoring).However, there may be scenarios where the Associated ID is absent in the RRC signaling messages (e.g., RRCReconfiguration, UECapabilityEnquiry) from the gNB. This absence may result in ambiguous UE behaviour unless clearly defined in the specification. 
We propose that the absence of Associated ID in RRC signalling messages be handled through clearly defined fallback behaviours at the UE side. The semantic meaning of this absence should depend on the deployment intent and network configuration scope. We summarize below how the UE may interpret the configuration in the absence of an Associated ID. Single AI/ML use case or model active can treat the configuration as globally applicable to the active AI/ML function. Multiple models/use cases active can assume the UE to apply configuration to all active models, unless this results in conflict or redundancy. Ambiguity can result in UE ignoring or rejecting the configuration and optionally provide feedback to the gNB through any suitable indication.

Proposal-2: RAN2 may define the absence of Associated ID in the following ways to guide UE behaviour consistently.
1) If a single use case or model is active, the configuration may be treated as globally applicable.
2)If multiple models/use cases are active, the UE may apply the configuration to all, unless conflicts arise.
3) In case of ambiguity, the UE may ignore or reject the configuration and optionally report failure to the gNB.

2.2 Inference Reporting Discussion

In AI/ML-based mobility management, inference input reporting is critical for enabling real-time decision-making, such as RRM measurement prediction, event prediction, or RLF prediction. This reporting must adhere to strict latency bounds to ensure timely network responses. The reported data must meet predefined accuracy thresholds to maintain model reliability, while avoiding redundant or outdated inputs. To reduce signaling overhead, the gNB may define a maximum size for inference input reports, enabling efficient bandwidth use. These constraints can vary depending on the specific AI/ML use case and model deployed at the UE or network side.

Proposal-3: RAN2 may define inference input reporting constraints per use case, including latency bounds, accuracy thresholds, and maximum report size, to ensure timely, efficient, and accurate AI/ML-based mobility operation.

# 3 Conclusions

We have the following proposals for the functionality management of RRM measurement prediction.

Proposal-1: RAN2 may consider defining the Associated ID as a generic identifier to support flexible data collection configuration, logging, and reporting across multiple AI/ML-based mobility use cases.

Proposal-2: RAN2 may define the absence of Associated ID in the following ways to guide UE behaviour consistently.

1) If a single use case or model is active, the configuration may be treated as globally applicable.

2)If multiple models/use cases are active, the UE may apply the configuration to all, unless conflicts arise.

3) In case of ambiguity, the UE may ignore or reject the configuration and optionally report failure to the gNB.

Proposal-3: RAN2 may define inference input reporting constraints per use case, including latency bounds, accuracy thresholds, and maximum report size, to ensure timely, efficient, and accurate AI/ML-based mobility operations.