3GPP TSG RAN WG2 #131				R2-2506028

Bengaluru (Bangalore), India, Aug. 25th – 29th, 2025

Agenda Item:	8.2.3

Source:		ASUSTeK

Title:				Discussion on Ambient IoT random access

Document for: Discussion and Decision

- Introduction

In RAN2#130 meeting [1], the following agreements on A-IoT random access were made.

- Agreements on RA - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further down-selected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it.

In this contribution, we would discuss the open issues in the A-IoT random access procedure.

- Discussion

2.1 	Issue 2-4: CBRA failure detection

In CBRA, the device would select an access occasion to transmit Access Random ID message (Msg1) after receiving A-IoT Paging message indicating itself. The access occasion could be determined based on Time Resource Indication, Frequency Resource Indication and the number of received Access Trigger messages, where the access occasion may be followed by the corresponding Access Trigger message. In response to transmitting the Random ID message, the device would monitor PRDCH for Random ID Response message (Msg2). After successful reception of the Random ID Response message, the device would transmit D2R Upper Layer Data Transfer message (Msg3) in response to indication and scheduling information in the Random ID Response message.

During the CBRA procedure, there could be reception or transmission failure occurred at the reader or the device side. For example, the device might fail to receive Msg2. In contrast, the reader might fail to receive Msg3 after the device receives Msg2 successfully. Once the reader failed to receive Msg3, the reader could perform Msg2 retransmission to handle CBRA failure; meanwhile, the device would keep monitoring Msg2 even after transmitting Msg3.

From previous RAN2 meetings, it was agreed that the device expecting Msg2 assumes CBRA failure if its Msg2 is not received before a boundary, and the device does not process Msg2 (re)transmission received after the boundary. There are two options for the boundary left to be down-selected:

- Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message

- Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS)

For option C, the device needs to decode multiple Access Trigger messages after transmitting Msg3. However, this does not seem to place a large burden on the device since the size of the Access Trigger message is relatively small. On the other hand, the reader would obtain scheduling flexibility in Msg2 retransmission. For example, the reader does not receive Msg3 from some devices after a Msg2 transmission echoing the device. There are chances that the device is just “unlucky” that it missed R2D transmission for Msg2 (e.g., temporarily blocked), and it would be able to receive Msg2 retransmission and transmit Msg3 successfully soon. The reader does not need to handle CBRA failure for all failed devices before transmitting the next Access Trigger message. Instead, the reader could schedule them in the next Msg2 transmission, which echoes other devices transmitting Msg1 indicated by the upcoming Access Trigger messages.

For option B, the device would have a slight gain in decoding effort since it only needs to decode one Access Trigger message. Nevertheless, this brings constraint for the reader that it should handle as many devices with CBRA failure as possible before sending the next Access Trigger message. Otherwise, the failed device would have to perform re-access until next paging round, which brings longer delay for the device. If the reader does not want to put the device on hold for such a long time, the number of Msg2 retransmissions before the next Access Trigger message must increase, and thus introduces latency to other devices.

Based on the above analysis, we prefer option C in detection of CBRA failure. Considering device complexity, the value of k is suggested to be fixed.

Proposal 1:  For detection of CBRA failure, select option C that the boundary is the reception of either the kth R2D trigger message or the subsequent paging message. The value of k is fixed.

2.2  Issue 2-5: random ID differentiation in Msg2

It was agreed that Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. Thus, there could be chances that two devices sending the same random ID in different access occasions are included in the same Msg2, resulting in Msg2 interpretation by both the two devices. Since scheduling information for Msg3 is included in Msg2, there would be collision in Msg3 transmissions from the two device. Without other information on top of RN16, the successful rate of Msg3 transmission would decrease. In addition, such failure could not be addressed by Msg2 retransmission. The collided devices have to re-access in the next paging round, which is not favorable as explained in 2.1.

According to the discussion in RAN2#130, including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. Based on the above analysis, we believe that the frequency index could be included in Msg2 to solve the issue. When the reader receives a same random ID from more than one device, the reader includes frequency index of the random ID messages from the devices in Msg2.

Proposal 2: The frequency index could be included along with RN16 in Msg2 to reduce collision of Msg1 between different devices.

- Conclusion

Proposal 1:  For detection of CBRA failure, select option C that the boundary is the reception of either the kth R2D trigger message or the subsequent paging message. The value of k is fixed.

Proposal 2:  The frequency index could be included along with RN16 in Msg2 to reduce collision of Msg1 between different devices.

- Reference

[1] Report of 3GPP TSG RAN WG2 meeting #130, St Julian's, Malta