3GPP TSG-RAN WG2 Meeting #131		       R2-2506031

Bengaluru, India: 25th Aug – 29th Aug 2025

Agenda Item:	8.3.2.2

Source:	ASUSTeK

Title:	Discussion on functionality management for measurement event prediction

Document for:	Discussion and Decision

- Introduction

In the RAN2 #130 meeting, companies have discussed about the performance monitoring KPIs for measurement event prediction. In this document, we would like to discuss the feasibility and benefits for each option.

- For direct and indirect measurement event prediction, study the following options, - Option 1: Use intermediate KPIs related to the event detection, such as F1-score, miss/false prediction rate, for monitoring. - Option 2: Use the HO KPIs such as HoF rate, Short-ToS rate, PingPong rate, etc.

- Discussion

The reason of having performance monitoring for an inference configuration is because AI models are used for prediction which may not be 100% accurate. Some AI models may work well in simulated environments but fail to work in the real world due to domain gap or the complicated radio conditions. Additionally, each UE may apply its own AI model tailored for a specific use case or purpose, which may not be disclosed to the NW. Therefore, the NW needs to monitor the performance of AI models to prevent the UE from using an AI model that is not performing well and thus wasting resources such as UE power or uplink resources for the inference reports.

An AI model is typically trained with data pairs comprising input data and ground truth data. The target of the AI model is to guarantee the difference between output from the AI model and the corresponding ground truth data is minimized. The accuracy of the AI model is based on how  the difference is minimized. Great accuracy implies the AI model is trained well, and in most cases, using the AI model in the targeted application is beneficial, especially when the output (or the ground truth) where the AI model is optimized for directly aligns with the application’s KPI. When the output (or the ground truth) of the AI model is aligned with the application’s KPI, it is simple to monitor the AI model by comparing the AI model outputs with collected ground truth as done in beam management, CSI prediction, RRM prediction use cases, and intermediate KPIs related to the event detection such as f1-score (i.e., option 1) for measurement event prediction.

When it comes to measurement event prediction, the monitoring may be a little more complicated and controversial, since the application’s KPI (i.e., improving handover performance) is not directly aligned with the output (or the ground truth) of the trained AI model. The AI model is trained under the assumption that some handover failures are caused by too late handover (e.g., due to too long NW processing time). If the assumption is correct, then monitoring the performance using option 1 can at least ensure the performance gains from here is monitored. If the AI model is performing well, the UE is expected to have a more seamless experience of handover. If the AI model is not performing well, the inference configuration likely needs some modification or deactivation.

Proposal 1: Intermediate KPIs related to the event detection can indicate whether the event prediction part in the handover system is performing well and should be at least monitored.

However, the measurement event prediction also has the potential to predict for more accurate handovers (e.g., direct event prediction). With this in mind, although the AI model is not trained based on the handover performance targets, we do see a motivation to monitor the HO KPIs such as HoF rate, Short-ToS rate, PingPong rate. However, in a functionality management perspective, the information alone may not be that useful. For example, if the HO KPIs are bad, it is unclear whether the cause is bad prediction of events or bad NW decisions.

Observation 1: Only monitoring HO KPIs cannot identify which part of the handover system is not performing well.

Proposal 2: HO KPIs can be monitored along with intermediate KPIs related to the event detection and complement each other.

One burden for monitoring the intermediate KPIs related to the event detection is due to the current modelling for measurement event prediction. According to the Text proposal of TR 38.744 [2], the KPI is based on the actual event triggering time.

#### 5.3 Measurement event prediction ##### 5.3.1 Evaluation methodology, metrics and assumptions The performance metric F1 score is defined as following: F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD] The ETD i.e. timing difference between ground-truth event and predicted event is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected. For direct prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3 (true event prediction): it increases by 1 when a ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1 (false event detection): it increases by 1 when no ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2 (missed event detection): it increases by 1 when a ground-truth event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold For direct prediction method, a measurement event could be predicted within an occurrence window starting from current time instance i.e. t0 and future time instance t1 with a probability as illustrated in Figure 5.3.1-1. Figure 5.3.1-2: occurrence window of direct prediction method

This can cause a burden to HO performance and restrictions to NW implementation as the NW have to wait for at most ETD after the expected event triggering time until the NW can perform handover to perform monitoring, which is not preferable. As an alternative, we propose to consider the handover options in SLS assumptions (e.g., option 1/2) for determining the successful predictions (e.g., consider success when actual measurements fulfil entering conditions after prediction).

Proposal 3: Consider the handover options in SLS assumptions (e.g., option 1/2) for determining the successful predictions (e.g., consider success when actual measurements fulfil entering conditions after prediction).

- Conclusion

Observation 1: Only monitoring HO KPIs cannot identify which part of the handover system is not performing well.

Proposal 1: Intermediate KPIs related to the event detection can indicate whether the event prediction part in the handover system is performing well and should be at least monitored.

Proposal 2: HO KPIs can be monitored along with intermediate KPIs related to the event detection and complement each other.

Proposal 3: Consider the handover options in SLS assumptions (e.g., option 1/2) for determining the successful predictions (e.g., consider success when actual measurements fulfil entering conditions after prediction).

References

[1] Chair minutes for RAN2 #130

[2] Text proposal of TR 38.744

Annex

Figure 5.5.1-1: Handover model option 1 Option 1 is illustrated in Figure 5.5.1-1. At current time i.e. t0 measurement event e.g. A3 event is predicted at some point of time in future. Network will not transmit handover command until a real measurement event is reported for the same neighbouring cell. In this way, the main benefit of this option is to save handover preparation time. Figure 5.5.1-2: Handover model option 2 Option 2 is illustrated in Figure 5.5.1-2. At current time i.e. t0 measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event is met based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier.