3GPP TSG-RAN WG2 #131	R2-2506057

Bengaluru, India, 25th – 29th August 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on remaining issues on AIoT data transfer

Document for:	Discussion

Agenda Item:	8.2.4

- Introduction

In the post e-mail discussion after RAN2#130 [1], following issues related to data transfer remained unsolved in the end.

| Group 3: Data transmission |  |  |
| --- | --- | --- |
| Subgroup: AS ID |  |  |
| Issue 3-3: AS ID release | Whether a release message is needed for AS ID release | Companies are invited to input views for Q#6 |
| Subgroup: D2R message content for data transmission |  |  |
| Issue 3-5: D2R message type | Whether to support D2R message type | Companies are invited to input views for Q#7 |
| (New)Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) | Companies are invited to input views for Q#9 |

Among these issues, this contribution explains our views on Issue 3-5 and Issue 3-7.

- Discussion

  - Issue 3-5: Whether to support D2R message type

At least for the procedure starting with a Release 19 reader sending paging to a Release 19 device, multiple companies have pointed out that the D2R message type is unnecessary. In other words, since the device sends the corresponding D2R message using the D2R resource indicated by the reader in the R2D message it received, the reader can implicitly identify all received D2R messages.

Therefore, the primary motivation for introducing the D2R message type in Release 19 is to ensure extensibility in Release 20. Specifically, it is important to guarantee that a Release 19 reader can ignore messages received from a Release 20 device that do not exist in Release 19.

Then the issue is whether Release 20 devices may coexist under Release 19 readers. In RAN#106, both an SID for outdoor scenarios and a WID for indoor scenarios were approved as part of Release 20 Ambient IoT. The WID Objective includes the following statement:

WID for Release 20 Ambient IoT [2] Note: Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band.

Based on this objective, considering that Release 20 Device 2b/C is intended for outdoor scenario operations, it can be assumed that it will not coexist with Release 19 readers deployed in indoor environments.

On the other hand, a scenario in which a Release 20 Device 1 accesses a Release 19 reader is anticipated. This coexistence is a realistic scenario, similar to the fact that a UE supporting the latest release may access a base station that has not yet been upgraded to the latest NR release. In the same way, indoor-oriented chips that support Release 20 and are available on the market may be introduced into customer environments where Release 19 Ambient IoT for indoor use has already been deployed.

Observation 1.	(Issue 3-5) Based on the WID of R20 Ambient IoT, the scenario in which Release 20 Device 1 is paged by Release 19 reader can exist.

Considering such scenarios, it is needed to allow a Release 19 reader to ignore D2R messages introduced in Release 20 that are received from a Release 20 Device 1. One solution is to introduce D2R message type field in every D2R message in the same way as R2D, but we are open to any other solutions more efficient in terms of overhead.

Proposal 1.	(Issue 3-5) Release 19 readers should ignore Release 20-introduced D2R message. Introducing D2R message type can be a solution.

  - Issue 3-7: How to set MDI when D2R NAS message is not available

In the last meeting [3], RAN2 agreed that if a device fails to generate a NAS message before the arrival of the D2R resource indicated by the reader, it should send a D2R Data Transfer message containing a 0 SDU. Also, an LS was sent to CT1 (cc. SA2) to ask whether this issue can be addressed, [4].

Agreements

- The device is expected to send a MAC response to the reader in the D2R occasion.   The MAC response contains the NAS message if available at the D2R occasion.   If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed.

- Send LS to CT1 to inform the agreement 1 to CT1 and explain that we have an issue with delayed NAS write success response.   RAN2 would prefer that this is handled by CT1 (and give the example of sending NAS response upon successful reception of write command).    Ask if this can be handled by CT1

We believe that the concerned situation in the agreements above can be addressed solely within RAN.

As illustrated in Figure 1, when a reader receives a message containing a 0 SDU, it can perform D2R scheduling again for the corresponding device before notifying AIoTF (this behavior is the same as the case where the SDU is segmented and the More Data Indicator in the previous D2R Data Transfer was set to 1). By doing so, the reader can complete the reception of all D2R data before transmitting it to AIoTF, and thus, there is no impact on CT1 and SA2.

Figure 1. RAN-based solution without impact on CN.

Therefore, to unify the reader’s behavior when receiving a message containing 0 SDU with that of receiving a D2R segment, the More Data Indicator should be set to 1 when transmitting a 0 SDU.

Observation 2.	(Issue 3-7) Althhough RAN2 sent CT1 to ask to solve the situation in which the device delivers Data Transfer message with 0 SDU, RAN2 can find a solution without the CN impact.

Proposal 2.	(Issue 3-7) More Data Indicator is set to 1 when transmitting Data Transfer message with 0 SDU.

- Summary and proposal

Observation 1.	(Issue 3-5) Based on the WID of R20 Ambient IoT, the scenario in which Release 20 Device 1 is paged by Release 19 reader can exist.

Proposal 1.	(Issue 3-5) Release 19 readers should ignore Release 20-introduced D2R message. Introducing D2R message type can be a solution.

Observation 2.	(Issue 3-7) Althhough RAN2 sent CT1 to ask to solve the situation in which the device delivers Data Transfer message with 0 SDU, RAN2 can find a solution without the CN impact.

Proposal 2.	(Issue 3-7) More Data Indicator is set to 1 when transmitting Data Transfer message with 0 SDU.

- References

[1]	[POST130][027][AIoT] MAC Running CR (Huawei).

[2]	RP-251885, “Solutions for Ambient IoT (Internet of Things) in NR Phase 2,” RAN#106.

[3]	“Chair’s notes,” RAN2#130.

[4]	R2-2504967, “LS on delayed A-IoT D2R NAS messages,” RAN2#130.