3GPP TSG-RAN WG2 Meeting #131							R2-2506088

Bengaluru, India, 25 - 29 Aug, 2025

Title:  	Discussion on UE-side data collection

Source: 	Huawei, HiSilicon, OPPO, China Telecom, CATT, China Unicom, ZTE, NTT DoCoMo

Agenda Item:	8.1.4

Document for:	Discussion and decision

- Introduction

In previous RAN2 meetings, RAN2 discussed candidate options for UE-side data collection and captured them in the TP [1].

At RAN2#130, some discussion was made for CP options, and then some analysis was agreed to be captured in the TP via [2].

For RAN2#131, the draft agenda has been published [3], and some aspects identified in RANP WF may be discussed.

- 8.1.4 UE side data collection Configuration details for Rel-19 data collection for UE-sided model can be discussed in contributions in 8.1.2.2. Discuss any of the aspects identified in RANP WF o Study RAN aspects related to data transfer over UP o Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. o Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). o Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. o Discuss on the scalability aspects of CP Including the outcome of [POST130][033][AI PHY] UE Side data collection (Ericsson) This AI is downprioritized as it will be in scope of Rel-20. If any contributionsa are submiited they should be focused on something critical needed to be included in the TR.

RAN#108 has approved a WID on R20 AI for PHY in [4]. One bullet is about data collection [RAN2], and there will be a check-point in RAN#110.

Data collection [RAN2]: - Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection

In this paper, we provide more analysis based on RAN2#131 agenda, including CP solution analysis and RAN aspects/impacts related to data transfer over UP.

- Discussion

Analysis of Option 2-CP (CN-based)

For Option 2-CP, it is transparent to RAN, and feasibility of NAS signalling transmission is up to other WGs, e.g. CT1.

Analysis of Option 3-CP (OAM-based)

In R19, NW-side data collection has been introduced and solutions are near stable (they have also been captured in the running 38.300 and 38.331 CRs). For Option 3-CP and NW-side data collection (OAM-centric), we do not see much difference, so Option 3-CP can largely re-use NW-side data collection.

Based on the LS [5], RAN1 provided data collection for each use case. For beam management, it can be seen that the required training data for UE-side/NW-side is the same. For other UE-side model (e.g. CSI prediction), if Option 3-CP is re-used, the only change is that more measurements will be added into NW-side data collection framework, but the new change will not be much.

| LCM purpose | UE-side/NW-side models | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | UE-side, NW-side | L1-RSRPs and/or beam-IDs | See Note 1 for L1-RSRPs | Relaxed |  |

Observation 1: Option 3-CP can re-use R19 NW-side data collection and extra efforts are minimal.

For UE capability requirements, [6] is discussing details and the discussion of UE logging AS layer memory size is also included.

According to the post-meeting email discussion [7], NW-side data collection has the following features:

For data collection for NW-side model training, data collection can be initiated by OAM or by gNB. The following enhancements are introduced for data collection for NW-side model over air interface: - The UE can be configured by gNB to log L1 measurement results in RRC message. - The UE can be configured by gNB to log L1 measurement and report them via RRC message. Editor's note: FFS if dynamic activation/deactivation is support. - Both periodic and L3 measurement event-triggered data logging are supported. The UE stores the logged data at the AS layer buffer. When the memory reserved for storing logged data becomes full or reaches an configured absolute threshold, the UE stops measurement and logging for data collection, and further sends an indication to the network, if configured, as specified in TS 38.331[12]. - When low power state is detected, the UE can indicate the low power state to the network, if configured. Upon reception of the low power state indication, the network should de-configure UE to release the data collection configuration. - Network can configure UE whether the logged data should be kept or not during handover. When configured to keep data, UE retrains the logged data during handover and indicates the availability of logged data to network after handover. Upon UE transitioning to RRC_IDLE/INACTIVE or UE experiencing RLF, UE discards any stored data.

With the above information, we think the challenges can be addressed and solved via NW-side data collection:

For Challenge#1 UE memory allocation: the current definition can be followed, and there is no need to add more requirements.

For Challenge#2 Segmentation for UE side data collection: the current definition can be followed, i.e. the UE stores the logged data at the AS layer buffer, and if some conditions are met, the UE stops logging and sends indication(s) to NW for data transfer. Therefore, there is no need to further discuss segmentation aspect.

For Challenge#3 Continuity of the collected data reporting: the current definition can be followed, i.e. network can configure UE whether the logged data should be kept or not during handover. With this solution, the continuity of the collected data reporting can be achieved.

Observation 2: For RAN2 identified challenges, we think R19 NW-side data collection has addressed and solved them, i.e.

- For Challenge#1 UE memory allocation: the current definition can be followed, and there is no need to add more requirements.

- For Challenge#2 Segmentation for UE side data collection: the current definition can be followed, i.e. the UE stores the logged data at the AS layer buffer, and if some conditions are met, the UE stops logging and sends indication(s) to NW for data transfer. Therefore, there is no need to further discuss segmentation aspect.

- For Challenge#3 Continuity of the collected data reporting: the  current definition can be followed, i.e. network can configure UE whether the logged data should be kept or not during handover. With this solution, the continuity of the collected data reporting can be achieved.

Analysis of Option 2-UP (CN-based)

For Option 2-UP, a R20 SA2 SI is now discussing the feasibility [8].

Similar to Option 1a, which is also a User Plane (UP)-based solution, a UE application client needs to be deployed above the UE Access Stratum (AS) layers or even above the device’s operating system (OS). This UE application client should connect to the Core Network (CN) function/entity and transmit data to the CN via the UP tunnel. The key difference from Option 1a lies in the control of the UE application client: whereas in Option 1a the UE application client is controlled by the UE, in the Option 2 UP solution, it should be controlled by the CN or operators, given that the connectivity exists between the UE application client and the CN entity.

Leaving aside the industrialization challenges of the UE application client— which would be a major problem for this solution—RAN2 should at least explore the coordination between the UE application client (responsible for data transmission) and the UE AS layers (responsible for data collection).

Figure 1 User plane solutions for data collection

Observation 3: In the option 2-UP solution, same as option 1a, a UE application client needs to be deployed above the UE Access Stratum (AS) layers or even above the device’s operating system (OS). The UE application client should be controlled by CN or operators.

Observation 4: The industrialization challenges of such UE application client in option 2-UP has not been fully analyzed.

Observation 5: For Option 2-UP, the coordination between the UE application client (responsible for data transmission) and the UE AS layers (responsible for data collection) has not been studied.

Analysis of Option 3-UP (OAM-based)

For Option 3-UP, RAN2 and SA5 had some LSes in [9], [10]. SA5 thinks UP tunnel between UE and server is the same as Option 1a and UE tunnel between UE and 5GC is the same as Option 2, and then Option 3-UP can be categorized to other options.

For UP tunnel between UE and gNB, RAN2 answer is that it is not considered for 5GA Rel-19/20.

- Conclusion

In this paper, we provide our analysis of UE-side data collection, following RAN2#131 agenda. We have the following observations:

Option 3-CP

Observation 1: Option 3-CP can re-use R19 NW-side data collection and extra efforts are minimal.

Observation 2: For RAN2 identified challenges, we think R19 NW-side data collection has addressed and solved them, i.e.

- For Challenge#1 UE memory allocation: the current definition can be followed, and there is no need to add more requirements.

- For Challenge#2 Segmentation for UE side data collection: the current definition can be followed, i.e. the UE stores the logged data at the AS layer buffer, and if some conditions are met, the UE stops logging and sends indication(s) to NW for data transfer. Therefore, there is no need to further discuss segmentation aspect.

- For Challenge#3 Continuity of the collected data reporting: the  current definition can be followed, i.e. network can configure UE whether the logged data should be kept or not during handover. With this solution, the continuity of the collected data reporting can be achieved.

Option 2-UP

Observation 3: In the option 2-UP solution, same as option 1a, a UE application client needs to be deployed above the UE Access Stratum (AS) layers or even above the device’s operating system (OS). The UE application client should be controlled by CN or operators.

Observation 4: The industrialization challenges of such UE application client in option 2-UP has not been fully analyzed.

Observation 5: For Option 2-UP, the coordination between the UE application client (responsible for data transmission) and the UE AS layers (responsible for data collection) has not been studied.

Proposal: It is proposed RAN2 to capture observation 1 to 5 into TR 38.843.

We also provide a Text Proposal in section 5.

- References

- R2-2407807, CR on RAN2 inputs to TR 38.843, Ericsson

[POST130][033][AI PHY] UE Side data collection (Ericsson)

R2_131_Agenda_v00

RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Qulacomm

R2-2311720, Reply LS on Data Collection Requirements and Assumptions, Source: RAN1, To: RAN2

[POST130][038][AI PHY] UE capabilities (Xiaomi)

[POST130][022][AI PHY] stage 2 CR (Vivo)

SP-250810, Revised SID on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML) Phase 2, vivo

R2-2501759, Reply LS on AI/ML UE sided data collection, Source: SA5, To: RAN, RAN2, Cc: SA, SA2, SA3

R2-2503168, LS on AI/ML UE sided data collection, Source: RAN2, To: SA5, Cc: SA, SA2, SA3, RAN

- Text proposal for UE-side data collection

- 7.2.1.3.2	Data collection for UE-side model training

<Text Omitted>

- 7.2.1.3.2.1	Data collection for UE-side model training – CP solutions analysis

Related to the solutions based on CP and captured in Section 7.2.1.3.2, the following challenges and their corresponding solutions in the Table 7.2.1.3.2.1-1 have been identified, especially for the case of large amount of data to be transferred from a UE.

Table 7.2.1.3.2.1-1. Data Collection CP solutions analysis

| Challenges | Description |
| --- | --- |
| UE memory allocation | The collected data may need to be stored in the access stratum (AS) buffer for the control plane-based data transfer via AS. UE AS memory definition for R19 NW-side data collection can be followed. |
| Segmentation for UE side data collection | A single RRC message can contain maximum about 9KB data. Thus RRC or higher layer segmentation is needed to transfer more data that can fit in a single RRC message The existing RRC message only supports up to 16 segments in UL, which amounts to a maximum of 144KB of collected data that can be transmitted. The UE stores the logged data at the AS layer buffer, and if some conditions are met, the UE stops logging and sends indication(s) to NW for data transfer. Therefore, there is no need to further discuss segmentation aspect. |
| Continuity of the collected data reporting | In case the collected data are not yet fully transferred before a handover, Xn / NG-AP signaling enhancements may be required for the continuity of the data reporting. Similarly, it could be further discussed the case of collected data not yet fully transmitted before a radio link failure, or before transitions to IDLE/INACTIVE mode. The network can configure UE whether the logged data should be kept or not during handover. When configured to keep data, UE retrains the logged data during handover and indicates the availability of logged data to network after handover, and then the target gNB can collect the data from the UE. With this solution, the continuity of the collected data reporting can be achieved. |

- 7.2.1.3.2.2	Data collection for UE-side model training – UP solutions analysis

Related to the solutions based on UP and captured in Section 7.2.1.3.2, the following challenges in the Table 7.2.1.3.2.2-1 have been identified.

Table 7.2.1.3.2.2-1. Data Collection UP solutions analysis

| Challenges | Description |
| --- | --- |
| Coordination between the UE application client and UE AS layers | The UP solution must rely on a UE application client that sits above the UE AS layers or even above the device’s operating system, with this client connecting to an entity or server inside the CN (and for option 1a, connecting to the OTT server outside the CN). Collected data may first need to be stored in the access stratum (AS) buffer before being delivered to the UE application client for data transfer. Since the UE application client requires connectivity to the CN entity or server, it should be controlled by the CN or operators, whereas the UE AS layers remain under UE control. The coordination between the UE application client (responsible for data transmission) and the UE AS layers (responsible for data collection) requires further discussion. |