3GPP TSG-RAN WG2 #131	R2-2506097

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.5.4

Source:	Ericsson, Nokia, Nokia Shanghai Bell, InterDigital, NEC

Title:	Joint utilization of paging and RA resource adaptations

Document for:	Discussion

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised in [4]. In this paper, we address the third objective given in the revised WID as:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we discuss how Rel-19 paging and RA resource adaptations can be jointly utilized.

# 2	Discussion

It is important to note that when POs are configured condensed, it is likely that multiple UEs are paged within rather a short period of time. Consequently, it is also very likely that paged UEs, if camping in that cell, will trigger the RA procedure almost simultaneously. This may cause congestion on the use of configured RA resources, especially if legacy RA resources are configured sparsely for the purpose of energy savings.

To accommodate such increase in congestion on RA resources, the network can temporarily increase RA resource capacity by activating adaptive RA resources. However, assuming a typical scenario in which UE initiates a procedure for network access via the closest possible RA occasion, it is likely that multiple collisions will occur on the earliest available adaptive RA occasion. To avoid collisions in such scenarios, it would be beneficial if a gNB could coordinate access to RA resources for paged Rel-19 UEs by signaling which of the UEs can access both legacy and activated adaptive RA occasions, and  which can access only legacy RA occasions as illustrated in Figure 1. This would allow the gNB to reduce the risk of collisions on RA resources under varying population ratios of legacy and Rel-19 UEs by balancing the load on legacy and adaptive RA resources.

…

Adaptive RA resources

gNB indicates that adaptive RA resources are available to UE 1, but not to UE 2

time

SSB transmission

Paging frames/occasions

UE 1

…

PO 2

UE 2

Figure 1: Indication of adaptive RA resource availability.

To minimize the probability of collision using RA resources when paging occasion adaptations are configured, it would be beneficial if gNB assigns which of the paged Rel-19 UEs can trigger access via both legacy and activated adaptive RA occasions and which can trigger access via only legacy RA occasions.

It is important to observe that such coordinated access, illustrated in Figure 1, cannot be achieved via paging DCI (as captured in the running 38.331 CR). This is because the paging DCI can be used to indicate the activation of adaptive RA resources for all UEs monitoring that PO, but not to indicate which of those UEs can utilize the activated adaptive RA resources.

Paging DCI can be used to indicate the activation of adaptive RA resources for all UEs monitoring that PO, but not to indicate which of those UEs can utilize the activated adaptive RA resources.

Note that this does not have an impact on MAC layer regarding how a UE selects the adaptive RA resources, if configured and activated, i.e., same UE behavior as for adaptive RA occasions activated by DCI, where the UE selects next available RA occasion as in legacy.  There is no prioritization in MAC for the selection of a particular RA resource set.

There is no impact on MAC layer regarding how a UE selects the adaptive RA resources, i.e. UE selects the next available RA occasion as in legacy.

To address the potential issue of increased collisions on RA resources in case of Rel-19 condensed POs, the gNB can configure adaptive RA resources that are not always available to all UEs. Then, when paging Rel-19 UEs, the gNB can indicate which of the paged Rel-19 UEs can access activated adaptive RA occasions.

Extend the paging message with a per-UE indicator that signals the availability of activated adaptive RA resources for the corresponding UE.

Adopt the TP below in the running 38.331 CR.

----------------------------------------------------------------TP STARTS----------------------------------------------------------------–	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                                                        OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                Paging-v1700-IEs                                                        OPTIONAL

}

Paging-v1700-IEs ::=                SEQUENCE {

pagingRecordList-v1700              PagingRecordList-v1700                                                  OPTIONAL, -- Need N

pagingGroupList-r17                 PagingGroupList-r17                                                     OPTIONAL, -- Need N

nonCriticalExtension                Paging-v1800-IEs                                                        OPTIONAL

}

Paging-v1800-IEs ::=                SEQUENCE {

pagingRecordList-v1800              PagingRecordList-v1800                                                  OPTIONAL, -- Need N

pagingGroupList-v1800               PagingGroupList-v1800                                                   OPTIONAL, -- Need N

nonCriticalExtension              Paging-v1900-IEs                                                             OPTIONAL

}

Paging-v1900-IEs ::=                SEQUENCE {

pagingRecordList-v1900              PagingRecordList-v1900                                                  OPTIONAL, -- Need N

nonCriticalExtension            SEQUENCE {}                                                             OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v1700 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1700

PagingGroupList-r17 ::=             SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17

PagingRecordList-v1800 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1800

PagingGroupList-v1800 ::=           SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF GroupPaging-r18

PagingRecordList-v1900 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1900

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v1700 ::=              SEQUENCE {

pagingCause-r17                     ENUMERATED {voice}      OPTIONAL    -- Need N

}

PagingRecord-v1800 ::=              SEQUENCE {

mt-SDT                              ENUMERATED {true}       OPTIONAL    -- Need N

}

PagingRecord-v1900 ::=              SEQUENCE {

adaptiveRA-Resources-r19            ENUMERATED {true}       OPTIONAL    -- Need N

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

GroupPaging-r18 ::=                 SEQUENCE {

inactiveReceptionAllowed-r18        ENUMERATED {true}       OPTIONAL    -- Need N

}

-- TAG-PAGING-STOP

-- ASN1STOP

PagingRecord field descriptions

adaptiveRA-Resources Indicates whether the UE can use adaptive RA resources as specified in RandomAccessAdaptationConfig-r19. Value true indicates that the adaptive RA resources is activated upon the reception of the paging message for the period of time as configured in validityDurationForAddlRACH-Adaptation.

-----------------------------------------------------------------TP ENDS------------------------------------------------------------------

# 3	Conclusion

In this contribution we discuss how Rel-19 paging and RA resource adaptations can be jointly utilized. Based on the discussion in the previous sections we made the following observations:

Observation 1	To minimize the probability of collision using RA resources when paging occasion adaptations are configured, it would be beneficial if gNB assigns which of the paged Rel-19 UEs can trigger access via both legacy and activated adaptive RA occasions and which can trigger access via only legacy RA occasions.

Observation 2	Paging DCI can be used to indicate the activation of adaptive RA resources for all UEs monitoring that PO, but not to indicate which of those UEs can utilize the activated adaptive RA resources.

Observation 3	There is no impact on MAC layer regarding how a UE selects the adaptive RA resources, i.e. UE selects the next available RA occasion as in legacy.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Extend the paging message with a per-UE indicator that signals the availability of activated adaptive RA resources for the corresponding UE.

Proposal 2	Adopt the TP below in the running 38.331 CR.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

3GPP TS 38.304 V18.1.0 User Equipment (UE) procedures in Idle mode and in RRC Inactive state

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

R2-2502150, “Comments to 38.331 CR for NES”, Ericsson, 2025 April 7-11, Wuhan, China.

R2-2502173, “Running RRC CR for enhancements for network energy efficiency”, Ericsson, 2025 April 7-11, Wuhan, China.

R2-2503607, “Running RRC CR for enhancements for network energy efficiency”, Ericsson, 2025 May 19-23, St. Julian’s, Malta.