3GPP TSG-RAN WG2 Meeting #131	R2-2506133

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on network sided model

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2 #130, the following agreements are made regarding data collection aspects for AI/ML mobility.

Agreements For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed) … - The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling) … For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

In addition to the data collection aspect, in RAN2 #130, the following agreements are made regarding to configuration and report of inference input to network sided models for AI/ML mobility.

Agreements

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

- To support cell/beam level prediction, one enhancement is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

This contribution discusses various aspects related to the network sided model, including network-side data collection, UE-side assistance information for inference input, and performance monitoring for RRM measurement prediction use cases, among others.

# 2	Discussion

## 2.1	Network sided data collection

## 2.1.1	Data collection/logging configuration

Agreements (RAN2 #130) For the collection of data for the training of a network-sided model for RRM measurement prediction: … - The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS. …

In RAN2#130, it was agreed that logging configurations could either be integrated into measConfig or established as a separate RRC IE. Given that all mobility and RRM measurement related configurations are currently managed through measConfig, we believe the most efficient approach to incorporate measurement data logging configurations for AI mobility use cases, particularly RRM measurement prediction, would be to use measConfig. This approach minimizes the need for specification modifications and avoids the introduction of new logging related entities. Therefore, the above discussion leads to the following observation and proposal:

Observation 1: Introducing additional IEs for logging configuration would unnecessarily complicate the specification compared to leveraging the existing measConfig for AI mobility.

Proposal 1: As a principle, aim to reuse existing measConfig structure as much as possible when designing logging configurations for AI mobility use cases.

Furthermore, if the measConfig is agreed upon for incorporating logging related configurations, we propose linking the logging configuration and logging report configuration within the same IE group as MeasObject and ReportConfig (see Figure 2.1-1). This balances the trade-off between minimizing network overhead for AIML use cases and accommodating non-AIML use cases. It leverages the existing measurement reporting framework to reduce specification changes and ensures that only necessary measurements are logged and reported. Additionally, the method is adaptable for training data collection when appropriate.

Figure 2.1-1: Logging configuration

Observation 2: Incorporating logging configuration for network sided data collection should be executed with minimal specification changes.

Proposal 2: Separate parameters for logging configuration and logging report configuration are introduced as part of the same IE group as MeasObject and ReportConfig.

As previously mentioned, the configuration for data collection will primarily use two functionalities: logging configuration and logging report configuration. These elements define the logging and reporting mechanisms. We can further explore the specifics of these two introduced configurations. For the logging configuration, as raised by both AI PHY and AI mobility discussions, it is essential to include baseline elements such as buffer size, logging interval, and queuing priorities. Additionally, the configuration should specify how logging is initiated and terminated. It is also necessary to incorporate the list of cells (PCIs) into the configuration IE. Regarding the logging report configuration, it should define the reporting criteria, including potential event triggers and the content of the report. Although these specifics are typically addressed during the WI phase, we believe it is beneficial to propose a general framework for logging and reporting configuration at this stage.

Network side RRM measurement prediction, especially temporal domain prediction, typically requires a time series of past measurements as input. Such time series can be needed for both training data collection and as input for inference. For the latter, there may be more strict time and overhead constraints. In current measurement framework specification, the only solution is to use periodic reporting, which has at least the following problems:

- Each measurement period is reported individually, which adds overhead.

- To make sure enough measurement time steps are collected, the reporting condition and reportAmount parameter need to be configured so that the reporting is started early enough, even when the network knows in advance, how many measurement periods it needs as input to its ML feature. The configuration is complex to optimize and leads to inefficient configurations.

- If no relevant event occurs, the reporting of the time series may have been unnecessary.

To resolve these issues, the UE should be able to buffer and report one or more past measurement periods in one measurement report, based on reporting criteria evaluation. The log size, reporting criteria and other aspects should be configurable in the measurement configuration.

Observation 3: The UE should be able to buffer and report one or more past measurement periods in one measurement report, based on reporting criteria evaluation.

Proposal 3: For the logging configuration, the following elements should be included:

- Buffer size: The number of measurement/sampling periods the UE is expected to log.

- Queue priority: when the configured buffer size is reached, discard the oldest entry in the log or stop collecting data.

- Logging interval: Periodicity with which measurements should be logged as a multiple measurement periods.

- Logging start/end conditions: Measurement event conditions are applied when UE is expected to start/stop logging.

- PCI list for logging: A list of PCI ranges for UE to log the measurements.

Proposal 4: For the logging report, the following elements should be included:

- Report condition/criteria: For buffer-based reporting, the log is triggered to be reported as soon as the configured buffer/log size is reached. For event-triggered reporting, the log is reported when the configured measurement event condition is fulfilled.

- Report content: By default, all logged measurements are reported, but instead the UE may be configured to report an aggregation, e.g., the average of all the logged measurements.

- Other reporting configurations: Number of logged measurement reports, the cell measurement quantities to be included in the logged measurement report, and max number of non-serving cells to include in the measurement report.

## 2.1.2	Data collection/logging mechanisms

Agreements (RAN2 #130) For the collection of data for the training of a network-sided model for RRM measurement prediction: … - The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity. …

From the RAN2#130 meeting, it was agreed that the UE can be configured with an L3 event to determine when logging should be performed. Once the event conditions are met, logging occurs according to the specified periodicity. However, it is still not clear what type of L3 event to be used and whether it will involve mobility related events or other measurement specific thresholds.

Observation 4: The specific L3 event that triggers logging requires further clarification.

In our view, the existing measurement events, e.g., A1 (serving cell becomes better than absolute threshold), A2 (serving cell becomes worse than absolute threshold), and A3 (neighbor becomes offset better than SpCell), can be reused with related configurations, such as reportInterval and reportAmount as specified in TS 38.331. In addition, s-MeasConfig establishes a threshold for NR SpCell RSRP measurement, determining when the UE must perform measurements on non-serving cells. Using this threshold for logging initiation ensures the collection of meaningful data related to mobility, encompassing both serving and neighbouring cells.

Observation 5: Initiating data collection at the UE using the s-MeasConfig threshold can ensure that measurements from both serving and neighbouring cells are included.

Additionally, a dedicated indication from the network may be employed to trigger data collection at the UE, using a mechanism similar to semi-persistent CSI-RS reporting with DCI or MAC CE as per TS38.214. Upon receiving this indication, the UE can choose to either log or report the measurement results based on its specific logging requirements. However, further discussion is required for RAN2.

Proposal 5: Consider the following mechanism to initiate and terminate the data logging for AI mobility use cases:

- Event triggered logging: measurement Event A* or s-MeasureConfig threshold can be reused.

- Network indicated logging: an existing or dedicated DL indication signal can be used.

## 2.1.3	Logging interval

In legacy logged MDT operations, when the report type is periodic, the UE logs data at the configured logging intervals. Similarly, for periodic RRM measurement logging, which is based on CSI-RS or SSB, the UE can log data at each interval, with the minimum logging interval corresponding to the transmission periodicity of the CSI-RS or SSB.

Proposal 6: For periodic logging, the minimum logging interval should be aligned with the CSI-RS/SSB transmission periodicity.

As discussed above, the current RAN2 AI mobility discussions and previous AI PHY agreements do not fully address how to prioritize which measurement data should be logged and the corresponding logging intervals when data importance varies. Given that the current study item spans multiple use cases and UE may operate in diverse radio environments, the UE may accumulate low‐priority or irrelevant data for network‐side model training, inference or monitoring. For example, data collected when a UE is with lower speed or is near the cell center may be considered low priority and lose its relevance for event prediction as an A3 trigger approaches. This not only degrades the overall performance of predictions but also forces the initiation of additional data collection efforts, which yields little benefit. Furthermore, transmitting non-critical data to the network consumes memory and signalling resources on the NW side, while unnecessarily occupying over-the-air bandwidth.

Observation 6: Considering the variety of radio environments in which the UE collects measurements, it is important to prioritize specific data types for logging.

Several prioritization aspects can be considered for data collection by the UE. For instance, prioritization can be determined based on UE radio conditions, such as measurements taken at the cell edge or in higher speed categories, which should be assigned higher priority. Additionally, one of the most critical use cases for AI/ML-based RRM measurement prediction is assessing the accuracy of AI/ML in predicting the frequency domain. As the frequency gap between two bands increases, the differences in radio propagation characteristics become more pronounced, making prediction more challenging. Consequently, a larger frequency gap relative to the serving cell carrier should be considered for higher prioritization

Proposal 7: RAN2 to consider different data prioritization levels for logging intervals to be used at the UE.

## 2.1.4	Report mechanism

Agreements (RAN2 #130) For the collection of data for the training of a network-sided model for RRM measurement prediction: … - The UE sends availability indication of collected logged data via UAI or RRCReconfigurationComplete message for HO case. - The availability indication can be triggered due to: - full buffer being reached (if configured), (FFS if buffer is per use case) - buffer threshold being reached (if configured), or (FFS if buffer is per use case) - low power (if configured) - Upon sending the availability indication, UE indicates: - Data is available - Reason for the triggering of the availability indication (full buffer. Threshold) - Low power indication …

From the RAN2 #130 meeting, it was agreed that the UE sends an availability indication of collected logged data via the UAI or RRCReconfigurationComplete message during handover scenarios. This availability indication can be triggered by reaching a full buffer, or by reaching a buffer threshold, or by low power conditions. Upon sending the availability indication, the UE specifies the availability of data, the reason for triggering the availability indication, such as a full buffer or threshold, or a low power indication.

However, in our view, sending an indication when the UE buffer is already full risks data loss due to overwriting, as the UE may continue logging data while the network collects existing buffer data. To address this, it is important to clarify the implications for different UEs based on buffer sizes and management logic when indicating before the buffer is full. Additionally, frequent data availability indicators for minimal data in the buffer may prove inefficient.

Observation 7: Triggering the availability indication when a buffer threshold is met is more robust (minimizing loss of data) than waiting for the buffer to be completely full.

Furthermore, when the UE is configured with logging intervals of different lengths and buffers to collect data with differing priorities, relying on a single buffer threshold to trigger reporting is unsuitable. To address this issue, the network can configure multiple buffer thresholds that align with different logging intervals and buffer sizes for triggering reporting. This approach ensures more efficient handling of data prioritization and reporting.

Proposal 8: RAN2 to consider the use of different buffer thresholds to trigger the reporting of logged data based on different priority levels.

## UE behavior during HO, IDLE/ INACTIVE and RLF

Agreements (RAN2 #130) For the collection of data for the training of a network-sided model for RRM measurement prediction: … h) The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO). - i) The UE releases the collected data upon transitioning to IDLE/INACTIVE j) For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found …

During the RAN2 #130 meeting, it was agreed that the UE should retain collected data during handover unless the network explicitly instructs it to release the data, such as during handover scenarios. The UE is expected to release the collected data when transitioning to IDLE/INACTIVE states.

For AI mobility use cases, as previously discussed, the UE may be configured with varying logging intervals and buffer sizes to manage collected data with different priorities, especially in response to changes in radio conditions or the need to perform inter-frequency measurements. Therefore, a simplistic binary approach to either release or retain logged data during handover, idle/inactive states, and RLF may not be sufficient to handle the prioritization. According to the current agreement, flushing all collected data during handover execution risks losing critical information needed to re-establish the connection with the target cell. Conversely, retaining all data, particularly low-priority data, could lead to inefficient memory usage by the UE.

Observation 8: Retaining all data, particularly low-priority data, could lead to inefficient memory usage by the UE.

Observation 9: AI mobility studies and RLF processes that automatically discard data during UE transitions to RLF could result in the loss of critical features necessary for data collection.

Proposal 9: RAN2 to discuss whether the UE can retain/discard data during handover based on their importance.

## 2.2	Inference input to the network sided model

During the RAN2 #130 meeting, it was agreed that the legacy RRM measurement configuration and reporting framework can be utilized for network-side model and RRM measurement prediction use cases. To enhance the analysis of inference input aspects and identify specific gaps and areas for investigation, we have summarized the inference input options for each sub-use case.

Table 2.2-1: RRM measurement prediction per use case analysis for inference input

| Use case | Sub-use case | Temporal domain | Frequency domain | Spatial domain |
| --- | --- | --- | --- | --- |
| Cell-level prediction | 1 | Time-series of L1 filtered beam level measurements | L1 filtered beam level measurements of frequency 1 | Set B of L1 filtered beam level measurements |
|  | 2 | Time-series of L3 filtered cell level measurements | L3 filtered cell level measurements of frequency 1 | N/A |
|  | 3 | Time-series of L1 filtered beam level measurements | L1 filtered beam level measurements of frequency 1 | Set B of L1 filtered beam level measurements |
| Beam-level prediction | 4 | Time-series of L1 filtered beam level measurements | L1 filtered beam level measurements of frequency 1 | Set B of L1 filtered beam level measurements |
|  | 5 | Time-series of L3 filtered beam level measurements | L3 filtered beam level measurements of frequency 1 | N/A |
|  | 6 | Time-series of L1 filtered beam level measurements | L1 filtered beam level measurements of frequency 1 | Set B of L1 filtered beam level measurements |

Note: Frequency 1 denotes the carrier frequency at which measurements are taken for inference input, whereas the model output corresponds to measurements taken at Frequency 2.

Proposal 10: Capture the overview of inference input to the network sided model in Table 2.2-1 in TR38.744.

In the existing measurement report configuration, specifically the measResultsNR IE, both cell-level measurements and L3 filtered beam-level results are incorporated. As per the consensus reached in the last meeting, L1 filtered beam-level results can be accommodated within the current specification by configuring the L3 filter coefficient to zero. Thus, all sub-use cases can be addressed by extending the current specification accordingly.

MeasResultsNR

PCI

MeasResult

CellResults

…

BeamResults / RSIndexResults

…

Figure 2.2-1: Measurement results IE in TS38.331 including cell level results and beam level results

Observation 10: The current L3 RRM measurement report framework supports both cell-level and beam-level reporting, but it captures measurements at only a single point in time.

Based on the above summary in Figure 2.2-1, we propose the following overall considerations on RRM measurement prediction use cases for the network sided model:

Proposal 11: Extend the current L3 measurement report framework for the UE configuration and report of inference input to network sided models in WI:

- Temporal domain prediction: L1 filtered beam-level RSRP, L3 beam-level RSRP, and L3 cell-level RSRP of one cell at multiple time instances;

- Frequency domain prediction: L1 filtered beam-level RSRP, L3 beam-level RSRP, and L3 cell-level RSRP of one or more cells on specific frequency(s);

- Spatial domain prediction: L1 filtered beam-level RSRP of one cell.

# 3	Conclusion

This document has made the following observations:

Observation 1: Introducing additional IEs for logging configuration would unnecessarily complicate the specification compared to leveraging the existing measConfig for AI mobility.

Observation 2: Incorporating logging configuration for network sided data collection should be executed with minimal specification changes.

Observation 3: The UE should be able to buffer and report one or more past measurement periods in one measurement report, based on reporting criteria evaluation.

Observation 4: The specific L3 event that triggers logging requires further clarification.

Observation 5: Initiating data collection at the UE using the s-MeasConfig threshold can ensure that measurements from both serving and neighbouring cells are included.

Observation 6: Considering the variety of radio environments in which the UE collects measurements, it is important to prioritize specific data types for logging.

Observation 7: Triggering the availability indication when a buffer threshold is met is more robust (minimizing loss of data) than waiting for the buffer to be completely full.

Observation 8: Retaining all data, particularly low-priority data, could lead to inefficient memory usage by the UE.

Observation 9: AI mobility studies and RLF processes that automatically discard data during UE transitions to RLF could result in the loss of critical features necessary for data collection.

Observation 10: The current L3 RRM measurement report framework supports both cell-level and beam-level reporting, but it captures measurements at only a single point in time.

And proposed the following:

Proposal 1: As a principle, aim to reuse existing measConfig structure as much as possible when designing logging configurations for AI mobility use cases.

Proposal 2: Separate parameters for logging configuration and logging report configuration are introduced as part of the same IE group as MeasObject and ReportConfig.

Proposal 3: For the logging configuration, the following elements should be included:

- Buffer size: The number of measurement/sampling periods the UE is expected to log.

- Queue priority: when the configured buffer size is reached, discard the oldest entry in the log or stop collecting data.

- Logging interval: Periodicity with which measurements should be logged as a multiple measurement periods.

- Logging start/end conditions: Measurement event conditions are applied when UE is expected to start/stop logging.

- PCI list for logging: A list of PCI ranges for UE to log the measurements.

Proposal 4: For the logging report, the following elements should be included:

- Report condition/criteria: For buffer-based reporting, the log is triggered to be reported as soon as the configured buffer/log size is reached. For event-triggered reporting, the log is reported when the configured measurement event condition is fulfilled.

- Report content: By default, all logged measurements are reported, but instead the UE may be configured to report an aggregation, e.g., the average of all the logged measurements.

- Other reporting configurations: Number of logged measurement reports, the cell measurement quantities to be included in the logged measurement report, and max number of non-serving cells to include in the measurement report.

Proposal 5: Consider the following mechanism to initiate and terminate the data logging for AI mobility use cases:

- Event triggered logging: measurement Event A* or s-MeasureConfig threshold can be reused.

- Network indicated logging: an existing or dedicated DL indication signal can be used.

Proposal 6: For periodic logging, the minimum logging interval should be aligned with the CSI-RS/SSB transmission periodicity.

Proposal 7: RAN2 to consider different data prioritization levels for logging intervals to be used at the UE.

Proposal 8: RAN2 to consider the use of different buffer thresholds to trigger the reporting of logged data based on different priority levels.

Proposal 9: RAN2 to discuss whether the UE can retain/discard data during handover based on their importance.

Proposal 10: Capture the overview of inference input to the network sided model in Table 2.2-1 in TR38.744.

Proposal 11: Extend the current L3 measurement report framework for the UE configuration and report of inference input to network sided models in WI:

- Temporal domain prediction: L1 filtered beam-level RSRP, L3 beam-level RSRP, and L3 cell-level RSRP of one cell at multiple time instances;

- Frequency domain prediction: L1 filtered beam-level RSRP, L3 beam-level RSRP, and L3 cell-level RSRP of one or more cells on specific frequency(s);

- Spatial domain prediction: L1 filtered beam-level RSRP of one cell.