3GPP TSG-RAN WG2 Meeting #131	R2-2506151

Bengaluru, India, 25 – 29 August 2025                                       Revision of R2-2504617

Agenda item:	8.9.2

Source:	Sateliot, Thales, Novamint

Title:	On cell (re)selection and idle mode task relaxation for S&F Satellite operation

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

As part of the 5GSAT_Ph3-ARC WID, SA2 agreed a core solution to support Store and Forward (S&F) operation as specified mainly in [1] – the main highlights are outlined below with emphasis on the expected UE behaviour when receiving a S&F Wait Timer and/or a S&F Monitoring List:

- The Store and Forward Satellite operation applies in E-UTRAN with satellite access and is suitable for delay-tolerant communication services (e.g. CIoT/MTC, SMS, etc.).

- When an on-board MME supports S&F Satellite operation and is operating in S&F Mode, the on-board eNB broadcasts an indication of operating in the S&F Mode, which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in the S&F Mode and if a UE is enabled for S&F Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

- The MME may optionally provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure.

- When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause

- How a UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE accesses a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

- The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNB that does not broadcast an indication of operating in the S&F Mode.

A more detailed discussion on the intended use of the “S&F Monitoring List” and “S&F Wait Timer” in a deployment based on the split-MME architecture is provided in R2-2504617 [4]. The main objective of the discussion is to stress that the use of these two features is not motivated by UE power saving but mainly intended to prevent UE from accessing satellites with no UE context or where the UE context has not been yet updated/synchronized.

In a different front, RAN WID on IoT_NTN_Ph3 [3] indicated the following in relation to supporting S&F operations although any impact to ME is generally within the scope of this WID:

- Strive to minimise UE impact

In a recent RAN #130, agreement was reached on the following points:

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in the S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

Some remaining issues as identified in RAN2 #130 especially in relation to the S&F mode of operation are:

- Cell (re)selection enhancements to consider “S&F Monitoring List” and “S&F Wait Timer”

- Relaxation of idle mode tasks in a S&F network deployment exhibiting discontinuous coverage

- WA on introducing the S&F mode indication in the neighbour cell list

- Aligning the use of satellite IDs

- Scope and uniqueness of satelliteId.

This paper covers issues i), ii) and iii).

Issues iv) and v) are covered in R2-2506152.

# 2	Discussion

2.1      Cell (Re)Selection enhancements

The following discussion was held in RAN2#130:

Several illustrative use cases are described in the following to further assess the need or benefits associated with a potential modification of the definition of suitable cell.

Case A – Satellite PLMN with all satellites operating in S&F.

The UE is registered in a satellite PLMN. The PLMN consists of 4 NGSO satellites with SatelliteID=1, 2, 3 and 4. All the satellites operate in the S&F mode all the time.

The UE has completed a NAS exchange (e.g. attach procedure) with satelliteID#1. The UE has received a “S&F Wait Timer” and a “S&F Monitoring List={satelliteID#1, satelliteID#2}”. SatelliteID#1 has moved away and no coverage is available from that satellite. The UE has MO data to send.

Case A1:

While the “S&F Wait Timer” is running, the UE performs a cell search and detects a cell from satelliteID#2 operating in S&F mode. In such a situation, the UE should not try to get access to the detected cell because of the S&F Wait Timer, even though the satelliteID#2 appears in the provided S&F monitoring list.

- Q.A1. Does this need any modification into the cell selection/re-selection procedure or can it be handled at the NAS layer?

- Our assessment: as of now, current 3GPP standard does not prevent the UE from camping on the cell of satelliteID#2 that eventually turns out to be unsuitable from NAS perspective (i.e. NAS layer should not trigger any NAS operation in a cell operating in S&F model while the S&F Wait Timer is running).

This results in unnecessary energy waste on the UE side whose AS layer may end up camping on a cell that cannot be actually be used by the NAS layer.

To overcome this situation, the following two solution approaches can be envisaged:

- Solution approach #1: the AS declaring the cell of SatelliteID#2 as unsuitable so that the UE can look for other appropriate cells without tying up with an unsuitable cell operated by satelliteID#2 as the UE has MO data to send. This requires enhancing the definition of suitable cell in 36.304. It is FFS whether NAS needs to provide S&F monitoring list content to the AS for cell (re)selection.

- Solution approach #2: the NAS declaring the cell of satelliteID#2 as unsuitable and pass that information on the AS so that the AS can look for other appropriate cells. It is FFS whether the AS needs to provide any information for the NAS to decide.

Case A2:

While the “S&F Wait Timer” is still running, the UE performs another cell search and detects a cell from satelliteID#3 operating in S&F mode. In such a situation, the UE should not try to get access to the detected cell because (1) the “S&F Wait Timer” is still running and (2) SatelliteID#3 is not included in the provided S&F Satellite List .

- Q.A2. Does this need any modification into the cell selection/re-selection procedure or can it be handled at the NAS layer?

- Our assessment: Similar situation as Case A1 arises. Either Solution approach #1 or Solution approach #2 needs to be standardized to overcome it.

Case A3:

When the “S&F Wait Time” has expired, the UE performs cell search and detects a cell from satelliteID#4 operating in S&F mode. In such a situation, the UE should not try to access that cell because SatelliteID#4 is not included in the provided S&F Satellite List.

- Q.A3. Does this need any modification into the cell selection/re-selection procedure or can be handled at the NAS layer?

- Our assessment: Similar situation as Case A1 arises. Either Solution approach #1 or Solution approach #2 needs to be standardized to overcome it.

Case A4:

Later on, the UE performs a new cell search and detects (1) a cell in S&F mode from satelliteID#2 and (2) a cell in S&F mode from satelliteID#3. The cell from satelliteID#3 is received with better signal strength so that the cell of satelliteID#3 is preferred from the AS layer perspective. However, because of the “S&F Monitoring List”, the UE should camp on the cell from Satellite#2 and start the MO transfer there.

- QA.4. Does this need any modification into the cell selection/re-selection procedure or can be handled at the NAS layer?

- Our assessment: as of now, current 3GPP standard does not prevent a UE from camping on satelliteID#3.

Therefore, the situation is critical at this point, given the AS layer may keep camping on satelliteID#3 cell which is not usable from NAS perspective and not be able to use satelliteID#4 cell which is the valid one from NAS perspective.

In order to avoid case A4, either Solution approach #1 or Solution approach #2 needs to be standardized.

Case B – Satellite PLMN with a subset of satellites operating in S&F.

The UE is registered in a satellite PLMN. The PLMN consists of 4 NGSO satellites with SatelliteID=1,2, 3 and 4. At the UE location, satelliteID#1 and satelliteID#2 operate in the S&F mode while satelliteID#3 and satelliteID#4 are able to operate in normal mode.

The UE has completed a NAS exchange (e.g. attach procedure) with satelliteID#1. The UE has received a “S&F Wait Timer” and a “S&F Monitoring List={satelliteID#1}”. SatelliteID#1 has moved away and no coverage is available from that satellite. The UE has MO data to send.

Case B1:

While the “S&F Wait Timer” is running, the UE performs a cell search and detects a cell from satelliteID#3. In such a situation, the UE can use the cell from satelliteID#3 given the “S&F Wait Timer” does not apply to satellite cells not operating in the S&F mode.

- Q.B1. Does this need any modification into the cell selection/re-selection procedure or can be handled at the NAS layer?

- Assessment: no AS enhancement is required in terms of cell (re)selection

Case B2:

Let’s assume that the UE has succeeded to send the MO data via satelliteID#3, and satelliteID#3 moves away. Now the UE has new MO data to send.

When the “S&F Wait Time” has expired, the UE performs cell search and now detects a cell from satelliteID#2 operating in S&F mode. In such a situation, the UE should not try to access that cell because SatelliteID#2 is not included in the S&F Monitoring List.

- Q.B2. Does this need any modification into the cell selection/re-selection procedure or can be handled at the NAS layer?

- Assessment: Similar situation as case A1. In order to avoid Case B2, solution approach #1 or solution approach #2 may be required.

Case B3:

Later on, the UE performs a new cell search and detects (1) a cell in S&F mode from satelliteID#1 and (2) a cell in normal mode from satelliteID#4. The cell from satelliteID#1 is a valid cell (belongs to the S&F monitoring List) and is received with a slightly better signal strength so that that cell is preferred from AS layer perspective. However, cell from satelliteID#4 is also a valid cell given it does not operate in the S&F mode and could be preferred from a NAS/Service perspective.

- Q.B3. Does this need any modification into the cell selection/re-selection procedure or can be handled at the NAS layer?

- Assessment: as of now, current 3GPP standard does not prevent the UE from camping on the cell of satelliteID#1 operating in S&F so that the UE may not benefit of using the cell operating in normal mode.

This may require changes to the existing cell (re)selection mechanisms available at both the NAS and the AS levels in order to be able to enforce preferences with regard to the selection of cells in S&F mode or normal mode when radio conditions are both suitable.

Observation 1: In multiple cases, as per current cell (re)selection specs, the UE may end up camping on a cell that turns out to be unsuitable from the NAS perspective for S&F Satellite operation - thus, a UE wasting scarce energy.

Observation 2: Situation is even critical in some cases (see Case A4), where, as per current specs, the UE may end up camping on a cell that turns out to be unsuitable from the NAS perspective even in the presence of neighbour cells that are not preferred from a AS perspective but are suitable from a NAS perspective for S&F satellite operation.

Proposal 1: RAN2 is respectfully requested to consider the following solution approaches to overcome the observed situations:

- Solution approach #1: the AS declaring a cell that is not usable from NAS perspective for S&F Satellite operation as unsuitable so that the UE can look for other appropriate cells. This requires enhancing the definition of suitable cell in 36.304. It is FFS whether NAS needs to provide S&F monitoring list content to the AS for cell (re)selection.

- Solution approach #2: the NAS declaring whether a cell is suitable or unsuitable from NAS perspective for S&F Satellite operation and instruct the AS to proceed accordingly so that the AS is not tied up with an un     suitable cell from NAS perspective and can look for other appropriate cells. It is FFS whether the AS needs to provide any information for the NAS to decide.

## 2.2      Relaxation of idle mode tasks in a S&F network deployment exhibiting discontinuous coverage.

The following discussion was held in RAN2#130:

Existing support for discontinuous coverage allows UEs in RRC_IDLE state to make use of provisioned satellite assistance information for the purpose of saving energy during periods of no coverage. Such satellite assistance information is signalled in an NTN cell, for example, using SIB32/SIB32-NB and SIB33/SIB33-NB.

It is possible that satellites belonging to an S&F constellation of a PLMN can provide discontinuous coverage. It was left to FFS whether the UE needs to take into consideration the NAS configured S&F monitoring list as part of the discontinuous coverage procedure as applicable to a UE in  idle mode.

To assess this need, let’s consider the same illustrative cases A and B described in the previous section 2.1 and let’s assume that the networks exhibit discontinuous coverage and broadcast SIB32 with the TLE information of all the satellites so that the UE can predict the coverage windows.

Case A – Satellite PLMN with all satellites operating in S&F.

In this case, from SIB32 contents, the UE will know which satellite is going to provide a given coverage window.

Let’s imagine that at some point in time the UE is out-of-coverage and the next coverage window is computed/predicted to be provided by a satellite that it’s not in the “S&F Monitoring List”. In such a situation,  the UE may skip that coverage window (i.e. the UE does not need to activate the AS layer) if the UE knows that the satellite is operating in the S&F mode (which is certain in this case given all satellites always operate in the S&F mode) and because that satellite is not in the list.

Therefore, in this scenario, relaxation of idle mode tasks (i.e., there is no need for the UE to activate AS layer) provides some benefit, avoiding the UE to wake up unnecessarily.

Case B – Satellite PLMN with a subset of satellites operating in S&F.

In this case, from SIB32 contents, the UE will also know which satellite is going to be within coverage at a given time.

Let’s imagine that at some point in time the UE is out-of-coverage and the next coverage window is computed/predicted to be provided by a satellite that it’s not in the “S&F Monitoring List”. Unlike in Case A, in such a situation,  the UE may NOT skip that coverage opportunity because the UE does not know whether the incoming satellite is operating in the S&F mode or not. So, in this case, the UE needs to wake up, detect the cell and check whether it is operating in the S&F mode. If the cell is NOT operating in the S&F mode, the UE can access the cell. If the cell is operating in the S&F mode, the UE may go back to sleep mode given the satellite ID of the serving cell is not within the “S&F Monitoring List”.

Therefore, in this scenario, relaxation of idle mode tasks (i.e., UE not having to activate AS layer) cannot be applied by default. This is because the UE does not know whether a cell is operating in the S&F mode or not.

From the above analysis, the following Observation and proposals can be made:

Observation 3: Relaxation of idle mode tasks during discontinuous coverage based on the “S&F Monitoring List” cannot be applied unless the UE knows in advance (i.e., before camping on the cell) whether a satellite is going to be operating in the S&F mode or normal mode.

Proposal 2: RAN2 is respectfully requested to consider introducing an indication on whether a satellite is operating in the S&F mode in SIB32.

Proposal 3: When a S&F monitoring list is configured by upper layers, if a UE is out of coverage and the UE can determine that a future coverage window opportunity is provided by a satellite that (1) it’s not in the S&F Monitoring List and (2) operates in S&F mode, the UE may not need to perform any idle mode tasks related to NTN during such coverage window opportunity.

## 2.3      WA on introducing an S&F mode indication in the neighbour cell list

The following discussion and WA were held in RAN2#130:

Open points in the WA are:

- Open point#1. UE behaviour

- Open point#2. Including the transition time along with the S&F mode indication

Introducing an S&F mode indication in the neighbour cell list shall help a Rel-19 UE select a suitable cell as part of its cell (re)selection procedure after taking necessary idle mode measurements and applying cell selection criteria as specified in [5]. This means that introducing such an indication in the neighbour cell list can equally be helpful to those UEs supporting the S&F mode and those that do not support the S&F mode.

For a UE supporting the S&F mode, the newly added indication is helpful in prioritizing cells as part of cell (re)selection that are meeting either of the following:

- In case a neighbour cell operates in the S&F mode, whether the corresponding satellite ID is included in the S&F monitoring list; or

- A neighbour cell operates in the normal mode.

Observation 4: Inclusion of an S&F mode indication in the neighbour cell list introduces a new UE behaviour for those Rel-19 UEs supporting the S&F mode at least in terms of cell (re)selection.

Please note that this new Rel-19 UE behaviour relies on the use of either Solution approach #1 or Solution approach #2 as discussed in section 2.1 above.

For a UE not supporting the S&F mode, the newly added indication is helpful in staying away from cells operated by satellites supporting the S&F mode.

Observation 5: Inclusion of an S&F mode indication in the neighbour cell list introduces a new Rel-19 UE behaviour for those Rel-19 UEs not supporting the S&F mode at least in terms of cell (re)selection.

Proposal 4: Given that the introduction of an S&F mode indication in the neighbour cell list introduces new UE behaviour at least in terms of cell (re)selection, RAN2 is respectively requested to make the below WA an agreement.

WA: In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

In relation to open point#2, it is again important to note that it is possible for some satellites to switch their operating mode (i.e., between a normal mode and the S&F mode) along their orbital enroute depending at least on service requirements of UEs in a given location, satellite capabilities (e.g., able to support service and feeder links simultaneously), or availability of gateways. For the same reasons mentioned above, including the transition time along with the S&F mode indication can be useful.

Proposal 5: RAN2 is respectively requested to consider including transition time in the neighbour cell list.

# 3	Conclusion

This paper has analysed the remaining open issues as identified as of RAN2 #130 in the context of S&F operation and makes the following observations and proposals:

Cell (re)selection and S&F:

Observation 1: In multiple cases, as per current cell (re)selection specs, the UE may end up camping on a cell that turns out to be unsuitable from the NAS perspective for S&F Satellite operation - thus, a UE wasting scarce energy.

Observation 2: Situation is even critical in some cases (see Case A4), where, as per current specs, the UE may end up camping on a cell that turns out to be unsuitable from the NAS perspective even in the presence of neighbour cells that are not preferred from a AS perspective but are suitable from a NAS perspective for S&F satellite operation.

Proposal 1: RAN2 is respectfully requested to consider the following solution approaches to overcome the observed situations:

- Solution approach #1: the AS declaring a cell that is not usable from NAS perspective for S&F Satellite operation as unsuitable so that the UE can look for other appropriate cells. This requires enhancing the definition of suitable cell in 36.304. It is FFS whether NAS needs to provide S&F monitoring list content to the AS for cell (re)selection.

- Solution approach #2: the NAS declaring whether a cell is suitable or unsuitable from NAS perspective for S&F Satellite operation and instruct the AS to proceed accordingly so that the AS is not tied up with an un     suitable cell from NAS perspective and can look for other appropriate cells. It is FFS whether the AS needs to provide any information for the NAS to decide.

Relaxation of Idle mode tasks in discontinuous coverage:

Observation 3: Relaxation of idle mode tasks during discontinuous coverage based on the “S&F Monitoring List” cannot be applied unless the UE knows in advance (i.e., before camping on the cell) whether a satellite is going to be operating in the S&F mode or normal mode.

Proposal 2: RAN2 is respectfully requested to consider introducing an indication on whether a satellite is operating in the S&F mode in SIB32.

Proposal 3: When a S&F monitoring list is configured by upper layers, if a UE is out of coverage and the UE can determine that a future coverage window opportunity is provided by a satellite that (1) it’s not in the S&F Monitoring List and (2) operates in S&F mode, the UE may not need to perform any idle mode tasks related to NTN during such coverage window opportunity.

S&F mode indication in the neighbour cell list:

Observation 4: Inclusion of an S&F mode indication in the neighbour cell list introduces a new UE behaviour for those Rel-19 UEs supporting the S&F mode at least in terms of cell (re)selection.

Observation 5: Inclusion of an S&F mode indication in the neighbour cell list introduces a new Rel-19 UE behaviour for those Rel-19 UEs not supporting the S&F mode at least in terms of cell (re)selection.

Proposal 4: Given that the introduction of an S&F mode indication in the neighbour cell list introduces new UE behaviour at least in terms of cell (re)selection, RAN2 is respectively requested to make the below WA an agreement.

WA: In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

Proposal 5: RAN2 is respectively requested to consider including transition time in the neighbour cell list.

# Reference

- TS 23.401, General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access, V19.2.0 (2025-03).

- S2-2502450, Reply LS on Satellite IDs for store-and-forward operation, Greece, Feb. 2025.

- RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek, Sep. 2025.

- R2-2504617, Discussion on “S&F Monitoring List” and “S&F Wait Timer” and potential RAN2 impacts, Sateliot, Malta, May. 2025.

- TS 36.304, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode, V18.4.0, (2025-06)

- TS 23.122, Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode, V19.3.0 (2025-06).

- TS 24.301, Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3, V19.2.0 (2025-03).

- TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification, V18.5.0 (2025-04)