3GPP TSG-RAN WG2 Meeting #131  	                            R2-2506170

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.5.3

Source: 	CEWiT

Title:	Discussion on Additional Aspects of OD-SIB1 Support

Document for:	Discussion

# Introduction

In RAN2#130 [1],  following agreements were reached on several aspects of OD-SIB1 handling:

Agreements on OD-SIB1

- Not to include access related information of NES into WUS configuration.

- Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed. RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

- TP1 in R2-2503636 is agreed with the removal of “until receiving SIBxx from this cell”.

- Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

- If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

- Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

However, there remain additional points where clarification and specification support can further improve UE behavior, signaling efficiency, and consistency across deployment scenarios. This contribution discusses UL carrier support for OD-SIB1 request, SIB1 request after connection release and MIB reacquisition for the request.

# Discussion

2.1. UL Carrier for SIB1 Request

In RAN2#130 meeting, an issue on whether to support OD-SIB1 request on SUL or not was  disucssed. The issue was not handled in RAN1 before, hence this issue can be discussed and decided in RAN2. Cells that support on-demand SIB1 may be configured with both Supplementary Uplink (SUL) and Normal Uplink (NUL) carriers. Therefore, the SIB1 request procedure should allow use of both uplink type in such cells. The UE can determine the UL carrier prior to sending a SIB1 request. Since SIB1 request uses the random access procedure, and UL carrier selection already occurs at RA initiation in legacy operation, there is no additional complexity in enabling both SUL and NUL for SIB1 requests. Further not allowing UE to send OD-SIB1 request on SUL, may cause an SUL capable UE to hop between SUL and NUL thereby increases the processings at UE and delays the SIB1 aquisition.

Proposal 1: OD-SIB1 request should be supported both on SUL and NUL.

2.2. MIB Handling for Kssb-Based OD-SIB1 Decisions

It was agreed in RAN1#118b [2], that Kssb field in MIB informs whether OD-SIB1 should be triggered in the current cell. In normal operation, changes to SI parameters, including Kssb, are indicated via SI change notification. This allows the UE to maintain accurate system information without autonomous, periodic reacquisition of MIB. From a specification design perspective, autonomous MIB reacquisition without explicit notification would increase UE power consumption and control channel usage, contradict the established SI change model where updates are network-triggered and introduce operational unpredictability in UE behavior. By relying on the last acquired MIB and only reacquiring it upon SI change notification, UE behavior remains predictable, power-efficient, and consistent with legacy SI handling principles.

Proposal 2: For Kssb-based OD-SIB1 initiation checks in a NES cell, the UE should rely on the Kssb from the most recently acquired MIB and reacquire it only when triggered by an SI change notification from the network.

2.3. SIB1 Request Upon Connection Release

An RRC_CONNECTED UE may not retain a valid SIB1 request configuration for another cell after a certain period, when it was originally obtained from SIB X of the connected cell. Following connection release, if the UE selects this other cell as the target, reusing an outdated configuration could create ambiguity in OD-SIB1 request behavior, potentially resulting in failed or unnecessary requests. This leads to wasted UE power and network resources. By introducing a step for re-acquiring the relevant configuration particularly the WUS configuration within a window before connection release, the UE can ensure it is working with the most up-to-date parameters before attempting an OD-SIB1 request. This avoids the risk of using outdated configuration data and ensures accurate and efficient SI acquisition. Allowing blind reuse of previously obtained SIB1 request configurations after connection release would negatively impact efficiency and UE performance. A re-acquisition approach removes these uncertainties.

Proposal 3: Upon connection release and reselection to a target cell, the UE should re-acquire the SIB1 request configuration for other cells instead of reusing the configuration obtained from connected cell’s SIB X.

# Conclusion

We propose RAN2 to discuss following proposal:

Proposal 1: OD-SIB1 request should be supported both on SUL and NUL.

Proposal 2: For Kssb-based OD-SIB1 initiation checks in a NES cell, the UE should rely on the Kssb from the most recently acquired MIB and reacquire it only when triggered by an SI change notification from the network.

Proposal 3: Upon connection release and reselection to a target cell, the UE should re-acquire the SIB1 request configuration for other cells instead of reusing the configuration obtained from connected cell’s SIB X.

# References

1. Report of 3GPP TSG RAN WG2 meeting #130, May, 2025.

2. Chairman Notes, RAN1#118b, October, 2024