3GPP TSG-RAN WG2 Meeting #131	R2-2506171

# Bengaluru, IN, 25–29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 1023 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 Running CR for NR NTN phase 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of Release 19 features for Non-Terrestrial Networks |  |  |  |  |  |  |  |  |
| Summary of change: |  | This running CR captures the RAN2 agreements for support NR NTN phase 3 in Rel-19. It will be updated as long as WI progresses and texts proposals from the other WGs. It includes the following features: - Downlink Coverage enhancements (TP from RAN1 R1-2504934) - Support of broadcast service (RAN2 agreements as far as RAN2#130 as discussed at RAN#130 : “=> We can consider RAN2 specific changes according to option 2 on top of what agreed by RAN3”) - NR NTN uplink capacity enhancement (TP from RAN1 R1-2504934) - Support of half-duplex UE in NR NTN i.e. RedCap and eRedcap (TP from RAN1 R1-2504934) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for Release 19 NR NTN phase 3 enhancements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 7.3.1 16.14.9, 16.14.X (new), 16.14.Y (new), 16.14.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213 CR 0712 TS 38.214 CR 0680 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.212 CR 0226 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS 38.211 CR 0154 TS 38.331 CR xxxx TS 38.304 CR yyyy TS 38.306 CR zzzz |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2502192 submitted to RAN2#129bis R2-2504630 submitted to RAN2#130 R2-2504761 submitted to RAN2#130 |  |  |  |  |  |  |  |  |

First Modified Subclause

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A2X	Aircraft-to-Everything

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

AR	Augmented Reality

ARP	Allocation and Retention Priority

ATG	Air to Ground

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BFD	Beam Failure Detection

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

BRID	Broadcast Remote Identification

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

cellDTRX-RNTI	Cell Discontinuous Transmission and Reception RNTI

CFR	Common Frequency Resource

CFRA	Contention Free Random Access

CG	Configured Grant

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

CSS	Common Search Space

DAA	Detect And Avoid

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DCR	Direct Communication Request

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

DSR	Delay Status Report

DTX	Discontinuous Transmission

E-CID	Enhanced Cell-ID (positioning method)

EC	Energy Cost

EHC	Ethernet Header Compression

ePWS	enhancements of Public Warning System

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

FSA ID	Frequency Selection Area Identity

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network selection

GNSS	Global Navigation Satellite System

GSO	Geosynchronous Orbit

H-SFN	Hyper System Frame Number

HAPS	High Altitude Platform Station

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

ISA	Intended Service Area

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

LBT	Listen Before Talk

LDPC	Low Density Parity Check

LEO	Low Earth Orbit

LTM	L1/L2 Triggered Mobility

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

ML	Machine Learning

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MO-SDT	Mobile Originated SDT

MP	Multi-Path

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MT-SDT	Mobile Terminated SDT

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

N3C	Non-3GPP Connection

NB-IoT	Narrow Band Internet of Things

NCD-SSB	Non Cell Defining SSB

NCGI	NR Cell Global Identifier

NCL	Neighbour Cell List

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NES	Network Energy Savings

NGAP	NG Application Protocol

NGSO	Non-Geosynchronous Orbit

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

OCC	Orthogonal Cover Code

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDB	Packet Delay Budget

PDC	Propagation Delay Compensation

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PER	Packet Error Rate

PH	Paging Hyperframe

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PQI	PC5 5QI

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PRS	Positioning Reference Signal

PS-RNTI	Power Saving RNTI

PSDB	PDU Set Delay Budget

PSER	PDU Set Error Rate

PSI	PDU Set Importance

PSIHI	PDU Set Integrated Handling Information

PSS	Primary Synchronisation Signal

PTM	Point to Multipoint

PTP	Point to Point

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QMC	QoE Measurement Collection

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RLM	Radio Link Monitoring

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

RVQoE	RAN visible QoE

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SDT	Small Data Transmission

SD-RSRP	Sidelink Discovery RSRP

SFI-RNTI	Slot Format Indication RNTI

SHR	Successful Handover Report

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SL-PRS	Sidelink Positioning Reference Signal

SL-RSRP	Sidelink RSRP

SMC	Security Mode Command

SMF	Session Management Function

SMTC	SS/PBCH block Measurement Timing Configuration

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SpCell	Special Cell

SPR	Successful PSCell Addition/Change Report

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRAP	Sidelink Relay Adaptation Protocol

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SSSG	Search Space Set Group

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TB	Transport Block

TCE	Trace Collection Entity

TNL	Transport Network Layer

TPC	Transmit Power Control

TRP	Transmit/Receive Point

TRS	Tracking Reference Signal

TSS	Timing Synchronization Status

U2N	UE-to-Network

U2U	UE-to-UE

UAV	Uncrewed Aerial Vehicle

UCI	Uplink Control Information

UDC	Uplink Data Compression

UDM	Unified Data Management

UE-Slice-MBR	UE Slice Maximum Bit Rate

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

VR	Virtual Reality

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

XR	eXtended Reality

Next Modified Subclause

### 7.3.1	Overview

System Information (SI) consists of a MIB and a number of SIBs, which are divided into Minimum SI and Other SI:

-	Minimum SI comprises basic information required for initial access and information for acquiring any other SI. Minimum SI consists of:

-	MIB contains cell barred status information and essential physical layer information of the cell required to receive further system information, e.g. CORESET#0 configuration. MIB is periodically broadcast on BCH.

-	SIB1 defines the scheduling of other system information blocks and contains information required for initial access. SIB1 is also referred to as Remaining Minimum SI (RMSI) and is periodically broadcast on DL-SCH or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED.

-	Other SI encompasses all SIBs not broadcast in the Minimum SI. Those SIBs can either be periodically broadcast on DL-SCH, broadcast on-demand on DL-SCH (i.e. upon request from UEs in RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED), or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED (i.e., upon request, if configured by the network, from UEs in RRC_CONNECTED or when the UE has an active BWP with no common search space configured or when the UE configured with inter cell beam management is receiving DL-SCH from a TRP with PCI different from serving cell's PCI). Other SI consists of:

-	SIB2 contains cell re-selection information, mainly related to the serving cell;

-	SIB3 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SIB4 contains information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters), which can also be used for NR idle/inactive measurements;

-	SIB5 contains information about E-UTRA frequencies and E-UTRA neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SIB6 contains an ETWS primary notification;

-	SIB7 contains an ETWS secondary notification;

-	SIB8 contains a CMAS warning notification;

-	SIB9 contains information related to GPS time and Coordinated Universal Time (UTC);

-	SIB10 contains the Human-Readable Network Names (HRNN) of the NPNs listed in SIB1;

-	SIB11 contains information related to idle/inactive measurements;

-	SIB15 contains information related to disaster roaming;

-	SIB16 contains slice-based cell reselection information;

-	SIB17 and SIB17bis contain information related to TRS configuration for UEs in RRC_IDLE/RRC_INACTIVE;

-	SIBpos contains positioning assistance data as defined in TS 37.355 [43] and TS 38.331 [12];

-	SIB18 contains information related to the Group IDs for Network selection (GINs) associated with SNPNs listed in SIB1.

-	SIB19 in TN contains NTN-specific parameters for NTN neighbour cells as defined in TS 38.331 [12].

For sidelink, Other SI also includes:

-	SIB12 contains information related to NR sidelink communication, ranging and sidelink positioning;

-	SIB13 contains information related to SystemInformationBlockType21 for V2X sidelink communication as specified in TS 36.331 clause 5.2.2.28 [29];

-	SIB14 contains information related to SystemInformationBlockType26 for V2X sidelink communication as specified in TS 36.331 clause 5.2.2.33 [29];

-	SIB23 contains information related to ranging and sidelink positioning.

For non-terrestrial network, Other SI also includes:

-	SIB19 contains NTN-specific parameters for serving cell and optionally NTN-specific parameters for neighbour cells as defined in TS 38.331 [12].

-	SIB25 contains TN coverage information as defined in TS 38.331 [12].

For MBS broadcast, Other SI also includes:

-	SIB20 contains MCCH configuration;

-	SIB21 contains information related to service continuity for MBS broadcast reception.

-	SIBXX contains the description of the intended service area of a broadcast service in an NTN cell.

For MBS multicast reception in RRC_INACTIVE state, Other SI also includes:

-	SIB24 contains the information required to acquire the multicast MCCH/MTCH configuration as defined in TS 38.331 [12].

For ATG network, Other SI also includes:

-	SIB22 contains ATG-specific parameters for serving cell and optionally ATG-specific parameters for neighbour cells as defined in TS 38.331 [12].

Figure 7.3.1-1 below summarises System Information provisioning.

Figure 7.3.1-1: System Information Provisioning

For a cell/frequency that is considered for camping by the UE, the UE is not required to acquire the contents of the minimum SI of that cell/frequency from another cell/frequency layer. This does not preclude the case that the UE applies stored SI from previously visited cell(s).

If the UE cannot determine the full contents of the minimum SI of a cell by receiving from that cell, the UE shall consider that cell as barred.

In case of BA, the UE only acquires SI on the active BWP.

If the UE is configured with inter cell beam management:

-	the UE is not required to acquire the SI from the serving cell while it is receiving DL-SCH from a TRP with PCI different from serving cell's PCI.

Next Modified Subclause

### 16.14.9	Support for NR NTN coverage enhancements

To improve NR uplink coverage in NTN, the following enhancements are supported:

-	PUCCH repetition for Msg4 HARQ-ACK configured in system information or dynamically in DCI for Msg4 when multiple repetition factors are configured in the system information:

-	UEs reports the capability of PUCCH repetition for Msg4 HARQ-ACK in Msg3 PUSCH;

-	When Msg4 HARQ-ACK is repeated, PUCCH repetition is applied for all PUCCH transmission before dedicated PUCCH resource is provided.

-	Improved channel estimation by NTN-specific PUSCH DMRS bundling enhancement that enables DMRS bundling in presence of timing drift, where the UE maintains phase continuity by considering effects of transmission delay variation between the UE and the uplink time synchronization reference point.

To improve NR downlink coverage in NTN, the following enhancements are supported for NTN operating in FR1-NTN bands:

- Repetition of PDCCH CSS.

- Repetitions of PDSCH carrying Msg4.

- Repetitions within 20 ms duration of PDSCH carrying SIB1.

To extend the NR downlink coverage in NTN, for FR1-NTN and FR2-NTN: extended periodicity of 160 ms (apart from the existing 20ms value) for the half frames with SS/PBCH blocks is assumed by UE during initial cell selection.

Next Modified Subclause

### 16.14.X	Support for NR NTN capacity enhancements

To improve uplink capacity in NTN, the following enhancements are supported for PUSCH transmission [56]:

- Inter-slot OCC with OCC length 2 to multiplex up to two UEs

- Inter-slot OCC with OCC length 4 to multiplex up to four UEs

Next Modified Subclause

### 16.14.Y Support of ISA for MBS Broadcast in NTN

An ISA can be signaled for location-dependent services. The ISA is a geographical information that can cover an area larger than one NTN cell (i.e. serving and neighbour cells) or portions thereof. If ISAs are defined for an MBS broadcast service, they are broadcasted as a list in SIBXX and also indicated as part of the MCCH with the ISA identifier.

The NTN-capable UE supporting the MBS broadcast may initiate the broadcast MRB establishment procedure when it is inside the ISA. The UE may initiate the broadcast MRB release procedure when UE leaves the ISA (see clause 5.9.3 of TS 38.331 [12]).

NOTE 1: When using ISA(s) for MBS broadcast service reception or MBS service continuity, it is up to UE implementation how to determine if it is in the ISA(s) of MBS broadcast service or not.

Next Modified Subclause

### 16.14.Z Support of Half-Duplex UE in NR NTN

To support Reduced Capability (RedCap) UEs and enhanced RedCap (eRedCap) UEs in NR NTN operating in FR1-NTN bands, enhancements to address issues caused by Timing Advance (TA) mismatch for Half-Duplex UE in paired spectrum are specified. This mismatch occurs between the actual TA used by the UE and the assumed TA for the UE at the gNB. These enhancements are specifically targeted at mitigating the issues in half-duplex collision cases at the UE side, including scenarios where semi-statically configured downlink reception collides with semi-statically configured uplink transmission, and where dynamically scheduled downlink reception collides with dynamically scheduled uplink transmission (see clause 17.2 of TS 38.213 [38]).

End of Changes

The following appendices shall be removed from final CR

# A	Appendix: RAN2 agreements for WI NR-NTN_Ph3-Core

## Downlink Coverage enhancements

RAN2#125bis

1.	With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work.

2.	We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input

RAN2#126

1.	Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements.

2.	RAN2 assumes that both EFC and EMC are supported

RAN2#127

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

RAN2#127bis

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

RAN2#128

- If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

- As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

- RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

- For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states.

RAN2#129

1.	RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer

2.	RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame

3.	RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell)

4.	The number of SMTC/gaps a UE needs to consider at any time will not be increased further

RAN2#129bis

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

RAN2#130

1.	the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue)

2.	To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

3.	We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

4.	No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)

Working Assumption:

1.	We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options:

Alt 1: UE provide the closest N reference locations/neighbor cells to network.

Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

## Uplink Capacity Throughput enhancement

RAN2#129bis

1.	There is no need to introduce dedicated access control mechanism for OCC capable UEs

## Support of Broadcast service

RAN2#125bis

1.	For MBS broadcast service we don’t restrict the work to any satellite constellation type

2.	We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

3.	We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

4.	The intended service area can cover the area of more than one NTN cells (or portions thereof)

5.	Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

6.	At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

RAN2#126

1.	For MBS broadcast service, both EFC and EMC are supported.

2.	RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

3.	MBS broadcast intended service area is provided via system information

4.	For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

5.	When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

RAN2#127

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

RAN2#127bis

1.	For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

RAN2#128

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

RAN2#129

1.	In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list

2.	It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH)

3.	Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

RAN2#129bis

1.	RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

2.	If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

3.	If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

4.	The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

RAN2#130

1.	Do not introduce service area information specific for FSAI in SIB21.

2.	Do not introduce service area per neighbour cell in MCCH

3.	RAN2 assumes that the ISA can be valid for all the frequencies providing the MBS session

4.	The new SIBxx containing MBS ISA(s) can be cell or area specific, which is up to NW implementation (No specs change).

5.	Capture in a Note in Stage 2 that when using ISA(s) for MBS broadcast service reception or MBS service continuity, it is up to UE implementation how to determine if it is in the ISA(s) of MBS broadcast service or not.

## Support of regenerative payload

RAN2#126

1.	Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

2.	Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

3.	RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

4.	RAN2 assumes that network verified UE positioning is supported for regenerative architecture

5.	RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

RAN2#127

1.	RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario.

2.	Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

3.	RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

4.	We do not optimize UE power saving for SDT procedure in regenerative payload architecture

RAN2#129bis

1.   Specific configurations of common TA and Kmac in regenerative architecture are not captured in the specs.