3GPP TSG-RAN WG2 Meeting #131	R2-2506189

Bengaluru, India, 25th – 29th August 2025

Source:	NTT DOCOMO, INC.

Title:	On remaining issue on RLC enhancements

Document for:	Discussion

Agenda Item:	8.7.5

- Introduction

This contribution is to provide our views on remaining issues on RLC enhancements.

- Discussion

  - Configuration of stopReTxDiscardSDU and t-RxDiscard

The following open issue hasn’t resolved in RAN2#130 meeting [1]

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RLC-10 | Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. - As raised by ZTE below | Issue Type: Not essential not important How to address it: based on companies’ contribution |

For DL, If the network stops retransmission of the SDU which is subject to discard, t-RxDiscard should be configured to the UE to prevent window stalling. When the network stops retransmission of the SDU, the pending SDU will remain in Tx window, and this causes stopping Tx window. To address this, t-RxDiscard should be configured to the UE and then upon expiry of t-RxDiscard, UEs discard the received SDU and trigger a STATUS report with a positive acknowledgement to the network, which moves Tx window properly. On the other hand, if t-RxDiscard is configured, there is no specific functionality for NW to support since upon expiry of t-RxDiscard, UEs will trigger a STATUS report with a positive acknowledgement and then the network will determine the SDU was successfully transmitted. In conclusion, we do not see any precondition or limitation necessary to configure t-RxDiscard.

For UL, if stopReTxDiscardedSDU is configured to the UE, NW should support t-RxDiscard functionality i.e. NW should discard the SDU and trigger a STATUS report with a positive acknowledgement with proper timer setting, in order to prevent window stalling. In other words, if UEs stop retransmission of the SDU which is subject to discard, the pending SDU will remain within Tx window, and this causes stop Tx window. To address this, NW should support t-RxDiscard functionality to move Tx window properly. On the other hand, if NW supports t-RxDiscard functionality, there is no specific functionality to configure since when NW discards the SDU and triggers STATUS report with a positive acknowledgement, UEs will determine the SDU was successfully transmitted. In conclusion, it is required for the network to perform discarding of outdated SDUs and transmitting a STATUS report with corresponding ACKs in order to avoid window stallings.

Based on the above considerations, the following NOTE should be added in the field description of stopReTxDiscardedSDU.

stopReTxDiscardedSDU Indicates whether the Tx side of the RLC entity should stop transmission and retransmission of the RLC SDUs or its segments when discard indication of the SDUs is received from the PDCP layer as specified in TS 38.323 [5] NOTE: If this field is present, the network should discard outdated SDUs and trigger a STATUS report with positive acknowledgements accordingly.

Proposal 1.	(RLC-10) For UL, if stopReTxDiscardedSDU is configured, the network should discard outdated SDUs and trigger a STATUS report with positive acknowledgements accordingly. This should be added as a NOTE in the field description of stopReTxDiscardedSDU.

  - Stopping of transmissions and polling

The following open issue hasn’t completely resolved although there were some agreements about this in RAN2#130 meeting [1]

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RLC-11 | FFS how to handle the case “if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with” - As raised by Nokia and Apple below | Issue Type: Not essential but important? How to address it: based on companies’ contribution |

- Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

The working assumption above should be confirmed because the assumption in the second agreement is correct. Regarding the case “if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, a proper NW implementation can avoid this case. For instance, the network can configure t-RxDiscard to the UE and can perform discarding obsolete SDUs as follows to trigger a STATUS report with positive acknowledgement in proper timing.

- DL: t-RxDiscard should be configured of the nearest value not exceeding the PDCP discard timer of the functionality to stop retransmission of the SDU which is subject to discard of NW. - UL: The network can discard obsolete SDUs and send positive acknowledgements to the UE in a proper timing since the network receives DSR and is aware of UE’s PDCP discard timer.

When Rx triggers a STATUS report with positive acknowledgement, Tx will determine the SDU was transmitted successfully and move Tx window. Therefore, the case concerned in RLC-11 won’t happen.

Proposal 2.	(RLC-11) triggering STATUS report by t-RxDiscard in proper timing can resolve the RLC-11 case.

- Summary and proposal

Proposal 1.	(RLC-10) For UL, if stopReTxDiscardedSDU is configured, the network should discard outdated SDUs and trigger a STATUS report with positive acknowledgements accordingly. This should be added as a NOTE in the field description of stopReTxDiscardedSDU.

Proposal 2.	(RLC-11) triggering STATUS report by t-RxDiscard in proper timing can resolve the RLC-11 case.

- References

[1]　 R2-2503767, “Discussion summary and list of RLC open issue for R19 XR,” vivo, RAN2#130.