3GPP TSG-RAN WG2 #132	Tdoc R2-2508311

Dallas, Texas, USA, Nov. 17th-21st

Agenda Item:	10.3.2.1

Source:	Ericsson

Title:	RRC Inactive state in 6G

Document for:	Discussion

# 1	Introduction

In this document we present our view on the RRC state model for the 6G radio interface (6GR), focusing particularly on the RRC Inactive state. We describe some of the problems experienced with RRC Inactive in NR and outline some possible solutions. We also discuss some high-level principles for the design of small data transmission in RRC Inactive.

# 2	Discussion

## 2.1	Inactive state in 6GR

At the RAN2#131bis meeting the following agreements were made regarding the RRC state model for 6GR:

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

Based on the discussions in RAN2#131bis, it seems clear that there will be an Idle and Connected state in 6GR with similar functionality as the Idle and Connected state in NR. Most companies also seem to like the functionality provided by the Inactive state in NR (such as fast transition to Connected state while being UE power efficient) and would like to see some form of Inactive like state also in 6G, although there are different views on how to exactly model the state. Some companies propose to model it as a separate state as in NR while others propose to model it as a sub-state of RRC Idle or even RRC Connected. In our view the exact modelling is not important at this point in time; what is important is rather the functionality that the state provides.

To simplify the discussion in the remainder of this document, we will refer to the Inactive like state in 6GR as RRC Inactive without making any assumptions on the exact modelling of the state.

In our view the key characteristics of RRC Inactive state is the following:

- UE AS context stored in UE and RAN enabling fast transition to Connected state with reduced signalling

- An identifier (such as I-RNTI or similar) used to identify and locate the stored UE context in RAN)

- Paging (either RAN or CN based paging, see section 2.3) used to reach the UE at incoming DL data/signalling

- UE based mobility based on cell selection/re-selection similar as in Idle state

- System information acquisition and other procedures (e.g. PLMN selection) also similar as in Idle state

In the following sub-sections, we highlight some of the problems with RRC Inactive in NR that should be addressed in 6GR.

## 2.2	Extending size of I-RNTI

In NR, I-RNTI is used to identify the last serving gNB and the UE context therein. As the I-RNTI is sent in Msg3 in the random-access procedure its size is limited by the Msg3 size. Two different I-RNTI formats were therefore defined in NR: a short I-RNTI (24 bits) intended for 56 bit Msg3 and full I-RNTI (40bits) intended for 72 bit Msg3. However, even with the full I-RNTI, the gNB part of the I-RNTI is still only 16 or 20 bits (see Figure 1 below) which is shorter than the gNB ID (22-32bits) contained in the global cell identity broadcasted in SIB1.

In NR, the gNB ID part of the I-RNTI is too short to uniquely identify the gNB holding the UE context

One solution is to truncate the gNB ID so that it fits into the I-RNTI, however, this increases the risk of identity conflicts and is difficult to do in practice since operators use their own gNB ID formats. Another solution is to assign a new additional shorter identifier to each gNB, however, this causes additional configuration complexity for the operator and also does not eliminate the risk of collisions.

Figure 1 NR Full I-RNTI format (40 bits). See Annex C of 38.300.

For 6GR we would therefore prefer to extend the size of the I-RNTI so that it fits the full gNB ID from the global cell identity. An example of what the new I-RNTI format could look like is shown in Figure 2.

Note that, similar to the legacy Full I-RNTI in NR, the PLMN ID can be included in the 6GR I-RNTI. This is needed in case of RAN sharing and allows each PLMN to assign their gNB IDs independently of the other PLMNs. Assuming that the RAN sharing is supported with at most 12 PLMNs in 6GR (same as in NR), 4 bits is sufficient for the PLMN ID.

Figure 2 Example 6GR I-RNTI format (42-60 bits).

To simplify the gNB identity planning and avoid identity collisions, the size of the I-RNTI should be extended to allow the PLMN ID and full gNB ID to be included.

To support the new I-RNTI format the size of Msg3 also needs to be increased. The size of Msg3 is given by the UL grant in RAR, and hence in principle any Msg3 size can be supported. However, UL coverage limits the possible sizes that can be used. Using physical layer enhancements, such as Msg3 repetition introduced as part of the Rel-17 coverage enhancement, it is possible that Msg3 size can be increased in 6GR without sacrificing cell coverage. RAN2 should therefore inform RAN1 early on of the Msg3 size requirements so that RAN1 can evaluate if it’s feasible to support and possibly adjust the Msg3 design.

Inform RAN1 early on of the required/wanted Msg3 size in 6GR so RAN1 can evaluate what is feasible to support and possibly adjust the Msg3 design.

## 2.4	UE context retrieval without Xn connectivity

In NR, the source gNB needs to have Xn connectivity to all potential target gNBs that the Inactive UE may resume in, in order for the target gNB to be able to retrieve the UE context. This means that a source gNB needs to have Xn connectivity to all other gNBs in the RAN Notification Area (RNA) plus the gNBs immediately neighboring the RNA (the latter is needed for UE context retrieval at RNA update). Setting up and maintaining all these Xn connections is complex and consumes resources in the gNB. Also, re-establishing all Xn connections after a gNB restart can take quite some time.

In NR, Xn connectivity is required to all gNBs in the RNA and to all gNBs neighbouring the RNA which is complex to configure and resource consuming for the gNB.

For 6GR it would therefore be preferred if the Xn connectivity requirement can be relaxed by e.g., allowing the UE context retrieval request/response to be relayed via the CN when Xn is not available. This would be similar to how NG-based handover is used as fallback when Xn-handover is not available.

Study how the target gNB can retrieve the UE context without requiring Xn connectivity to the source gNB.

## 2.3	Paging of Inactive UEs

In NR, paging of Inactive UEs are managed by the RAN (RAN paging) while paging of Idle UEs are managed by the CN (CN paging). RAN and CN paging differ in a few key aspects:

- Location tracking: For CN paging the CN keeps track of the UE’s location on Tracking Area (TA) level through the CN registration update procedure, while for RAN paging the RAN keeps track of the UE’s location on RNA level through the RNA Update procedure.

- Paging distribution: In CN paging the serving AMF pages the UE in the UE’s registration area using the Ng paging request procedure, while in RAN paging the source gNB pages the UE in the RNA using the Xn paging request request procedure.

- Paging identifier: In CN paging the UE is addressed using its S-TMSI allocated by the serving AMF, while in in RAN paging the UE is addressed using its I-RNTI allocated by the source gNB.

In NR, separate paging mechanisms are used for Idle and Inactive UEs (CN paging and RAN paging)

Having separate paging mechanisms for Idle and Inactive UEs increases complexity in both UE and network, and for 6GR we should therefore investigate if we can simplify and use CN paging for both Idle and Inactive UEs. Using only CN paging would simplify for the RAN as RNAs would no longer need be configured and the RAN would no longer be responsible for the paging distribution. It would also simplify for the UE as it no longer has to perform RNA updates, only TA updates. Having a single paging mechanism would also be in line with the SA 6G SI objective of avoiding “duplication of functionality in RAN and CN” (see SP-250806). However, before going down this route we need to carefully investigate if removing RAN paging has any negative impact on performance, e.g. increased DL latency or increased RAN-CN signalling. Also, SA2 and RAN3 need to be involved at some point as this impacts the RAN-CN functionality distribution.

Study if CN based paging can be used for both Inactive and Idle UEs to reduce UE and network complexity and make network maintenance simpler for the operator.

Another option is to keep RAN paging for Inactive UEs but try to make it easier to configure and deploy. One area for simplification is the RNA configuration – in NR, the RNA can be configured in three different ways: as a cell list, as a list of RAN areas, or as a list of tracking areas.  We should revisit if all these configuration options are really needed. Another problem with RAN paging is that if the RNA spans more than one gNB, all gNBs within the RNA need to have Xn connectivity to all other gNBs in the RNA. As mentioned in previous section on UE context retrieval, setting up and maintaining all these Xn connections is complex and resource consuming for the gNB, and for 6GR it would therefore be preferred if RAN paging can be supported without requiring full Xn connectivity (e.g. by allowing the the paging request to be relayed via the CN when Xn is not available).

If RAN paging is still used in 6G, study how the target gNB can page the UE in the RNA without having Xn connectivity to all other gNBs in the RNA.

## 2.4	Transferring of data RRC Inactive

There seems to be an interest for some functionality similar to the Small Data Transmissions (SDT) feature in NR also for 6GR. Some companies argue for the SDT feature to become a baseline for the work in 6GR. We must however emphasize that this is currently a study item and using specific functionalities “as is” could lead to more confusion than clarity, especially since SDT is a tailored solution with a high complexity, for the 5G state model, and the 6G state model is only being studied at this point.

For small data transmission in 6GR, it would be better to agree on study requirements rather than agreeing on reusing functionality as baseline.

To advance the discussion we do however propose to consider functionalities for RRC Inactive in 6GR to be studied and the latest agreement from RAN2#131bis supports that.

Some principles that should be included in the study of data transmissions in RRC Inactive in 6GR:

- The functionality should target aspects based on a general use case, not only IoT.

- The functionality should be targeted to efficient transmission in RRC Inactive for 6GR, tied to fast transition to RRC Connected for 6GR if beneficial.

- The functionality should be targeted for small overhead, thus ensuring that data transfers in are resource effective.

- The functionality should avoid complicated early indication (RACH partitioning).

- Network is in control of the conditions for when the UE is allowed to trigger sending such data from RRC Inactive

- When such data transmission is ongoing, the network can decide to bring the UE to RRC Connected to continue normal data transmission.

Based on the above principles we propose the following:

Baseline principles to study for the data transmissions related to the 6GR RRC Inactive state should include:

  - The target use case should apply to all transmissions, not only IoT.

  - Target smaller data amounts and few transport blocks with small overhead.

  - Avoid complicated RACH partitioning solutions with cross dependencies to other features.

  - Target network control of conditions for using data transmissions in RRC Inactive.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In NR, the gNB ID part of the I-RNTI is too short to uniquely identify the gNB holding the UE context

Observation 2	In NR, Xn connectivity is required to all gNBs in the RNA and to all gNBs neighbouring the RNA which is complex to configure and resource consuming for the gNB.

Observation 3	In NR, separate paging mechanisms are used for Idle and Inactive UEs (CN paging and RAN paging)

Observation 4	For small data transmission in 6GR, it would be better to agree on study requirements rather than agreeing on reusing functionality as baseline.

Based on the discussion in the previous sections we propose the following:

Proposal 1	To simplify the gNB identity planning and avoid identity collisions, the size of the I-RNTI should be extended to allow the PLMN ID and full gNB ID to be included.

Proposal 2	Inform RAN1 early on of the required/wanted Msg3 size in 6GR so RAN1 can evaluate what is feasible to support and possibly adjust the Msg3 design.

Proposal 3	Study how the target gNB can retrieve the UE context without requiring Xn connectivity to the source gNB.

Proposal 4	Study if CN based paging can be used for both Inactive and Idle UEs to reduce UE and network complexity and make network maintenance simpler for the operator.

Proposal 5	If RAN paging is still used in 6G, study how the target gNB can page the UE in the RNA without having Xn connectivity to all other gNBs in the RNA.

Proposal 6	Baseline principles to study for the data transmissions related to the 6GR RRC Inactive state should include:

a.	The target use case should apply to all transmissions, not only IoT.

b.	Target smaller data amounts and few transport blocks with small overhead.

c.	Avoid complicated RACH partitioning solutions with cross dependencies to other features.

d.	Target network control of conditions for using data transmissions in RRC Inactive.

# References

SP-250806, “Study on Architecture for 6G System”