3GPP TSG-RAN WG2 Meeting #132	R2-2508475

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.12.2

Source:	Nokia

Title:	MIMO - MAC Open issues

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Following RAN2#131, the NR_MIMO_Ph5 work item was declared complete in RAN2, and all critical points are considered to be addressed. However, in our view some clarifications are still needed on the MAC and  in this contribution we share our views on some of the MAC open issues listed in  [Post131bis][215][MIMO_Ph5] CR for TS 38.321 (Samsung).

# 2	DRX Active time in Mode-A UE IBR

As per 38.321, DRX active is defined as:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: …… - a PDCCH scheduling a UE-initiated CSI reporting has not been received after transmitting UE Initiated Report Indication on PUCCH for mode-A UE-initiated CSI reporting (as specified in TS 38.214 [7]).

During the open issue discussion, it was observed that if the PDCCH for a CSI report is never received—either because the network does not schedule the PDCCH after receiving the UEIRI, or because the network fails to receive the UEIRI on the PUCCH—the UE remains in DRX active time indefinitely, as the condition for remaining active is continuously met. This results in unnecessary power consumption.

According to RAN1, the UE may transmit multiple UEIRIs (either continuously or intermittently), or it may transmit only a single UEIRI. The specification does not mandate a specific transmission behavior, leaving room for implementation-dependent variations. Therefore, it cannot be assumed that all UEs will repeatedly transmit UEIRIs for a triggered UEI-based CSI report.

To account for these differing UE implementations and to mitigate unnecessary power consumption, we support Proposal 2 from the offline discussion.

Proposal 1: For DRX operation, after transmitting a UEIRI on the PUCCH, the UE shall remain in active time until it either receives a PDCCH scheduling the Mode-A UEI-based CSI report on the PUSCH, or until the next PUCCH occasion for UEIRI transmission - whichever occurs first.

# 3 UEI PUCCH resource release

For a UEI report configuration (CSI-ReportConfig) linked to SCell, the PUCCH resource used to transmit the UEI report can be configured either on the same SCell or on a different cell designated as the PUCCH cell. Additionally, in the case of Mode-B Type-1 CG associated with a UEI report configuration on an SCell, the CG must reside on that same SCell.

This setup supports cross- CC UEI CSI reporting, meaning the PUCCH used for UEI report indication and the PUSCH used for the actual report may be located on different cells. For Mode-B UEI reporting, the PUCCH and Type-1 CG PUSCH can be configured on separate cells, and thus may belong to different TAGs. If the TAT linked to the sTAG for the Type-1 CG PUSCH expires while the TAT for the PUCCH is still active, the expected UE behavior becomes ambiguous.

Following offline discussions, companies have expressed strong concerns about releasing the PUCCH resource due to its potential impact on network complexity and UCI reliability. To mitigate risks such as unnecessary UCI collisions and increased power consumption, we are open to supporting Option 2, even though our initial preference was Option 1 as outlined in the offline document.

Proposal 2: For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running,  UE does not release the PUCCH, UE does not transmit UEIRI on the PUCCH if UEI CSI report is triggered.

# 4	Conclusion

Proposal 1: For DRX operation, after transmitting a UEIRI on the PUCCH, the UE shall remain in active time until it either receives a PDCCH scheduling the Mode-A UEI-based CSI report on the PUSCH, or until the next PUCCH occasion for UEIRI transmission—whichever occurs first.

Proposal 2: For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running,  UE does not release the PUCCH, UE does not transmit UEIRI on the PUCCH if UEI CSI report is triggered.