3GPP TSG-RAN WG2 Meeting #132	           R2-2508497

Dallas, USA, November 17th – 21st, 2025

Agenda item:	9.3.3

Source:	China Telecom

Title:	LCM for measurement event prediction

Document for:	Discussion

- Introduction

In this paper, we would like to discuss LCM for UE-sided model for RRM measurement event prediction based on AI/ML. As concluded during RAN2#131 meeting, there is no consensus on the feasibility of performance monitoring of the direct event prediction and this would need to be resolved in WI phase before proceeding with specifying direct event prediction. Thus we mainly discuss indirect event prediction aspects.

- Discussion

During study phase, RAN2 had the following LCM related agreements for UE-sided model for RRM measurement event prediction:

RAN2#130 Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology). RAN2#131 Aim for a single framework for event prediction and performance monitoring. From RAN2 point of view indirect event prediction, RSRP differences can be used as the performance metric for monitoring. Capture in the TR that there is no consensus on the feasibility of performance monitoring of the direct event prediction and this would need to be resolved in WI phase before proceeding with specifying direct event prediction. For RRM measurement event prediction, the following are included in the inference configuration: - The length of PW for the associated AIML model - Event-related information (event type and event-related parameters) For RRM measurement event prediction, the event prediction report can include: - RRM measurements (For normative phase to define what types of measurements) - Time-related information about the predicted event (e.g., the time/window of occurrence) For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure. This can be achieved without spec impact for measurement reporting. FFS for normative phase if additional information needs to be added.

Regarding indirect measurement event prediction, in our understanding, part of inference configuration can reuse the inference configuration for RRM measurement prediction. We think it is better to discuss event-related information for measurement event prediction first and avoid duplicate discussion for the common part. Before going into the details of signalling design, we want to first understand the general procedure for mobility based on the measurement event prediction. Considering temporal domain case A, a simple example procedure is as the following figure.

Figure example procedure for the measurement event prediction based mobility

According to RAN2#131 agreements, the inference configuration include event-related information and the event prediction report can include time-related information about the predicted event, which is also reflected in the figure above. Before the event prediction reporting, the UE need to determine whether an event is fulfilled with predicted RRM measurement results (and optionally actual measurement results), i.e. whether an event is indirectly predicted. Regarding temporal domain case A, there are two possible trigger conditions for event prediction reporting. One possible trigger condition is that the predicted RRM measurement results (and optionally actual measurement results) fulfill entering condition for the event. Another possible trigger condition is that the predicted RRM measurement results (and optionally actual measurement results) fulfill condition for the event, i.e. all measurements including predicted measurements and optionally actual measurements taken during TTT. Regarding temporal domain case B, it seems straightforward to reuse the event condition as event prediction reporting trigger. Regarding frequency domain case, similar to temporal domain case A, the two possible trigger conditions are also applicable.

Proposal 1: For indirect event prediction, RAN2 down select the trigger condition for event prediction report among:

- predicted RRM measurement result and/or actual measurement result fulfil condition for the event

- predicted RRM measurement result or actual measurement result fulfil entering condition for the event

As we can see, the gNB can perform handover preparation in advance thanks to the event prediction report. In the figure above, gNB send the handover command to the UE when the gNB receives the MeasurementReport from UE, which is triggered by legacy measurement event based on actual measurement result. In our understanding, if the MeasurementReport is not triggered based on actual measurement result, the gNB can release corresponding handover preparation related configuration and resources based on time-related information about the predicted event in the event prediction report, which is up to implementation. However, if the gNB can send the RRCReconfiguration (handover command) when the event prediction report is received, the handover latency can be further reduced. The UE will not apply the RRCReconfiguration right now. If the measurement event is not actually fulfilled in the future based on actual measurement results, the UE can release the stored RRCReconfiguration.

Proposal 2: RAN2 confirm the timing gNB sending handover command, i.e. gNB send handover command to UE when event prediction report is received OR gNB send handover command to UE when MeasurementReport is received.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: For indirect event prediction, RAN2 down select the trigger condition for event prediction report among:

- predicted RRM measurement result and/or actual measurement result fulfil condition for the event

- predicted RRM measurement result or actual measurement result fulfil entering condition for the event

Proposal 2: RAN2 confirm the timing gNB sending handover command, i.e. gNB send handover command to UE when event prediction report is received OR gNB send handover command to UE when MeasurementReport is received.