3GPP TSG-RAN WG2 Meeting #132	R2-2508519

Dallas, USA, 17th –21st November, 2025

Agenda Item:	10.3.3.2

Source:	Huawei, HiSilicon

Title:	Discussion on network and UE energy efficiency

Document for: Discussion and Decision

# 1	Introduction

The 6G network and UE energy efficiency was discussed in RAN2#131bis meeting with the following agreements [1]:

Design goal - Strive to develop solutions that consider network energy efficiency and UE power saving For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution, we discuss the energy efficiency techniques considering both network and UE sides from RAN2 point of view.

# 2	Discussion

## 2.1	UE energy saving

- DL WUS mechanism

For energy saving techniques in time domain, one basic idea is to reduce the time of PDCCH monitoring, e.g., the PDCCH monitoring for paging in IDLE, or PDCCH monitoring for data scheduling in CONNECTED. Following this logic, DL wake-up signals (WUSs) were introduced in 5G. Specifically, PEI and LP-WUS were introduced in IDLE to reduce the unnecessary PO monitoring and associated SSB measurements. DCP and LP-WUS were introduced in CONNECTED to avoid PDCCH monitoring when there is no data transmission for the UE. However, since the different wake-up schemes were introduced in different releases, there were duplicated functionalities within each of them which made it difficult to be commercialized. Building on lessons from 5G, in 6G Day-1, we should target a single wake-up mechanism for IDLE and CONNECTED modes. The commonality of the above mentioned 5G mechanism is using a DL signal to wake up a UE for PDCCH monitoring. Therefore, a DL WUS mechanism can be considered a starting point for 6G.

The DL WUS mechanism highly relies on the RAN1 study, e.g., the DL WUS signal design. In RAN1#122bis meeting, the following agreements related to DL WUS were reached. RAN2 discussion can focus on higher layer procedure design for DL WUS.

Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities

Observation 1: A wake-up mechanism is beneficial for UE energy saving due to PDCCH monitoring reduction in time domain. RAN1 has agreed to study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement.

In IDLE mode, a UE is required to monitor POs for paging message reception in every I-DRX cycle. In addition, before the UE performs paging monitoring (and measurements), 2 or 3 SSBs are needed for finer synchronization, during which the UE can only stay in light-sleep mode to avoid synchronization degradation. It brings higher power consumption due to the potential unnecessary PO monitoring and the shorter deep-sleep mode for SSB measurement. In this context, DL WUS can be studied to reduce or even avoid (i.e., via per UE wake-up) the unnecessary wake-up due to the paging for other UEs, to save UE energy in IDLE.

Proposal 1: RAN2 to study DL WUS for IDLE mode, including DL WUS for paging.

In CONNECTED mode for 5G, cycle-based C-DRX was designed as a basic UE power saving feature. However, considering dynamic and random data arrival for most services, the cycle-based C-DRX with fixed OnDuration cannot match the dynamic traffic. In this case, the UE needs to monitor PDCCH in some OnDuration periods even if there is no data transmission. Hence, DL WUS (i.e., DCP/LP-WUS) was introduced to avoid some “blank” C-DRX Active Time periods. In Rel-19 LP-WUS, a more flexible LP-WUS based C-DRX (i.e., LP-WUS Option 1-2) was introduced. It provides both UE energy saving and low latency for data transmission, and can be baseline for the study of enhanced C-DRX in 6G, i.e., DL WUS-based C-DRX instead of 5G cycle-based C-DRX.

Proposal 2: RAN2 to study DL WUS for CONNECTED mode, including DL WUS-based C-DRX, e.g., a more flexible C-DRX enabled by dynamic DL WUS triggering instead of relying on DRX cycle.

In addition to PDCCH monitoring reduction based on DL WUS, UE power saving related to measurement should also be considered together with DL WUS in 6G. Because if a UE needs to perform RRM measurements or RLM/BFD measurements, it cannot enter a deeper sleep mode even if there is no data transmission. Similar to the RRM measurement relaxation and offloading introduced in 5G, RAN2 should also study the RAN2 impacts on measurement relaxation based on RAN1/4 progress.

- UE assistance information

Overheating and UE power saving assistance information (UAI) are RAN2-led UE energy saving features to save energy in frequency and spatial domains in 5G. A UE can suggest the preferred reduced number of carriers, bandwidth and MIMO layers and then can turn off part of its components to save power effectively. In 6G, it is still an effective method for UE energy saving, and it can be discussed as one candidate use case within dynamic capability update framework as in a companion contribution [2].

Observation 2: Overheating/UE power saving UAI mechanisms are discussed within dynamic capability update framework.

## 2.2	Network energy saving

In 5G, UE C-DRX was defined in Rel-15 but cell DTX/DRX was introduced only in Rel-18. Two separate cell DTX and UE C-DRX configurations and mechanisms were agreed, with some duplicated UE behaviours (e.g., UE is not required to monitor PDCCH scrambled by some RNTIs during the non-active time) and additional restrictions (e.g., the network should ensure that there is at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX active duration). It makes cell DTX and UE C-DRX complicated for both network and UEs. Thus, a simplified design (e.g., to avoid redundant configuration and duplicated behaviours) is desirable considering both network transmission and UE reception behaviours.

Both cell DTX and UE C-DRX correspond to DL transmission, to obtain the energy saving gain for the network coming from longer sleep duration, the cell DTX should be aligned with UE C-DRX. Especially considering a flexible UE C-DRX, if the UE C-DRX Active Time is triggered by DL WUS dynamically, the cell DTX should also be dynamic to match the “Active Time” and “sleep duration”. Thus, a joint design for flexible cell DTX and flexible UE C-DRX in CONNECTED mode needs to be considered.

Observation 3: Separate cell DTX and UE C-DRX mechanisms were defined in different 5G releases with duplicated UE behaviours and additional restrictions between cell DTX and UE C-DRX. To obtain the energy saving gain coming from longer sleep duration, the cell DTX should be aligned with UE C-DRX, especially considering a flexible UE C-DRX.

Proposal 3: RAN2 to study joint and simplified (e.g., to avoid duplication) design for flexible cell DTX and flexible UE C-DRX.

Cell DRX is designed to reduce network energy consumption for reception. However, the semi-static and cell-common configurations cannot adapt to traffic condition variations. To satisfy all UEs’ QoS requirements in mixed traffic scenarios, the parameters are typically set in conservative manner, i.e., “On-duration" is set to be long in order not to impact the QoS experience of users with minimum delay requirement, which degrades the energy saving gain. Therefore, a more flexible cell DRX mechanism should be studied considering network energy saving gain and latency for UL data transmission. It should allow the network to adapt its active/inactive time based on dynamic traffic patterns.

Proposal 4: RAN2 to study cell DRX considering network energy saving gain and latency for UL data transmission.

## 2.3	Joint network/UE energy saving

For energy saving techniques in time domain, increasing the sleep time of both network and UE can maximize the overall system energy saving benefit. In 5G, the DL/UL data transmission opportunities are up to network implementation, e.g., the data can be scheduled as soon as possible if there are available resources, which is good for latency. However, latency may not always be top priority because as long as the transmission delay is within the QoS delay budget, the user experience will not be impacted. Frequent scheduling is not good for power saving since both the network and the UE need wake up multiple times.

Observation 4: As long as the transmission delay is not larger than user’s QoS delay budget, the user experience will not be impacted. Frequent scheduling is not energy efficient since both the network and the UE need wake up multiple times.

Figure 1. Illustration of clustered DL data transmission

Thus, in order to maximize the sleep duration for both network and UE in 6G, clustered UL/DL data transmission in time domain can be considered and studied. Figure 1 illustrates the clustered DL data transmission. In the example shown in the figure, the data transmission of the two UEs is scheduled closely in the time domain, which is enabled by flexible wake-up signalling. From the network perspective, the UEs’ experience is guaranteed while at the same time the network can have a longer sleep and then schedule the DL data transmission in one-shot for multiple UEs. UEs also benefit from the longer sleep time. By utilising clustered UL/DL data transmission, both network and UE can get the energy saving benefit. The clustered UL/DL data transmission can also include clustered control signalling.

Proposal 5: RAN2 to study clustered UL/DL data transmission for joint network and UE energy saving.

# 3	Conclusion

This contribution includes the following observations and proposals:

UE energy saving

Observation 1: A wake-up mechanism is beneficial for UE energy saving due to PDCCH monitoring reduction in time domain. RAN1 has agreed to study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement.

Proposal 1: RAN2 to study DL WUS for IDLE mode, including DL WUS for paging.

Proposal 2: RAN2 to study DL WUS for CONNECTED mode, including DL WUS-based C-DRX, e.g., a more flexible C-DRX enabled by dynamic DL WUS triggering instead of relying on DRX cycle.

Observation 2: Overheating/UE power saving UAI mechanisms are discussed within dynamic capability update framework.

Network energy saving

Observation 3: Separate cell DTX and UE C-DRX mechanisms were defined in different 5G releases with duplicated UE behaviours and additional restrictions between cell DTX and UE C-DRX. To obtain the energy saving gain coming from longer sleep duration, the cell DTX should be aligned with UE C-DRX, especially considering a flexible C-DRX.

Proposal 3: RAN2 to study joint and simplified (e.g., to avoid duplication) design for flexible cell DTX and flexible UE C-DRX.

Proposal 4: RAN2 to study cell DRX considering network energy saving gain and latency for UL data transmission.

Joint network/UE energy saving

Observation 4: As long as the transmission delay is not larger than user’s QoS delay budget, the user experience will not be impacted. Frequent scheduling is not energy efficient since both the network and the UE need wake up multiple times.

Proposal 5: RAN2 to study clustered UL/DL data transmission for joint network and UE energy saving.

# 4	References

- RAN2#131bis, ChairNotes.

- R2-2508616, Discussion approach for 6G UE capability, Huawei, HiSilicon.