3GPP TSG-RAN WG2 Meeting #132	R2-2508566

Dallas, USA, 17th – 21st November 2025

Agenda item:		10.3.1.1

Source:		Lenovo

Title:		Discussion of User Plane Functionalities

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss the User Plane functionalities and related functional requirements.

# Discussion

### 5G NR Protocol stack UP

The 5G NR protocol stack was initially developed based on the protocol stack designed for LTE, with modifications to accommodate the specific key performance indicators (KPIs) of 5G. This protocol stack has evolved, with new optimizations and features added across various releases to support a wide range of services and their diverse service requirements. A notable issue with this approach is that the continual addition of new functions and optimizations has resulted in the duplication of functionalities across different protocol layers, leading to an overall complex protocol behaviour and inherent inefficiencies. In current Rel-19 XR enhancements WI, optimizations are introduced addressing some of those inefficiencies, e.g. enhancements targeting the inefficient ARQ retransmission protocol behaviour stemming from the independent receiving operations implemented in PDCP and RLC layer as further discussed below.

For 6G, the aim should be therefore to streamline the protocol stack compared to 5G, by removing functionalities which are found to be inefficient/unnecessary or redundant.

### 6G User Plane functionalities

In 5G NR, a large number of different PDU formats are specified for Layer 2 protocols (e.g., MAC PDUs, RLC PDUs), addressing many specific exceptions. The header design philosophy largely aimed at optimizing overhead “down to the last bit”. While this approach minimizes signaling overhead, it has resulted in complex and fragmented header structures in the specifications.

For 6G, such complexity should be avoided. Instead, Layer 2 header formats should be designed with simplicity, extensibility, and implementation efficiency in mind, even if this entails a marginal increase in overhead. This would improve protocol clarity, ease of implementation, and long-term maintainability of the specifications.

Observation 1:  The Layer 2 PDU format/header design philosophy largely aimed at optimizing overhead “down to the last bit”. While this approach minimizes signaling overhead, it has resulted in complex and fragmented format/header structures in the specifications. For 6G, such complexity should be avoided.

Configuration adaptability

In 5G systems, radio and protocol parameters are configured to (semi-)static values based on worst-case network assumptions. Any subsequent adjustment typically requires an RRC Reconfiguration procedure, which is signaling-intensive and incurs a non-negligible delay. This results in sub-optimal performance, particularly in scenarios with highly dynamic conditions.

For 6G, it would be beneficial to introduce mechanisms that allow the UE to autonomously adapt selected parameters within network-defined bounds, thereby reducing reliance on frequent RRC reconfigurations. Such adaptation could leverage AI/ML models executed at the UE, enabling adaptation of parameters intelligently to meet key performance indicator (KPI) targets.

There may be different categories of parameters /configurations as follows:

- Fully Network-Controlled Parameters

  - UE shall strictly follow the configuration provided by the network.

- UE-Adjustable Parameters with Network Notification

  - UE may adjust parameter values within network-defined ranges, with mandatory reporting of the applied value to the network.

- UE-Autonomous Parameters

  - UE may autonomously adapt parameter values, e.g. based on predefined rules or conditions, without the need to inform the network. Examples of those category of parameters could be e.g. RLC timers (t-Reassembly), LCH priority, PBR value

This framework would enable reduced signaling overhead through fewer RRC reconfiguration procedures and a faster adaptation to local and dynamic conditions resulting in an enhanced QoS tailoring to diverse UE applications.

Proposal 1: RAN2 should study solutions which enable UE to adapt NW parameter/configurations, e.g. RLC timer or LCH priority, intelligently within some limited ranges which are under network control.

Application Awareness in Access stratum/L2

The 5G protocol stack has been designed in an information-agnostic manner where neither the content of data packets nor their inter-relationships are considered when allocating radio resources. Packets are transmitted in buffer order without considering application-level semantics or QoE contribution. While Rel-18/19 introduced the concept of an application layer data unit (PDU set) to enable limited application awareness in the RAN, the uplink (UL) remains constrained as the gNB is for example not aware of metadata associated with a PDU. Furthermore, inter-dependencies across PDUs or bearers, or AL-FEC information are currently not exploited for resource allocation at the RAN. Different data units of the same service are treated equally despite unequal contribution to user experience.

For 6G, it would be desirable that application awareness is natively supported in the RAN, to help the network utilise the resources more efficiently and improve the quality of experience (QoE). The 6G protocol stack should be designed to support application-optimized communication, e.g. semantics and context of the data should be considered for resource allocation given that different data packets of a service contribute differently to the user experience. Mechanisms should be introduced for the application layer to convey metadata to the RAN, e.g., packet priority, dependency, relevance, or semantic weight. The metadata should be available to both UE and gNB in the AS for uplink and downlink resource optimization. RAN scheduling decisions should incorporate semantic importance of PDUs.  Retransmission, HARQ, and resource allocation strategies should be adapted based on application relevance. In general, the 6G protocol stack should facilitate better integration and collaboration between applications and the radio access network, enabling more flexible and efficient service delivery, e.g. define standardized interfaces between application and RAN layers to enable seamless metadata exchange by for example labels or headers.

Observation 2: native application-awareness at the Access Stratum in 6G would be beneficial and should be further studied.

Pre-processing in 6G

In 5G NR the functionality of RLC concatenation, which was supported in LTE, has been removed from the RLC layer, mainly to allow for pre-processing in the RLC layer and PDCP layer. Upon arrival of an PDCP SDU the SDU can be processed straight down to the RLC PDU before the reception of an UL grant. This can reduce the processing complexity at the time of MAC PDU generation when the UL grant arrives, e.g. the channel coding of certain CB(s) can already start early. While NR required higher data rates compared to LTE, 6G not only demands significantly greater data rates but also necessitates support for a much wider variety of services, including e.g. immersive communications, AI-native networks, sensing related data. To meet these requirements, we think that pre-processing should be supported also in 6G.

Proposal 2: pre-processing functionality is supported in 6G.

Segmentation /Reordering

In 5G NR, “RLC segmentation” refers to the process where the Radio Link Control (RLC) layer breaks down large data packets into smaller segments for transmission over the air interface, ensuring they fit within the limitations of the lower layer (UL/DL grants), and then reassembling them at the receiving end once received. Essentially segmentation manages the RLC PDU size for efficient transmission by dividing it into smaller, manageable chunks.

Due to the varying channel conditions, large SDUs may not fit into the available grant sizes without segmentation, leading to either transmission failures or inefficient use of radio resources. Segmentation is essential for enabling flexible, reliable, and efficient data transmission over the radio interface. We think that segmentation and correspondingly reassembly is an essential functionality also for 6G.

Without reordering, higher protocol layers could receive PDUs in the wrong sequence, potentially causing application errors or triggering unnecessary retransmissions. In 5G NR the Packet Data Convergence Protocol (PDCP) layer handles this by performing reordering to ensure in-order delivery before passing the data to the next layer, but this reordering can be disabled for specific data radio bearers. Reordering ensures reliable, correctly sequenced data delivery over variable 5G radio channels. In-order delivery thus maintains the end-to-end sequential integrity of data streams—an essential feature for the 6G user plane protocol.

For certain services like latency-sensitive applications out-of-order delivery is particularly advantageous since those applications benefit from processing available data immediately without waiting for the complete sequence. Out-of-order delivery provides flexibility for data transmission under fluctuating radio conditions, minimizing latency. It complements reordering and in-order delivery mechanisms by offering an alternative for services that prioritize timely data availability over strict sequence integrity.

Proposal 3: Segmentation/reassembly functions as well as reordering/in-order delivery functions and out-of-order delivery is supported in 6G.

Sequence number handling in RLC/PDCP

Another issue with the 5G protocol stack design is that at most one complete PDCP SDU or PDCP PDU, e.g. since no concatenation is supported on PDCP layer, can be carried in a RLC PDU. Even though there is a one-to-one mapping between PDCP PDU and RLC PDU, the PDCP layer as well as the RLC layer assigns its own sequence number (SN), which increases the general Layer 2 overhead as well as the processing complexity.

One straightforward solution for this is to have shared SN between PDCP and RLC. In the standalone architecture, the RLC SN and PDCP SN are the same for one RLC PDU for all these cases, so only one SN transmitted over the Uu interface is feasible. However, if DC is supported, the support of a common single SN for split bearer needs to be further studied.

Proposal 4: RAN2 to study the usage of a common SN for PDCP and RLC.

PDCP concatenation

As mentioned above in 5G NR concatenation of PDCP SDUs is not supported. Even though there is a one-to-one mapping between PDCP PDU and RLC PDU, the PDCP layer as well as the RLC layer assigns its own sequence number (SN), which increases the general Layer 2 overhead as well as the processing complexity.

Supporting concatenation at PDCP layer offers several notable benefits that can be further studied for 6G:

- The number of L2 headers is significantly reduced as the number of concatenated SDUs increases, lowering overall overhead. Currently, each IP packet has headers of each PDCP, RLC and MAC layer in 5G.

- The processing efficiency of integrity protection/verification and (de)ciphering of UP can be significantly improved by ‘concatenation’ of small PDCP PDUs e.g., TCP ACKs.

- It enables full utilization of the data processing capacity of hardware accelerators designed for 9000-byte payloads, maximizing hardware efficiency.

Proposal 5: RAN2 to study the concatenation of SDUs at the PDCP layer, e.g. PDCP concatenation.

Layer 2 retransmission protocols/error recovery

One promising area for further investigation is the consolidation of data recovery mechanisms currently implemented across multiple protocol layers in 5G. These include Hybrid Automatic Repeat reQuest (HARQ) at the PHY/MAC layers, Automatic Repeat reQuest (ARQ) at the RLC layer, data recovery at the PDCP layer, and retransmission mechanisms at higher layers such as TCP. Exploring potential simplifications or streamlining of these overlapping mechanisms within the Access Stratum (AS) could lead to improved efficiency and reduced protocol complexity.

At the MAC/PHY layers, HARQ enables rapid retransmissions to correct transmission errors. However, residual HARQ errors can still occur due to factors such as inaccurate link adaptation—where the selected Modulation and Coding Scheme (MCS) is not well-matched to current channel conditions—or errors in HARQ feedback signaling. These feedback errors may include ACK-to-NACK, NACK-to-ACK, or Discontinuous Transmission (DTX)-to-ACK misdetections on PUCCH or PDCCH.

To recover from such residual HARQ errors, the RLC layer provides ARQ functionality, where retransmissions are performed at the level of RLC Service Data Units (SDUs) or RLC SDU segments in Acknowledged Mode (RLC AM). Additionally, at the PDCP layer, retransmissions of lost data can occur under specific events such as handover, where HARQ and RLC processes are reset.

As discussed in our companion paper [x] by increasing the reliability of the HARQ protocol and also considering the additional reliability offered by AI/ML expected to be available from day one the recovery mechanism on RLC layer can be simplified. In our understanding there is a need for having both retransmission mechanisms which are currently standardized in RLC and PDCP. However, both PDCP and RLC receivers need to maintain some state variables, timers and windows which adds to the processing complexity. One possible investigation area is study whether to have a retransmission functionality in one layer which covers all the purposes. This may reduce the complexity of UP protocol.

Proposal 6:	RAN2 to study potential L2 protocol simplification by considering a common retransmission mechanism at L2 covering RLC ARQ and PDCP retransmissions.

Logical Channel Prioritization

The currently specified Logical Channel Prioritization (LCP) procedure in 5G NR is quite in-flexible: all PDUs belonging to a given logical channel (LCH) / radio bearer are treated uniformly from a QoS and scheduling perspective because the LCH priority is statically configured. In practice, packets multiplexed within the same LCH can have very different contributions to user experience and their relative importance may change over time (for example due to delay evolution or application-layer significance). XR-related enhancements introduced priority adaptation based on delay status (e.g. elevating an LCH’s priority when delay-critical data is pending). However, these enhancements operate only at the inter-LCH level: when an LCH’s priority is elevated, all packets of that LCH are elevated, regardless of whether an individual packet is delay-critical or not. In other words, intra-LCH prioritization is not supported.

Further, current UL resource allocation and prioritization do not consider other packet-level factors that affect user experience, such as:

- Per-packet delay urgency (time-varying deadlines within an LCH);

- Per-packet importance (relative contribution to application-level QoE, which may change dynamically);

- Inter-packet dependencies (e.g. coding/decoding or rendering dependencies between packets within or across LCHs).

These limitations can cause suboptimal scheduling and transport block formation: delay-critical or high-value packets may be delayed behind lower-value packets from the same LCH, or resources may be allocated inefficiently because static LCH priorities do not reflect packet-level urgency.

In 6G RAN2 should study a more flexible LCP and transport block generation framework that supports a fine-grained, e.g. per-packet, prioritization and time-variant priority adaptation. The framework should LCP procedure to adjust the prioritization to the changing significance of a data packet from the perspective of user experience thereby improving resource efficiency and end-user experience.

Proposal 7: RAN2 to discuss enhancements for the Uplink prioritization /LCP procedure compared to NR LCP, e.g. adjust to the changing significance of a data packet from the perspective of user experience

# Conclusion

In this contribution, we discuss User Plane (UP) aspects of the 6G Radio Protocol Architecture. We have the following observations and proposals:

Observation 1:  The Layer 2 PDU format/header design philosophy largely aimed at optimizing overhead “down to the last bit”. While this approach minimizes signaling overhead, it has resulted in complex and fragmented format/header structures in the specifications. For 6G, such complexity should be avoided.

Proposal 1: RAN2 should study solutions which enable UE to adapt NW parameter/configurations, e.g. RLC timer or LCH priority, intelligently within some limited ranges which are under network control.

Observation 2: native application-awareness at the Access Stratum in 6G would be beneficial and should be further studied.

Proposal 2: pre-processing functionality is supported in 6G.

Proposal 3: Segmentation/reassembly functions as well as reordering/in-order delivery functions and out-of-order delivery is supported in 6G.

Proposal 4: RAN2 to study the usage of a common SN for PDCP and RLC.

Proposal 5: RAN2 to study the concatenation of SDUs at the PDCP layer, e.g. PDCP concatenation.

Proposal 6:	RAN2 to study potential L2 protocol simplification by considering a common retransmission mechanism at L2 covering RLC ARQ and PDCP retransmissions.

Proposal 7: RAN2 to discuss enhancements for the Uplink prioritization /LCP procedure compared to NR LCP, e.g. adjust to the changing significance of a data packet from the perspective of user experience

# References

[1]