3GPP TSG RAN WG2 Meeting #132                                                           R2-2508630

# Dallas, USA, Nov. 17th-21st, 2025

Agenda item:	10.3.2.1

Source:	Ofinno

Title:	RRC Modelling and connection management

Document for:	Discussion and decision

# Introduction

This document discusses the control plan aspects and key features for 6G system based on the agreed 6G SID [1].

Agreements Continue study - Idle mode – - Connected mode – - Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW. Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements. We can consider the pros/cons of different modelling, after having some discussions on the functionality.

# Discussion

## Inactive mode/ sub-state:

In RAN2#131bis meeting, it was agreed to study a third RRC mode/state/sub-state besides an idle mode (or RRC IDLE state) and a connected mode (or RRC CONNECTED state). However, companies have different views on this third RRC (sub)state or mode (referred for simplicity as 6G-RRC_INACTIVE).

5G introduced RRC INACTIVE state as to bridge RRC IDLE state (no RRC context in RAN) and RRC CONNECTED state (full context in RAN) to address the inefficiencies of the IDLE-CONNECTED model for bursty and frequent traffic. A full transition from IDLE to CONNECTED for every small data burst incurred high latency and significant battery drain, as the UE repeatedly had to go through a long signaling sequence. By retaining the UE context in the RAN, 5G's RRC INACTIVE state significantly reduces signaling overhead and latency, making it a powerful tool for accommodating new traffic profiles. This design choice represents a crucial lesson learned from prior generations, as it fundamentally re-architected the control plane to optimize for the IDLE-CONNECTED traffic profile. Based on the lesson, it is clear that some form of a 6G-RRC_INACTIVE is required or still preferable in 6G.

The agreements in the last meeting well describe requirements when designing the 6G-RRC_INACTIVE.

- UE based mobility

As in RRC INACTIVE state of NR, cell selection/reselection of the idle state may be used. As a baseline, we can consider that the UE based mobility is assisted by the network i.e., configuration required for the cell reselection is provided by the network. In NR, mobility within RRC INACTIVE is limited to NR RAT. On reselection to an inter-RAT cell, the UE enters RRC IDLE of the target RAT. RAN2 should study the benefit of supporting RRC INACTIVE mobility in inter-RAT cell reselection at least to NR.

- UE context keeping and identification

As in RRC INACTIVE state of NR, UE context may be stored in last serving base station and UE context retrieval procedure may be performed. Alternatively, deployments may allow a base station - within the RNA-like potentially defined for the6G-RRC_INACTIVE – to have an anchor base station that stores the UE context for the UE in the 6G-RRC_INACTIVE. RRC_INACTIVE in NR requires a UE AS context relocation when resuming or performing SDT (except for RNAU). With the potential changes of the design and architecture in 6G, RAN2 might need to study whether a UE AS context relocation when resuming or performing SDT in a different BS (than the anchor BS).

- Energy efficiency of UE and network

Long and configurable long paging DRX cycle as in NR, may lead to long sleep periods of the UE and the network.

- Fast transition from the 6G-RRC_INACTIVE to connected state

The transition mainly comprises time taken for a random-access procedure and UE context retrieval latency. If the UE moves to a new gNB outside its RNA, the new gNB may retrieve the UE's context from the last serving gNB via the Xn interface (in case of NR). This context retrieval and potential path switch introduce a delay, though it's still generally faster than a full RRC connection establishment. The context retrieval may be reduced by managing RNA efficiently as described below.

- RNA management

In 5G, while RRC INACTIVE to RRC CONNECTED transition is faster than RRC IDLE to RRC CONNECTED transition, it still contributes to initial access latency (e.g., random access and RRC message exchange/ reconfigurations). The UE context is maintained on one RAN node and RNA area are statically configuration according to the RAN node. This is sub-optimal for highly dynamic scenarios with varying traffic patterns and mobility profiles.

If network can identify the UE’s future data activity and mobility, the network can proactively move the UE to the suitable RRC state. For example, the traffic pattern of an XR service is largely different from that of an AI service or an IoT service. They may have different idle periods as well. A UE in high mobility may find a larger RNA area more efficient than a small RNA area that is suitable for UEs with low to medium mobility. 6G can reap the benefits of RRC INACTIVE state if it is optimized/ tailored to specific device types, traffic patterns and mobility profiles.

- Paging/wake-up simplifications/enhancements

In case of NR, if the last serving gNB receives downlink data from a core network while the UE is in RRC INACTIVE state, it may page in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s). This adds control plane complexity on the Xn interface to manage reachability. As described above, in 6G, by managing a smaller RNA area (e.g., cells within single base station) except high mobility UEs, paging overhead via an interface between base stations may be reduced without forcing the UE to perform more frequent RNA update.

In addition, in order to save power by reducing a false alarm, PEI and LP-WUS were introduced in NR. The purpose of two functionalities is considered same. In order to be more power efficient UE behavior, LP-WUS introduced in release 19 may be baseline for 6G paging in the 6G-RRC_INACTIVE as well as the idle state.

During RAN2#131bis, some companies indicated the limited/failed commercialization and deployment of RRC INACTIVE state is a result of high complexity on UE and network. However, RRC INACTIVE state is mandatory to be supported by NR UEs and therefore is implemented in UEs. In our view, the core barrier to commercial deployment is not implementation feasibility but unclear service/use-case that explicitly values RRC INACTIVE’s low CP latency. With the growth of IoT and diverse traffic patterns in NR, efficient handling of small, sporadic data transmissions became crucial for both network performance and UE battery life. In addition to RRC INACTIVE state, 3GPP introduced SDT procedure where a UE can transmit small payloads that can be delivered with minimized signaling, while in RRC INACTIVE state.

Observation 1: Limited/ failed commercial deployment of NR RRC INACTIVE is not due to implementation feasibility but unclear services/ use-cases that utilizes its gains. Most of the functionalities of NR RRC INACTIVE are valid for 6G-RRC_INACTIVE.

- The functionality of NR RRC INACTIVE state is taken as baseline for 6G-RRC_INACTIVE while allowing necessary enhancements to optimize for the 6G supported services. whether to additionally support:

- Inter-RAT cell reselection in RRC_INACTIVE (at least to/from NR)

- Wake up signal design (e.g., PEI and/or LP-WUS like) to page in 6G-RRC_INACTIVE

As a broad concept in relation to data transmission, RRC INACTIVE state refers to a two-step process where a first step is to transmit an RRC resume message to enter RRC CONNECTED state, and a second step is to transmit data once the UE is in RRC CONNECTED state. On the other hand, SDT refers to a single step process where the data is embedded within the RRC resume, thereby reducing latency for initiating data transmission as compared to that of other RRC INACTIVE state data transmissions. SDT can be a powerful enabler for use cases/ services that depend on low control plane latency gain and therefore can be a bedrock to RRC INACTIVE state in 6G.

However, SDT in NR has its own shortcomings. SDT is performed either as RA-SDT or as CG-SDT if total UL data across SDT-enabled bearers ≤ configured threshold, and for CG-SDT - if the CG is valid as well. Otherwise, the normal data transmission scheme is used (e.g., RRC resume from RRC INACTIVE state to RRC CONNECTED state).  An SDT attempt (e.g., sending data in RRCResumeRequest) fails if the anchor gNB is unreachable. The procedure will time out, forcing the UE to transition to RRC IDLE state and start a full connection from scratch, defeating the purpose of SDT.

Observation 2: The primary distinction between NR RRC Inactive and SDT is the "two-step" nature of data transfer in RRC Inactive versus the "one-step" mechanism of SDT. SDT offers lower latency for the first data packet, but RRC Inactive provides a more flexible state for subsequent.

In RRC INACTIVE state, similar latency and signaling gains can be achieved in both UL and DL using the fast transition mechanisms implemented for RRC INACTIVE state. For UL, having SDT along with RRC INACTIVE helps improve the latency and signaling gains for allowing the UE a faster transition to a data transmitting state.  However, for DL, a UE in RRC Inactive must be paged and execute the full RRC Resume procedure to receive DL data. There effectively exist a functional gap in fast transition between UL and DL use cases in RRC INACTIVE state. Having a faster transition for DL use cases, in RRC INACTIVE, could further improve efficiency.

Observation 3: There is a clear gap in functionality for efficient small DL data delivery to UEs in low-power states, as compared that of UL data transmission.

- RAN2 to evaluate the feasibility and define mechanisms for fast transition from RRC INACTIVE state for DL data delivery and UL Data transmission. SDT of NR is considered a baseline to allow data and/or signaling transmission while remaining in the third RRC state.

## Connection management

During early NR discussions, RAN2 discussed several options to reduce the latency for state transition to RRC CONNECTED state. Eventually, the consensus in RAN2 landed on limiting implementation complexity. As a result, a full transition from RRC IDLE state to RRC CONNECTED state requires a series of distinctive steps: RRC setup, context establishment, and security activation, all performed separately as illustrated in figure 1. This transition involves a significant delay in entering RRC CONNECTED state and efficiently performing data transmission/ reception. This sequential execution of procedures introduces a considerable delay before a UE can effectively transmit or receive data in the RRC CONNECTED state. While the RRC INACTIVE state was introduced to facilitate faster transitions to RRC CONNECTED state while maintaining power efficiency, the initial transition from RRC IDLE state remains a bottleneck.

Figure 1: procedures that contribute latency to transition to RRC CONNECTED state

6G has more stringent latency requirements as compared to 5G and framework as a foundation may prove inadequate in meeting these requirements. A critical examination of the RRC IDLE to RRC CONNECTED transition process is essential. In our view, it is beneficial to study mechanism that reduce the time taken for transitioning from RRC IDLE state to RRC CONNECTED state. This can be achieved in several ways. One potential approach is to consolidate multiple procedures or messages into a single, more efficient operation. For example, the RRC Setup procedure could be modified to include essential context information, eliminating the need for a separate RRC reconfiguration phase. This would require careful consideration of the information included in the initial setup message to avoid excessive overhead. Another approach is to reduce the overall number of procedures required to reach the RRC CONNECTED state and begin data transmission/reception.

- RAN2 to evaluate the feasibility and define mechanisms that reduce transition time from RRC IDLE to RRC CONNECTED state

RRC Reestablishment is invoked when link failure requires re-establishing an RRC connection (RRCReestablishmentRequest/Complete) using stored context at RAN and C-RNTI of the cell where link failed. This may involve additional recovery and security steps. RRC Resume is the intended fast path from RRC INACTIVE to RRC CONNECTED using stored context at RAN (RRCResume/RRCResumeComplete) and a resume ID. When the RAN node receiving the RRCResume fails to retrieve the UE context, the UE falls back to an RRC setup procedure. In NR, resume and reestablishment rely on different UE identifiers for retrieving the UE context i.e., C-RNTI and I-RNTI. Furthermore, security mechanisms are handled differently between these procedures.

The RRC Resume and RRC Reestablishment procedures are distinct but have overlapping functions. Except for the cause and triggering event, the signaling appears to be similar for these procedures. Both resume and reestablishment rely on stored UE context. The RAN node response to the request for RRC Resume/ RRC Reestablishment decides whether to resume, reestablish, setup or reject the RRC connection.

Implementing both RRC Resume and RRC Reestablishment separately in 6G results in some duplicated procedures and extra complexity (e.g., RAN node may keep multiple contexts for the same UE). Considering there is minimal difference between the 2 procedures, we think it is beneficial to have a single/ unified procedure to 1) recovery from link failure, and 2) transition from RRC INACTIVE to RRC CONNECTED state.

- RAN2 to support a unified RRC recovery procedure for: reestablishing RRC connection/ link from link failure and resuming RRC connection/ transition from RRC INACTIVE to RRC CONNECTED.

# Conclusion

The contribution discussed aspects related to RRC states and connection management for 6G. The following observations and proposals are made:

6G-RRC_INACTIVE

Observation 1: Limited/ failed commercial deployment of NR RRC INACTIVE is not due to implementation feasibility but unclear services/ use-cases that utilizes its gains. Most of the functionalities of NR RRC INACTIVE are valid for 6G-RRC_INACTIVE.

- The functionality of NR RRC INACTIVE state is taken as baseline for 6G-RRC_INACTIVE while allowing necessary enhancements to optimize for the 6G supported services. whether to additionally support:

- Inter-RAT cell reselection in RRC_INACTIVE (at least to/from NR)

- Wake up signal design (e.g., PEI and/or LP-WUS like) to page in 6G-RRC_INACTIVE

Observation 2: The primary distinction between NR RRC Inactive and SDT is the "two-step" nature of data transfer in RRC Inactive versus the "one-step" mechanism of SDT. SDT offers lower latency for the first data packet, but RRC Inactive provides a more flexible state for subsequent.

Observation 3: There is a clear gap in functionality for efficient small DL data delivery to UEs in low-power states, as compared that of UL data transmission.

- RAN2 to evaluate the feasibility and define mechanisms for fast transition from RRC INACTIVE state for DL data delivery and UL Data transmission. SDT of NR is considered a baseline to allow data and/or signaling transmission while remaining in the third RRC state.

Connection management

- RAN2 to evaluate the feasibility and define mechanisms that reduce transition time from RRC IDLE to RRC CONNECTED state

- RAN2 to support a unified RRC recovery procedure for: reestablishing RRC connection/ link from link failure and resuming RRC connection/ transition from RRC INACTIVE to RRC CONNECTED.

# Reference

- RP-252912, Study on 6G Radio, SID, September 2025.