3GPP TSG-RAN WG2 #132	R2-2508670

Dallas, USA, 17 – 21 November 2025

Agenda item:		10.3.1.3 Scheduling, retransmissions and uplink scheduling information

Title:		Discussion on UL scheduling improvements

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#131bis, RAN2 discussed a lot of User Plane (UP) aspects and reached the following agreements as base for further study [1]. In this contribution, we discuss the UL scheduling mechanisms including CG, SR/BSR, and retransmissions, and provide our views.

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

# 2. Discussion

2.1	Scheduling enhancements for CG

In the last meeting, RAN2 agreed to support at least dynamic grant (DG) and configured grant (CG), where further study on the CG is to be done. In the 5G NR, there are two types of CG, i.e. type 1 and type 2. The type 1 CG is configured and activated by RRC, while the type 2 CG is configured by RRC and activated by DCI. There can be appropriate services for each. For example, the type 1 CG could be more useful for periodical and/or deterministic traffic. The type 2 CG could be more suitable for a mixed traffic.

The CG has drawback regarding the UL resource utilization. For example, multiple CG resources can be configured with the UE, but it is possible that many of those are not really used. Each of CG resource could be small piece of radio resource, while as the number of UEs configured with CG resources or the number of reserved CG resources become larger, the unused resources also become larger.

To solve the UL resource utilization issue, one possible way is to introduce a contention-based transmissions on the same CG resources among multiple UEs. The fundamental mechanism is left to RAN1 discussion, while theoretically this would be possible. It would be expected that when the CG resources tend to be waste (i.e. many unused resources), the contention probability could be also lower. It is worth studying this approach in 6G for better resource utilization.

Proposal 1: RAN2 to study potential enhancements for the CG by introducing contention-based transmissions.

2.2	Need for enhancements to SR/BSR/DSR latency

2.1	Problems with the legacy SR/BSR/DSR

At the previous RAN2 meeting, some companies shared potential issues regarding UL data transmission latency, which seem to be introduced by the SR/BSR/DSR procedure for obtaining the grant in NR.

The key point to reduce UL data transmission latency is how to enable the UE to indicate necessary UL transmission information to the network earlier. So far, for the DG-based UL scheduling, the basic steps are to send the SR for PUSCH resource request, followed by the BSR (if there are remaining data) for better resource allocation, i.e., 2-steps starting with the SR which would cause delay from the beginning. Two potential directions could be considered to reduce the latency:

- Direction 1: Enhanced SR to indicate more UL transmission (e.g., multi-bit SR to indicate rough buffer size information)

- Direction 2: Enhanced BSR with latency reduction

Regarding the Direction 1, the SR resources and SR bits are configured on the PUCCH, which is managed by L1/RAN1. Although higher layer flexibility (e.g., SR to LCH mapping) is RAN2 scope, the fundamental change on the SR resources should be studied by RAN1. RAN2 can wait for progress in RAN1, if any.

Observation 1: As the legacy SR configurations have already some flexibilities, RAN2 can wait for RAN1 makes some progress on physical layer design for PUCCH or the like in 6G.

Regarding the Direction 2, in 5G NR, dynamic uplink grant scheduling triggered by BSR is the de-facto standard. When no UL resource is available to carry the BSR MAC CE, the UE must initiate the SR procedure to obtain an additional grant and UL data transmission can start only after the subsequent UL resource allocation. This serial “SRGrant BSRData” sequence introduces a non-negligible end-to-end latency penalty that is incompatible with 6G sub-millisecond targets. Therefore, in 6G, RAN2 shall further study enhancements on BSR mechanism for E2E communication latency reduction. We discuss some possible enhancements in the next sub-section 2.2.

Observation 2: The overall procedure of DB-based UL scheduling via BSR will impact certain end-to-end data transmission latency.

Proposal 2: RAN2 study potential enhancements on BSR mechanism for latency reduction.

2.2	Possible BSR enhancements

The Figure X below shows the 5G NR dynamic UL scheduling procedure:

Figure 1. 5G NR dynamic uplink scheduling procedure

There is always study on reducing latency from BSR reporting. From our observation and consideration, some alternatives can be considered.

- Alt 1: Contention-based UL MAC CE delivery over RACH

Embed the specific UL MAC CE inside Msg-A of 2 Step RA or Msg-3 small payload of 4 Step RA, turning the random-access procedure into a one-shot scheduling request. Thus, if there is no contention happened, the SR procedure can be saved, which can somehow reduce the overall latency. However, if contention is still happened, the latency will further increase due to contention resolution procedure is needed. Therefore, RACH procedure cannot be simply reused for UL MAC CE transmission unless some enhancement is studied on contention resolution mechanism.

- Alt 2: Random selection for UL MAC CE transmission via a constructed UL resource pool or UL CG.

In details, the principle is similar to NR V2X Tx resource pool. In NR V2X, UE can either perform sensing based resource selection or random resource selection with some additional condition within the configured Tx resource pool. Since sensing is a huge challenge to UE’s power saving, of which V2X UE does not assume to be a power saving UE, sensing mechanism cannot be widely used for eMBB UE. However, random selection-based resource pool configuration can be considered for eMBB service. Similar to alt-1, random selection mechanism within the configured Tx resource pool or an UL CG will somehow cause resource collision. Therefore, collision resolution mechanism shall be further studied. In comparison with UL CG, a Tx resource pool will set UE to perform resource selection from both time domain and frequency domain, while UL CG configuration only allows the UE to perform resource selection from time domain.

- Alt 3: AI based BSR to predict upcoming data.

Current BSR mechanism is only a report of the existing buffered data for each logical, which would only happen after the data is coming. Yet with the help of native AI in 6G, the coming data can somehow be predicted via collected historical user plan data through AI. Thus, the AI based BSR will report the predicted coming data from upper layer rather than the existing buffered data. The latency can therefore be directly reduced. However, such mechanism needs a huge improvement of UE capability in 6G, which may not be easily commercialized in the very beginning of 6G commercialization.

Proposal 3: RAN2 study the BSR mechanism with latency reduction via at least one of the following alternatives:

- Alt 1: Contention based UL MAC CE delivery over RACH

- Alt 2: UL MAC CE transmission via random selection within shared UL CG/ resource pool.

- Alt 3: AI-based predicted BSR to indicate the upcoming UL data.

2.3	L2 ARQ and HARQ

Error correction is a critical function in UP, traditionally achieved through HARQ at the MAC layer and optionally ARQ at the RLC layer in previous generations. HARQ (re)transmission offers rapid and efficient error correction, enabled by immediate feedback and combined decoding; however, it may occasionally fail due to lower layer signal reliability issues (e.g., UCI/ACK/NACK). Additionally, HARQ cannot differentiate between logical channels or radio bearers, as data from multiple LCH/RBs are multiplexed into a single transport block and treated uniformly.

When higher reliability is required for specific LCHs or RBs, the ARQ procedure acts as a secondary mechanism. ARQ retransmission is initiated upon receipt of an ARQ NACK (via RLC status report) from the receiver. While ARQ provides robust error correction, it is generally slower and less efficient than HARQ, involving complex transmit/receive window management, polling, and status reporting. Nevertheless, ARQ allows differentiated handling for various LCHs/RBs, including enabling/disabling Acknowledged Mode (AM), and configuring different polling and status report parameters.

HARQ+ARQ in UL                               HARQ+ARQ in DL

Figure 2. ARQ and HARQ

In 6G, we could study the possibility to remove ARQ and only use (enhanced)HARQ, or utilising HARQ feedback to trigger fast ARQ retransmissions.

Beginning with HARQ without ARQ, the following observations arise:

- In the uplink direction, the decision to continue or terminate HARQ (re)transmission of a transport block is entirely managed by the network, which issues an uplink grant specifying the appropriate HARQ process ID and NDI value. Without ARQ, HARQ alone can deliver reliable data provided that the network continues retransmissions until successful decoding; yet two consideration remains, 1) this approach must adhere to packet delay budget constraints, 2) because HARQ retransmission is per TB, to achieve  per LCH/RB/QoS flow/packet control, even with service delay awareness, we have to either restrict the data mapping to a TB or compromise QoS requirement among difference services mapping into the same TB.

- In downlink direction, the network similarly determines whether to proceed or stop HARQ (re)transmissions. If ACK/NACK signalling is sufficiently reliable, HARQ alone can ensure data transmission reliability; yet several considerations remain: (1) HARQ operation is subject to the packet delay budget limit—fortunately, in the downlink, the gNB has access to PDB information; (2) the reliability of HARQ ACK/NACK signalling is always an issue for lossless delivery; 3) same as for uplink,  because HARQ retransmission is per TB, to achieve per LCH/RB/QoS flow/packet control, even with service awareness, we have to either restrict the data mapping to a TB or compromise QoS requirement among difference services mapping into the same TB.

Come to HARQ with ARQ

- In case loss-based delivery is ok for a service, enhanced HARQ with service awareness might be good enough without ARQ is enabled

- In case loss-less data delivery is required for some services, HARQ itself is not enough due to the NACK/ACK error, ARQ function is needed.

- When ARQ is in place, even though it might be risky to use HARQ ACK to terminate ARQ retransmission, it is beneficial to use HARQ NACK feedback to trigger fast ARQ retransmission, where occasionally ACK to NACK error would only mean occasionally unnecessary retransmission.

Observation 3: HARQ could be enhanced by improving HARQ ACK/NACK reliability and with service /delay awareness

Observation 4: If NACK to ACK error exists and lossless delivery is required for certain service, we could not fully remove ARQ in 6G

Observation 5: HARQ NACK feedback triggered fast ARQ retransmission is beneficial, while HARQ ACK feedback triggered ARQ retransmission termination is risky

It is also the time to revisit our target of HARQ and ARQ. In [2], it provides an interesting observation that “Although NR was standardized during the dominance of loss-based congestion control, application servers are now increasingly adopting delay-based algorithms like BBR and L4S.”  this means loss-less delivery with HARQ plus ARQ (with legacy RLC AM mode is configured) become less attractive. Instead, enhanced HARQ or legacy HARQ plus enhanced RLC AM as for XR service in Rel19 is more attractive to ensure certain reliability rather than guaranteed loss-less delivery.

Observation 6: 6G RAN UP error correction shall target loss-allowed and delay-aware data delivery

Proposal 4: RAN2 consider following regarding HARQ and ARQ:

- Coordinate with RAN1 to study HARQ feedback reliability improvement and service-aware HARQ procedure

- We shall keep ARQ in 6G

- RAN2 study HARQ NACK triggered ARQ retransmission by tight coordination between HARQ and ARQ

- In addition to legacy RLC UM and RLC AM, ARQ retransmission within limit time budget (i.e. enhanced RLC AM for XR) should be a 6G day 1 feature (a new mode name is preferred)

# 3. Conclusion

In this contribution we discussed the UL scheduling mechanisms including CG, SR/BSR, and retransmissions, and provide our views.

Scheduling enhancements for CG

Proposal 1: RAN2 to study potential enhancements for the CG by introducing contention-based transmissions.

Enhancements to SR/BSR/DSR latency

Observation 1: As the legacy SR configurations have already some flexibilities, RAN2 can wait for RAN1 makes some progress on physical layer design for PUCCH or the like in 6G.

Observation 2: The overall procedure of DB-based UL scheduling via BSR will impact certain end-to-end data transmission latency.

Proposal 2: RAN2 study potential enhancements on BSR mechanism for latency reduction.

Proposal 3: RAN2 study the BSR mechanism latency reduction via at least one of the following alternatives:

- Alt 1: Contention based UL MAC CE delivery over RACH

- Alt 2: UL MAC CE transmission via random selection within shared UL CG/ resource pool.

- Alt 3: AI-based predicted BSR to indicate the upcoming UL data.

L2 ARQ and HARQ

Observation 3: HARQ could be enhanced by improving HARQ ACK/NACK reliability and with service /delay awareness

Observation 4: If NACK to ACK error exists and lossless delivery is required for certain service, we could not fully remove ARQ in 6G

Observation 5: HARQ NACK feedback triggered fast ARQ retransmission is beneficial, while HARQ ACK feedback triggered ARQ retransmission termination is risky

Observation 6: 6G RAN UP error correction shall target loss-allowed and delay-aware data delivery

Proposal 4: RAN2 consider following regarding HARQ and ARQ:

- Coordinate with RAN1 to study HARQ feedback reliability improvement and service-aware HARQ procedure

- We shall keep ARQ in 6G

- RAN2 study HARQ NACK triggered ARQ retransmission by tight coordination between HARQ and ARQ

- In addition to legacy RLC UM and RLC AM, ARQ retransmission within limit time budget (i.e. enhanced RLC AM for XR) should be a 6G day 1 feature (a new mode name is preferred)

# References

[1] RAN2#131bis Chair notes