# 3GPP TSG-RAN2 Meeting #132	R2-2508755

# Dallas, USA, Nov 17-21, 2025	Revision of R2-2507467

Agenda Item:	8.18.1

Work Item:	LTE_terr_bcast_Ph2-Core

Source: 	Qualcomm Incorporated

Title: 	WI TerrBcast ASN.1 comments file

Document for:	Discussion/Decision

This document provides the ASN.1 comments file for WI LTE based 5G Broadcast (TerrBcast in short for the purpose of ASN.1 review).

The changes compared to R2-2508755 are shown with green background.

# LTE Based 5G Broadcast: Comments file

Template:

# Xnnn

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Xnnn | TerrBcast |  |  |  |  |  | vnnn | ToDo |

[Description]:

[Proposed Change]:

[Comments]:

Instructions:

- Copy the template RIL comments fields above (including the Heading Xnnn)

- Paste the RIL comments fields at its position while respecting the order of the RILs in the Review file (i.e. keep the order of the spec).

- Fill in the fields, see R19 ASN.1 Guideline.

- Companies may comment whether they agree or disagree.

- Can copy spec text and use Word “Track changes”, etc.

- Do not delete text added by other companies.

# S903

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S903 | TerrBcast | 2 | Configuration of Last MTCH of time-interleaved MCH | R2-2507581 | Vinay (Samsung) |  | V004 | Agreed |

[Description]: M and/or N value for last MTCH service (residual space) may be less than other preceeding MTCH services of the time-interleaved MCH. Further, RAN1 parameter list also allows to have the time-interleaving disabled for the last MTCH service.

RAN 1 parameter list:

| pmch-TimeInterleaving-N-lastMTCH | New |  | This field indicated an additional optional TFI configuration for the last MTCH service (residual space) | {2,4,8,16} | The absent of this field indicates time-interleaving for these MTCHs is disabled, i.e., N=1 |
| --- | --- | --- | --- | --- | --- |

However, RRC spec is ambiguous as does not address/support the case 3

- Case 1: Last MTCH service applies same M and N as other scheduled MTCH services of the MCH (i.e., pmch-TimeInterleaving-N-lastMTCH is absent)

- Case 2: Last MTCH service applies M and/or N lesser than other scheduled MTCH services of the MCH (i.e., pmch-TimeInterleaving-N-lastMTCH is present)

- Case 3: Last MTCH has not applied time-interleaving but other scheduled MTCH services have time-interleaving enabled (i.e., pmch-TimeInterleaving-N-lastMTCH = 1 indicated)

[Proposed Change]:

pmch-TimeInterleavingConfig-r19		SEQUENCE {

pmch-TimeInterleavingM-r19			ENUMERATED {sf4, sf8, sf16, sf32},

pmch-TimeInterleavingN-r19			ENUMERATED {n2, n4, n8, n16},

pmch-TimeInterleavingM-LastMTCH-r19	ENUMERATED {sf4, sf8, sf16, sf32}	OPTIONAL,	-- Need OR

pmch-TimeInterleavingN-LastMTCH-r19	ENUMERATED {n1, n2, n4, n8, n16}			OPTIONAL,	-- Need OR

pmch-SoftBufferSizeParameters-r19	PMCH-SoftBufferSizeParameters-r19,

pmch-CyclicShiftAlpha-r19 			ENUMERATED {alpha1, alpha2, alpha3}	OPTIONAL 	-- Need OR

}					OPTIONAL, -- Need OR

pmch-TimeInterleavingConfig Presence of the field indicates time interleaving is enabled as specified in TS 36.212 [22] and TS 36.213 [23].

pmch-TimeInterleavingM Indicates the separation, in number of MBSFN subframes not containing MCCH and MSI, between two successive transmissions of the same TB (except for the last MTCH service if pmch-TimeInterleavingM-LastMTCH is present) as specified in in TS 36.212 [22] and TS 36.213 [23] when time interleaving is enabled. Value sf4 indicates 4 subframes, value sf8 indicates 8 subframes and so on.

pmch-TimeInterleavingM-LastMTCH Indicates the separation, in number of MBSFN subframes not containing MCCH and MSI, between two successive transmissions of the same TB for the last MTCH service (residual space) as specified in TS 36.212 [22] and TS 36.213 [23] when time interleaving is enabled. Value sf4 indicates 4 subframes, value sf8 indicates 8 subframes and so on. If this field is absent and pmch-TimeInterleavingN-LastMTCH is absent, pmch-TimeInterleavingM applies also for the last MTCH service.

pmch-TimeInterleavingN Indicates the TBS scaling factor (except for the last MTCH service if pmch-TimeInterleavingN-LastMTCH is present) as specified in in TS 36.212 [22] and TS 36.213 [23] when time interleaving is enabled. Value n2 indicates scaling factor 2, value n4 indicates scaling factor 4 and so on.

pmch-TimeInterleavingN-LastMTCH Indicates the TBS scaling factor for the last MTCH service (residual space) as specified in in TS 36.212 [22] and TS 36.213 [23] when time interleaving is enabled. Value n2 indicates scaling factor 2, value n4 indicates scaling factor 4 and so on. Value n1 indicates time interleaving is not applied for last MTCH service. If this field is absent, pmch-TimeInterleavingN applies also for the last MTCH service.

[Comments]: [QC as Rapp, v003] Comment is valid. Suggest discussing during meeting based on Tdoc and merging the TP to WI-specific RRC CR.

[QC as Rapp, v004] Updated status to “Agreed” based on agreement in RAN2#131bis. Proposed changes covered by RRC CR in R2-2507927.