3GPP TSG-RAN WG2 #132	R2-2508802

# Dallas, USA, 17th - 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 1059 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Assistance info for NR-NTN to IoT-NTN Cell Selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar, Boost Mobile Network, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To enable an enhanced mechanism for inter RAT cell selection between NR-NTN and IoT NTN. This ensures that UEs supporting both NR-NTN (or NR-TN) and IoT NTN can effectively perform IoT NTN cell selection as needed. The changes cover for both eMTC UE and NB-IoT UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description of assistance information for cell selection/idle mobility to IoT NTN from NR NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-RAT cell selection to IoT NTN for UEs with dual NTN RAT capabilities (NR NTN and IoT NTN) may be less efficient without assistance info and may result in service interruption or degraded performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.14.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CR 5604, TS 38.306 CR 1389 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

## 16.14	Non-Terrestrial Networks

<<Unchange parts are omitted>>

### 16.14.3	Mobility and State transition

#### 16.14.3.1	Mobility in RRC_IDLE and RRC_INACTIVE

The same principles as described in 9.2.1 apply to mobility in RRC_IDLE for NTN and the same principles as described in 9.2.2 apply to mobility in RRC_INACTIVE for NTN unless hereunder specified.

The network may broadcast multiple Tracking Area Codes (TACs) per PLMN in an NR NTN cell. A TAC change in the System Information is under network control, i.e. it may not be exactly synchronised with real-time illumination of beams on ground.

For the NTN-TN mobility, the network may broadcast cell information on NR TN and EUTRA TN coverage areas in SIB25. This is supported for Earth-Fixed, Quasi-Earth-fixed and Earth-Moving cells. The coverage information consists in a list of geographical TN areas, with associated frequency information also indicated. UE can skip TN measurement based on the broadcast TN coverage information.

The UE can determine the network type (terrestrial or non-terrestrial) implicitly by the existence of cellBarredNTN in SIB1.

The NTN ephemeris is provided in SIB19. In an NTN cell, it includes serving cell's NTN payload ephemeris and optionally neighbouring cell's NTN payload ephemeris.

The network may provide assistance information for inter-RAT cell selection from NR NTN to IoT NTN and vice versa from IoT NTN to NR NTN. A UE may use the assistance information (e.g., ephemeris information, carrier frequency) provided by the network for cell selection from NR NTN to IoT NTN.

End of change