3GPP TSG-RAN WG2 Meeting #132			         R2-2508825

Dallas, USA, Nov. 17th-21th

Agenda Item:	10.3.3.2

Source: 	LG Electronics Inc.

Title:         	Consideration on Energy efficiency for 6G

Document for: 	Discussion and Decision

Introduction

In the last meeting, RAN2 made some agreements on power saving for 6G as follows.

Design goal

1. 	Strive to develop solutions that consider network energy efficiency and UE power saving

Agreement

For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose.   Focus on solutions that have RAN2 impacts.

This document presents our views on power saving for 6G.

Discussion

LP-WUS operation

Long UE battery life is always a critical KPI in mobile networks, and in 5G NR, various power saving features for connected UE (UE DRX, DCP, LP‑WUS, etc.) have been introduced in RAN2 from the initial release onward. However, since each feature was developed independently in different releases, dependency issue between them has emerged, leading to increased complexity and inefficiency. For example, DCP and LP‑WUS are always configured together with DRX, and RAN2 specification unnecessarily became complicated to clearly define UE behavior in at least the following cases:

- When a DCP or LP‑WUS indication is missed before onDuration.

- When a DCP or LP‑WUS indication overlaps with DRX active time.

These situations become even worse when dual DRX groups are configured. In addition, analogous features have been defined across multiple releases. For example, one option of LP‑WUS is almost identical to DCP. Such duplication and dependency increase specification complexity and impose unnecessary burdens on UE implementation and network optimization.

Therefore, RAN2 should learn from the experience in 5G NR and, rather than maintaining multiple features for the same purpose, define the most efficient power saving mechanism as a baseline solution in 6G. In particular, unlike DCP and DRX, LP‑WUS operates based on LR (Low Power Wake‑up Radio) which can achieve greater power savings than MR (Main Radio). The longer the UE remains in LR, the greater the battery saving benefit. Therefore, we propose LP‑WUS should be defined as a baseline UE power saving scheme without any dependency on DRX.

Proposal 1.	Introduce LP‑WUS as a baseline for UE power saving scheme in RRC_CONNECTED in 6G.

If the LP-WUS is taken as a baseline, the periodic PDCCH monitoring used in DRX (e.g. PDCCH monitoring based on onDuration per DRX cycle) does not need to be considered for LP-WUS. With LP-WUS, the UE can monitor PDCCH only when the LP-WUS indicates to do so. Such event-triggered PDCCH monitoring can save UE power greatly compared to the periodic PDCCH monitoring. However, it should be discussed how to transmit CSI/SRS, which was periodically transmitted during DRX onDuration in NR. Since there is no periodic PDCCH monitoring, CSI/SRS transmission may be modelled as not being associated with onDuration.

Beside onDuration, whether to keep PDCCH monitoring behavior associated with the expected scheduling (e.g. drx-InactivityTimer and drx-RetransmissionTimer) should be discussed. In NR, drx-InactivityTimer is used for new transmission and drx-RetransmissionTimer is used for retransmission, and the UE monitors PDCCH while those timers are running. However, given that immediate PDCCH monitoring can be achieved by using LP-WUS, PDCCH monitoring for the expected scheduling is no longer needed.

Meanwhile, in 6G, more diverse type of traffic and periodicity are expected to be used, so multiple LP-WUS configurations is desirable from latency perspective. In NR, multiple DRX configurations was proposed several times, but it has not been introduced so far. We think multiple LP-WUS configurations is essential for various traffic periodicities to achieve low latency, and should be supported from Day-1.

One may think that single LP-WUS configuration with dense LP-WUS occasion is enough to support various traffic periodicities. However, as LP-WUS resource is a dedicated resource, configuring dense LP-WUS occasion to each UE is very resource inefficient. Thus, in terms of resource efficiency, it is desirable to configure LP-WUS occasion to fit to traffic periodicity than configure resource densely.

Proposal 2.	Study how to operate LP-WUS without DRX and how to configure LP-WUS in 6G.

Cell DTX/DRX operation

In NR, cell DTX/DRX were introduced to achieve network energy saving (NES) by allowing the gNB to temporarily suspend downlink or uplink activity. In practice, however, the realised NES gain is very limited, because NR still permits considerable DL/UL transmission during the cell DTX/DRX OFF period, so that RRC_IDLE UEs, which are unaware of cell DTX/DRX, can continue to operate as expected (e.g., assuming periodic SSB and configured RA opportunities), and RRC_CONNECTED UEs do not suffer unacceptable user-experience degradation (e.g., essential HARQ retransmissions). As a result, the gNB cannot fully power down its transmitter/receiver chains during the cell DTX/DRX OFF period and cannot consistently enter a deep-sleep state.

Observation 1.	The NES gain from NR cell DTX/DRX is very limited because several exceptional DL/UL transmissions are still allowed during DTX/DRX OFF periods.

To deliver meaningful NES, the DL/UL transmission allowed during cell DTX/DRX OFF period should be minimised, ideally to a truly RF OFF period in which the gNB can power TX/RX fully down. At the same time, the user experience in RRC_IDLE and RRC_CONNECTED also should be preserved. Accordingly, for 6G Day-1 we propose cell DRX/DTX that guarantees (near-)complete cell Tx/Rx OFF during cell OFF period while preserving user experience in RRC_IDLE and RRC_CONNECTED via alternative mechanisms, without relying on cell OFF period exceptions. For example, within the cell OFF period the gNB could keep only a low-power receiver active and use wake-up signalling to temporarily activate the main receiver when an urgent, network-permitted UL transmission arises. Alternatively, such urgent DL/UL could be steered to an auxiliary cell/carrier that remains ON, allowing the NES cell to stay fully RF OFF.

Proposal 3.	Study Cell DTX/DRX for 6G, focusing on significantly reducing downlink and uplink activity during OFF periods (and potentially achieving complete suspension when feasible), in order to maximize network energy saving while minimizing UE experience degradation.

RRM measurement offloading

In Rel-19, measurement offloading to a low-power receiver (LR) was introduced so that serving-cell measurements can be performed by the LR alone. Unlike cell DTX/DRX, its performance is not constrained by legacy UE presence so it seems not essential to introduce it from DAY1. However, the Rel-19 LP-WUS study indicated that LP-WUS monitoring alone, without serving-cell offloading to the LR, cannot provide meaningful UE power saving. Therefore, if LP-WUS monitoring is studied as an essential feature for 6G Day-1, measurement offloading to LR also should be Day-1 to realise meaningful gains.

In the NR measurement offloading, the LR can only measure the serving cell. Thus, even when the serving cell’s quality is good enough to offload the serving cell measurement to the LR, the UE still needs to wake the main receiver to measure neighbor cells at the required measurement periodicities.

Observation 2.	Even when measurement offloading is enabled, the UE still needs to wake the main receiver periodically to measure neighbour cells.

Since intra-frequency neighbour cells typically require tighter measurement periodicity than inter-frequency cells, enabling the LR to perform intra-frequency measurements can yield substantial additional power saving. Therefore, we propose to study an enhanced measurement offloading in which the LR can measure not only the serving cell but also intra-frequency neighbour cells under well-defined conditions.

Proposal 4.	Study enhanced 6G RRM measurement offloading whereby the LR performs serving- and intra-frequency neighbor cell measurements, to realize meaningful UE power savings in conjunction with LP-WUS monitoring.

SSB periodicity -aware RRC procedures

There is ongoing discussion in RAN1 and RAN2 to extend the default SSB periodicity beyond 20 ms to improve network energy saving. However, even if a long periodicity (e.g., 160 ms) is supported, it will not be universally applicable across all scenarios and conditions. Therefore, if a longer default periodicity is adopted, 6G Day-1 should also support mechanisms to dynamically change SSB density when needed, such as Rel-19 on-demand SSB or SSB adaptation, so that UEs can still rely on dense SSB transmission under specific conditions (e.g., access, beam recovery), while preserving energy savings otherwise.

In NR, although SSB transmission periodicity can change dynamically via lower-layer signalling, such changes have no automatic impact on other RRC procedures. While RRC procedures are basically SSB periodicity agnostic, several RRC parameters configured by the network are closely related with the effective SSB periodicity. For example, in L3 filtering used to stabilise RRM measurement results, the weighting between the most recent measurement result and the historical filtered result is determined by the filter coefficient configured via RRC. With a long SSB periodicity, it may be desirable to reduce the weight of the past measurement result, e.g., by applying a smaller filter coefficient to reflect the sparser measurement periodicity. Similarly, suitable values for T310 and N310 also may depend on the SSB periodicity, e.g. a smaller N310 to avoid delayed RLF declaration at long periodicities. However, even when the SSB periodicity changes via lower-layer signalling, these parameters do not change unless the network explicitly updates them using RRC re-configuration.

Observation 3.	When the SSB periodicity is changed via lower-layer signalling (e.g., OD-SSB or SSB adaptation), RRC parameters that are functionally tied to the effective SSB periodicity (e.g., L3 filter coefficient, T310/N310) are not automatically updated. This can distort RRM measurement filtering or lead to delayed/premature RLF declaration.

It is undesirable to rely solely on RRC reconfiguration for these updates given that SSB periodicity may change dynamically. We therefore propose to study RRC parameter adaptation whereby selected parameters (e.g., filter coefficient, T310, N310) are adjusted in response to SSB periodicity changes without RRC re-configuration.

Proposal 5.	Study RRC parameter adaptation triggered by dynamic SSB periodicity changes without RRC re-configuration for 6G.

Conclusion

Based on the above discussions, we present the following observations and proposals:

LP-WUS operation

Proposal 1.	Introduce LP‑WUS as a baseline for UE power saving scheme in RRC_CONNECTED in 6G.

Proposal 2.	Study how to operate LP-WUS without DRX and how to configure LP-WUS in 6G.

Cell DTX/DRX operation

Observation 1.	The NES gain from NR cell DTX/DRX is very limited because several exceptional DL/UL transmissions are still allowed during DTX/DRX OFF periods.

Proposal 3.	Study Cell DTX/DRX for 6G, focusing on significantly reducing downlink and uplink activity during OFF periods (and potentially achieving complete suspension when feasible), in order to maximize network energy saving while minimizing UE experience degradation.

RRM measurement offloading

Observation 2.	Even when measurement offloading is enabled, the UE still needs to wake the main receiver periodically to measure neighbour cells.

Proposal 4.	Study enhanced 6G RRM measurement offloading whereby the LR performs serving- and intra-frequency neighbor cell measurements, to realize meaningful UE power savings in conjunction with LP-WUS monitoring.

SSB periodicity -aware RRC procedures

Observation 3.	When the SSB periodicity is changed via lower-layer signalling (e.g., OD-SSB or SSB adaptation), RRC parameters that are functionally tied to the effective SSB periodicity (e.g., L3 filter coefficient, T310/N310) are not automatically updated. This can distort RRM measurement filtering or lead to delayed/premature RLF declaration.

Proposal 5.	Study RRC parameter adaptation triggered by dynamic SSB periodicity changes without RRC re-configuration for 6G.