3GPP TSG-RAN WG2 Meeting #132       	                      R2-2508844

Dallas, US, 17th-21th Nov 2025

Agenda item:	9.3.3

Source:	CMCC

Title:	Discussion on RRM measurement event prediction

WID/SID:	NR_AIML_Mob

Document for:	Discussion

- Introduction

Based on WID [1], the following objectives are associated to RRM measurement event prediction:

- Specify support for measurement event prediction for UE sided models [RAN2]: - Specify signaling and protocol aspect to enable reporting inference outcome of UE sided model for measurement event prediction - Measurement event A1~A6 are in the scope in this release - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach (considering further progress for AI/ML PHY) - Inference configuration - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection, for which AI/ML PHY request/configuration framework is taken as baseline Note 3: Data transfer and model delivery for UE sided model are out of scope of this WID. And model related choices can be up to UE’s implementation.

As per Chair’s guidance, this contribution will discuss UE capability, applicability reporting procedure, inference configuration/reporting for RRM measurement event prediction.

- Discussion

During the study phase, RAN2 studied two methods to predict measurement event, namely indirect and direct measurement event prediction as illustrated in Figure 1 and Figure 2 respectively [2].

Figure 1: Indirect measurement event prediction

In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model at first. Afterwards, the UE derives whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

Figure 2: Direct measurement event prediction

In direct prediction, measurement event is predicted directly by an AI/ML model, i.e. the output of the model is the likelihood of an event occurrence.

For the potential spec impacts of RRM measurement event prediction, the following texts are captured in TR 38.744 [2].

- 6.1.3 Measurement event prediction It is up to UE’s implementation to decide on choice between indirect and direct event prediction methodology. On top of inference parameters captured in section 6.1.2.1.2, event-related parameters e.g. event type are part of inference configuration of measurement event prediction. UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). As baseline event type A1~A6 can be predicted and reported. UE can report measurement results, predicted measurement event together with its timing related information. When UE is configured with intra-frequency temporal domain case B or inter-frequency prediction, UE reports measurement event by following existing measurementReport message. This can be achieved without specification impact. For indirect event prediction, RSRP difference can be used as performance monitoring metric. NOTE 1: The spec impact captured in section 6.1.2.1 is applicable for measurement event prediction unless otherwise described explicitly in this section. NOTE 2: A single framework is aimed for direct and indirect event prediction. NOTE 3: There is no consensus on the feasibility of performance monitoring of direct event prediction. it should be concluded before proceeding with normative work.

The applicability reporting procedure for AI/ML based beam management is specified in section 22.2.3 of TS 38.300 and illustrated as in the following Figure [3], and RAN2 agreed to use the similar procedure as baseline for AI Mob including RRM measurement prediction and measurement event prediction. In general, for UE sided model, the network provides full inference configuration (i.e. Option A) or inference related parameters (i.e. Option B) based on UE supported functionalities. And then the UE determines functionality applicability, and reports the applicability status to network. The details will be discussed in the following sections.

Figure 3: Initial applicability and applicability status change reporting

Proposal 1: For applicability reporting, the procedure for AI/ML based beam management specified in section 22.2.3 of TS 38.300 is taken as baseline.

- 2.1 UE capability

As illustrated in Figure 3 [3], the network inquires about the UE capability information, and the UE indicates its supported functionalities to the network via UECapabilityInformation message, where the supported functionalities are used for the network to provide inference configuration. Therefore, the granularity of supported functionality should be discussed. This issue was also discussed in SI but had no consensus.

During the discussion in R19 AI/ML PHY, RAN1 determined the following feature groups (i.e. supported functionalities) for beam management use case [4], and further defined the components for each feature group, including inference related parameters (e.g. RS type and maximum number of Set A, Set B), and reporting related parameters (e.g. support reporting of predicted beam index, predicted RSRP), etc..

- UE-side beam prediction for BM Case1 for inference (support of beam prediction with reporting of predicted beam index)

- UE-side beam prediction for BM Case1 with predicted RSRP for inference (support of beam prediction with reporting of predicted beam index and predicted RSRP)

- UE-side beam prediction for BM Case2 for inference (support of beam prediction with reporting of predicted beam index)

- UE-side beam prediction for BM Case2 with predicted RSRP for inference (support of beam prediction with reporting of predicted beam index + predicted RSRP)

It can be observed that the granularity of supported functionality is dependent on the inference configuration and reporting. Furthermore, the inference configuration of indirect measurement event prediction is the same as RRM measurement prediction. Therefore, we think that the discussion on granularity of supported functionality for measurement event prediction can be postponed until the inference configuration for RRM measurement prediction and measurement event prediction are clarified.

Observation 1: In AI/ML PHY, the granularity of supported functionality is dependent on the inference configuration and reporting.

Proposal 2: RAN2 postponed the discussion on the granularity of supported functionality for measurement event prediction until the inference configuration and reporting are clarified.

On the other hand, the fields applicabilityReportingCSI-r19 and applicabilityReportingOther-r19 [5] are introduced as part of UE capabilities associated to applicability reporting for AI-based beam management and CSI prediction in R19. In details, applicabilityReportingCSI-r19 indicates whether the UE supports applicability reporting and/or its updates based on inference configuration provided via CSI-ReportConfig, and applicabilityReportingOther-r19 indicates whether the UE supports applicability reporting and/or its updates based on inference related configuration provided via OtherConfig [6].

TS 38.331, 6.3.3 UE capability information elements [5]

TS 38.306, 4.2.25 AI/ML Parameters [6]

For RRM measurement event prediction, the similar parameters should be introduced to indicate whether the UE supports applicability reporting based on inference configuration or inference related parameters to the network. The details of inference configuration and signaling will be discussed in section 2.3.

Proposal 3: The two new UE capability parameters similar to applicabilityReportingCSI-r19 and applicabilityReportingOther-r19 should be introduced for RRM measurement event prediction, to indicate whether the UE supports applicability reporting based on inference configuration or inference related parameters.

- 2.2 Applicability reporting

As illustrated in Figure 3 and specified in [3], the UE receives the full inference configuration and/or inference related parameters from the network, and determines the applicable AI/ML functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in the UE. Then the UE reports initial functionality applicability in RRCReconfigurationComplete message, including explicit applicability and inapplicability status for each functionality. Furthermore, a “release configuration” flag is introduced to indicate UE’s preference to release a configuration for inapplicable functionality. When the UE indicates that an inference configuration is inapplicable, the network is expected to release it, i.e., autonomous release by UE is not supported.

On the other hand, if the network enables the applicability reporting via OtherConfig, and the applicability status changes, the UE can report to the network via UEAssistanceInformation message. When an inference configuration becomes inapplicable, the UE shall report its inapplicability via UAI.

The above procedures are also applied to measurement event prediction.

Proposal 4: The UE can be configured with a full inference configuration and/or inference related parameters for applicability reporting.

Proposal 5: The UE determines the applicable AI/ML functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in the UE.

Proposal 6: The UE can report functionality applicability via RRCReconfigurationComplete message, including explicit applicability and inapplicability status for each functionality, and a “release configuration” flag is introduced to indicate UE’s preference to release a configuration for inapplicable functionality.

Proposal 7: If the network enables applicability reporting via OtherConfig, and the applicability status changes, the UE can report to the network via UEAssistanceInformation message.

- 2.3 Inference configuration and reporting

RAN2 agreed in R19 that the existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report. And the following inference parameters can be configured to UE for inference and assessing applicability, which are applied for both RRM measurement prediction and measurement event prediction:

-	PW length for intra-frequency temporal domain case A

-	Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction

-	Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement)

-	Optional list of cells for intra-frequency temporal domain case A, for which network expects inference report (if available)

-	Optional Beam pattern e.g. to save SSB transmission for intra-frequency spatial domain prediction

-	Optional associated ID

In this contribution, we focus on the parameters only for measurement event prediction. The same parameters as RRM measurement prediction will be discussed in our contribution on RRM measurement prediction.

For measurement event prediction, RAN2 agreed that event type and event-related parameters are also part of inference configuration of measurement event prediction on top of inference parameters mentioned above. In legacy RRM measurement configuration and reporting framework, the event parameters can include the event threshold or offset, reportOnLeave, hysteresis, timeToTigger (i.e. TTT), and useAllowedCellList for event A1~A6 [5].

During R19 SI, A3-offset and TTT were used as simulation assumptions for event A3 based on TR 36.839 [7]. In general, hysteresis is also configured for event A3 in commercial network. Therefore, A3-offset, TTT and hysteresis can be configured to the UE for inference. For other event besides A3, the AX-threshold similar to A3-offset will be used. In legacy, the field reportOnLeave is used for L2 U2N relay UE, NTN or UAV scenarios, and useAllowedCellList indicates whether only the cells included in the allow-list of the associated measObject are applicable. For now, we think at least the event offset or threshold, TTT, and hysteresis can be configured to the UE for inference, and whether to configure reportOnLeave or useAllowedCellList can be further discussed.

Proposal 8: For AI based measurement event prediction, the following event-related parameters can be configured to the UE for inference, except for the same parameters as RRM measurement prediction.

- Threshold for event A1, A2, A4, A5

- Offset for event A3, A6

- Hysteresis

- TimeToTigger

- FFS: reportOnLeave, useAllowedCellList

For inference results reporting, as captured in TR 38.744 [2], the UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). As baseline event type A1~A6 can be predicted and reported. That is, when the event A1~A6 is predicted, the UE should report the inference results.

Proposal 9: Event triggered reporting is applied for AI based measurement event prediction, i.e. when the event A1~A6 is predicted, the UE should report the inference results.

Considering there are two methods for measurement event prediction (i.e. indirect and direct prediction), the reporting content will be different.

For indirect prediction, the model output is predicted RRM measurement results, and then the predicted time instance of event occurrence will be derived. Therefore, the UE can report predicted measurement results, predicted measurement event together with its timing related information to the network.

Proposal 10: For indirect prediction, the predicted measurement results, predicted measurement event together with its timing related information can be reported to the network.

For direct prediction, the model output is the likelihood of an event occurrence, e.g. X%, and no predicted RRM measurement results are generated. Therefore, the UE can report the probability of an event occurrence to the network. On the other hand, a threshold (50%) was used during the evaluation in SI, if the network configures a threshold to the UE, the UE can also report an indication to the network to indicate whether the threshold is satisfied.

Proposal 11: For direct prediction, the predicted probability of an measurement event occurrence and/or an indication which indicates whether the threshold is satisfied (if configured) can be reported to the network.

# 3	Conclusion

Here are the observations and proposals on RRM measurement event prediction.

General

Proposal 1: For applicability reporting, the procedure for AI/ML based beam management specified in section 22.2.3 of TS 38.300 is taken as baseline.

UE capability

Observation 1: In AI/ML PHY, the granularity of supported functionality is dependent on the inference configuration and reporting.

Proposal 2: RAN2 postponed the discussion on the granularity of supported functionality for measurement event prediction until the inference configuration and reporting are clarified.

Proposal 3: The two new UE capability parameters similar to applicabilityReportingCSI-r19 and applicabilityReportingOther-r19 should be introduced for RRM measurement event prediction, to indicate whether the UE supports applicability reporting based on inference configuration or inference related parameters.

Applicability reporting

Proposal 4: The UE can be configured with a full inference configuration and/or inference related parameters for applicability reporting.

Proposal 5: The UE determines the applicable AI/ML functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in the UE.

Proposal 6: The UE can report functionality applicability via RRCReconfigurationComplete message, including explicit applicability and inapplicability status for each functionality, and a “release configuration” flag is introduced to indicate UE’s preference to release a configuration for inapplicable functionality.

Proposal 7: If the network enables applicability reporting via OtherConfig, and the applicability status changes, the UE can report to the network via UEAssistanceInformation message.

Inference confiration and reporting

Proposal 8: For AI based measurement event prediction, the following event-related parameters can be configured to the UE for inference, except for the same parameters as RRM measurement prediction.

- Threshold for event A1, A2, A4, A5

- Offset for event A3, A6

- Hysteresis

- TimeToTigger

- FFS: reportOnLeave, useAllowedCellList

Proposal 9: Event triggered reporting is applied for AI based measurement event prediction, i.e. when the event A1~A6 is predicted, the UE should report the inference results.

Proposal 10: For indirect prediction, the predicted measurement results, predicted measurement event together with its timing related information can be reported to the network.

Proposal 11: For direct prediction, the predicted probability of an measurement event occurrence and/or an indication which indicates whether the threshold is satisfied (if configured) can be reported to the network.

# 4	Reference

- RP-251864 New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR

- TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2

- R1-2506627 Updated RAN1 UE features list for Rel-19 NR after RAN1 #122

- TS 38.331, NR; Radio Resource Control (RRC); Protocol specification

- TS 38.306, NR; User Equipment (UE) radio access capabilities

- TR 36.839, Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks