3GPP TSG-RAN WG2 #132	R2-2509140

# Dallas, TX, USA, 17th – 21st November 2025

Agenda Item:	8.1.3

Source:	Ericsson

Title:	Discussion on RILs for NW-side data collection (H009/Z005, S052)

Document for:	Discussion

# 1	Introduction

In this paper, we discuss and address some RILs identified for NW-side data collection.

# 2 	Discussion

## 2.1 (H009/Z005) Interactions with PHY for NW-side data collection

The RILs H009/Z005 propose to add in the procedural text for logging for NW-side data collection, that the lower layers are instructed to start/stop the L1 measurements based on whether the L3 event for logging is fulfilled or not. In other words, the UE should not unnecessarily perform the L1 measurements when the RRC layer does not log them (due to the L3 event not being fulfilled).

While the intention of the proposed changes is meaningful, it was already discussed in past RAN2 meetings that the same behaviour can be achieved via UE implementation. Furthermore, RAN2#131 sent an LS to inform RAN1 about the agreements regarding logging for NW-side data collection and RAN1 has not replied to ask for any further changes. It may not be sufficient that RAN2 independently decides to adopt the proposed changes in H009/Z005, to add procedural text to instruct the lower layers when to start/stop performing L1 measurements, if the subsequent behaviour is not aligned with the RAN1 specs (i.e. RAN1 specs do not capture that the UE indeed starts/stops the L1 measurements). Thus, for a fully specified solution, coordination with RAN1 is required again. To avoid additional LSs at this late stage, it is simpler to leave it to UE implementation when to stop performing L1 measurements.

(H009/Z009) It is up to the UE implementation to stop performing L1 measurements during the time that a configured L3 event for NW-side data collection is not fulfilled. (Reject H009/Z009.)

## 2.2 (S052) Quantity configuration for Event triggered Logging of  CSI measurements

There is support for event triggered logging of CSI measurements for NW-side data collection. It is then possible to configure A1 or A2 events for the corresponding serving cell (where the data collection is to be performed) in order to control when the logging should take place.

The RIL S052 discusses how the quantity configuration for the L3 filtering, which is to be used for the event evaluation, should be done. It includes two options; a) to provide quantity configuration in the CSI measurement framework, and b) to use the configuration for the ServingCellMO.

To our understanding it is clear that the configuration for the servingCellMO for the corresponding serving cell is to be used for the event evaluation (which is done for the serving cell).

As can be seen below, both the servingCellMO and the csi-MeasConfig are included within the ServingCellConfig. The servingCellMO is mandatory present for the SpCell and optionally present for the SCells. If event triggered logging is to be configured for an SCell, the servingCellMO then also needs to be provided for that SCell if not already configured. It can be checked whether any clarification is needed.

(S052) The UE uses quantity configuration for L3 event evalution as provided by servingCellMO for the same serving cell.

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                                OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                                    OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                               OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink                         OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                                   OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                                                                  OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease { PDCCH-ServingCellConfig }                                OPTIONAL,   -- Need M

pdsch-ServingCellConfig             SetupRelease { PDSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

csi-MeasConfig                      SetupRelease { CSI-MeasConfig }                                         OPTIONAL,   -- Need M

sCellDeactivationTimer              ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}       OPTIONAL,   -- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                            OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

dummy1                              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pathlossReferenceLinking            ENUMERATED {spCell, sCell}                                              OPTIONAL,   -- Cond SCellOnly

servingCellMO                       MeasObjectId                                                            OPTIONAL,   -- Cond MeasObject

...,

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	(H009/Z009) It is up to the UE implementation to stop performing L1 measurements during the time that a configured L3 event for NW-side data collection is not fulfilled. (Reject H009/Z009.)

Proposal 2	(S052) The UE uses quantity configuration for L3 event evalution as provided by servingCellMO for the same serving cell.