3GPP TSG-RAN WG2 Meeting #131	draft R2-2507285

Bengaluru, India, 25th - 29thAugust 2025

Title:	LS on S&F mode indications to NAS

Response to:	-

Release:	Rel-19

Work Item:	IoT_NTN_Ph3-Core

Source:	RAN2

To:	CT1, SA2

Cc:	-

Contact person:	Siva Vakeesar

Siva.Vakeesar@sateliot.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN2#131 had discussion on the following:

- It has been escalated that currently there is no coordination between AS and NAS in terms of selecting a suitable cell and as a result what is suitable from AS may turn out to be unsuitable to NAS. This will unnecessarily prolong cell (re)selection process while unnecessarily draining battery energy of IoT devices. At the moment AS of a UE knows whether a detected cell operates in the S&F mode and a corresponding Satellite ID, while the NAS has the S&F monitoring List.  For AS to select a suitable cell, related cell (re)selection behaviour needs to be specified in TS 36.304 while ensuring NAS passes the S&F monitoring list on to AS. If, on the other hand, for NAS to consider appropriate cell, AS needs to pass whether the detected cell operates in the S&F mode and a corresponding satellite ID. RAN2#131 has already agreed to pass S&F mode indication of a detected cell on to upper layers and specify related UE behaviour in TS 36.331. Hence, RAN2 believes that the only information that is required for NAS to assist in selecting a suitable cell is Satellite ID of a detected cell such that it would be compared against the contents of S&F monitoring list available at the NAS level. In this respect,

  - RAN2 asks whether the Satellite ID information broadcast in SIB should be sent to NAS so that NAS can help AS select a suitable cell on time without wasting energy of a UE.

- Also, for the S&F monitoring list to be useful, there needs to be consistency in terms of Satellite ID being used in AS (e.g., SIB31/SIB32/SIB33) and NAS. This means that the same identifier has to be used at the AS and NAS levels when it refers to the same satellite. In this respect, RAN2#131 agreed on the following in principle:

- Stage-2 description in 36.300 to state that the satellite identifier values used at the AS-level (e.g., satellite identifiers included in SIB messages) and the NAS-level (e.g. satellite identifiers included in the S&F Monitoring list) are set consistently, i.e. with the same value, when they refer to the same satellite

# 2	Actions

ACTION: 	RAN2 respectfully asks CT1 to take above information 1) and 2) into account, consider possible specification changes in NAS (if required) and provide answer for 1).

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #131bis	2025-10-13 ~ 2025-10-17 	Prague, CZ

TSG RAN WG2 Meeting #132	2025-11-17 ~ 2025-11-21 	Dallas, US