3GPP TSG-RAN WG2 Meeting #131                                                        R2-250xxxx

Bengaluru , IN, 25 – 29 Aug, 2025

Title: 	Discussion on TP for Reflective Bearer mapping

Source: 	ZTE(Rapporteur)

Agenda item:	8.13.3

Document for:	Discussion and Decision

# Introduction

Per below F2F discussion, in this contribution, we will further discuss the TP for Reflective bearer mapping.

- [AT131][406][Relay] TP on reflective bearer mapping (ZTE)

Scope: F2F offline to check the TP from R2-2505102 for correctness/completeness.

Intended outcome: Report to Wednesday relay session in R2-2506312

Schedule: Tuesday 1030-1100 In Brk3

Deadline: Tuesday 2025-08-27 1900 IST

# Discussion

According to taday’s online discussion, this offline focus on the TP for reflective bearer mapping, so let’s assume that the reflective bearer mapping is to be supported, and please provide the comments on how to complete the TP for reflective bearer mapping.

So, if companies think some changes are necessary to support reflective bearer mapping, please create a new option and clarify/provide the details.

## RRC SRAP configuration for multi-hop remote UE

Rapporteur think legacy remote UE’s list works well for indirect path remote UE, i.e. Network can put indirect remote UE’s L2 ID in sl-L2IdentityRemote-r17 of  SL-RemoteUE-ToAddMod-r17 in sl-MappingToAddModList-r17. From intermediate relay UE’s perspective, since it can not recognize the indirect remote UE’s L2 ID, it only considers the L2 ID to be the index of corresponding SRAP configuration, nothing is broken.

For example:

For remote UE1’s traffic, the relay UE3’s SRAP configuration would be:

SL-L2RelayUE-Config-r17 ::= SEQUENCE { sl-RemoteUE-ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17 OPTIONAL, -- Need N => Reflective mapping(UE1 SRAP configuration), sl-RemoteUE-ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16 OPTIONAL, -- Need N } SL-RemoteUE-ToAddMod-r17:{ sl-L2IdentityRemote-r17 => Reflective mapping(UE1 L2 ID), SL-SRAP-Config-r17, [[ sl-SRAP-ConfigRelay-ToAddModList-r19 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-Config-ToAddMod-r19 OPTIONAL, -- Need R =>Current Running CR(UE1’s SRAP configuration) sl-SRAP-ConfigRelay-ToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-ConfigId-r19 OPTIONAL -- Need R ]] } SL-SRAP-Config-r17 ::= SEQUENCE { sl-LocalIdentity-r17 => Reflective mapping(UE1 local ID), sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17 OPTIONAL, -- Need N sl-MappingToReleaseList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17 OPTIONAL, -- Need N ... }

- For reflective bearer mapping, downselect below options for RRC SRAP configuration at relay UE for remote UE’s traffic:

- Option1: re-use legacy remote UE add/mod/release list to configure indirect remote UE’s SRAP configuration in intermediate relay UE.

- Option2: please clarify the detailed changes.

## Downlink Egress link determination

Reflective bearer mapping is that UE can determine downlink egress link based on uplink traffic, so rapporteur propose below TP to achieve this:

#### 5.2.2.1 Egress link determination For a SRAP Data PDU to be transmitted, SRAP entity shall: - If there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU: - If sl-L2IdentityRemote configured for the concerned sl-LocalIdentity is the directly connected Remote UE; - Determine the egress link on PC5 interface corresponding to sl-L2IdentityRemote configured for the concerned sl-LocalIdentity as specified in TS 38.331 [3]. - else: - Determine the egress link on PC5 interface corresponding to the connected UE from which the uplink SRAP Data PDU having UE ID filed matching the sl-LocalIdentity in this SRAP Data PDU is received.

- For reflective bearer mapping, downselect  below options for downlink egress link determination at relay UE for remote UE’s traffic:

- Option1: adopt the above TP

- Option2: please clarify the detailed changes

## Downlink and uplink Egress RLC channel determination

Reflective bearer mapping has nothing to do with RLC channel determination, so no dedicated changes is needed to support it. Same as baseline procedure, we still need network to configure RLC channel mapping, so the new added sl-EgressRLC-Channel-UL and sl-EgressRLC-Channel-DL are still needed, which is common part for reflective bearer mapping and current running CR, TP having common part change can be seen in annex clause 4.2.

- For reflective bearer mapping, ignoring the common part changes, downselect below options for RLC channel determination at relay UE for remote UE’s traffic:

- Option1: no dedicated change is needed to support reflective bearer mapping

- Option2: please clarify the detailed changes

## Error handling

- Some companies have concern on that reflective bearer mapping does not work for error handling. From our view, current error handling is based on information configured in sl-SRAP-ConfigRelay, as shown in below:

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall: - Discard the received SRAP Data PDU.

- Since we only need reflective bearer mapping for downlink next hop UE determination, the RB to PC5 RC5 RLC channel mapping for uplink and downlink are still configured via sl-SRAP-ConfigRelay. So, intermediate relay UE can still obtain the indirect remote UE’s sl-LocalIdentity or sl-RemoteUE-RB-Identity , and “ sl-L2IdentityRemote of the ingress link” can be derived via reflective bearer mapping. Therefore, above error handling procedure can be re-used without any change.

- For reflective bearer mapping, downselect below options for error handling  at relay UE for error handling for remote UE’s traffic:

- Option1: re-use legacy error handling procedure

- Option2: please clarify the detailed changes

# Conclusion

# Annex

## 38.331

#### – SL-L2RelayUE-Config The IE SL-L2RelayUE-Config is used to configure L2 U2N relay operation related configurations used by L2 U2N Relay UE, or L2 U2U relay operation related configurations used by L2 U2U Relay UE. SL-L2RelayUE-Config information element -- ASN1START -- TAG-SL-L2RELAYUE-CONFIG-START SL-L2RelayUE-Config-r17 ::= SEQUENCE { sl-RemoteUE-ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17 OPTIONAL, -- Need N sl-RemoteUE-ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16 OPTIONAL, -- Need N ..., [[ sl-U2U-RemoteUE-ToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-U2U-RemoteUE-Config-r18 OPTIONAL, -- Need N sl-U2U-RemoteUE-ToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16 OPTIONAL -- Need N ]] } SL-RemoteUE-ToAddMod-r17 ::= SEQUENCE { sl-L2IdentityRemote-r17 SL-DestinationIdentity-r16, sl-SRAP-ConfigRelay-r17 SL-SRAP-Config-r17 OPTIONAL, -- Need M ... } #### – SL-SRAP-Config The IE SL-SRAP-Config is used to set the configurable SRAP parameters used by L2 U2N Relay UE and L2 U2N Remote UE as specified in TS 38.351 [66]. SL-SRAP-Config information element -- ASN1START -- TAG-SL-SRAP-CONFIG-START SL-SRAP-Config-r17 ::= SEQUENCE { sl-LocalIdentity-r17 INTEGER (0..255) OPTIONAL, -- Need M sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17 OPTIONAL, -- Need N sl-MappingToReleaseList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17 OPTIONAL, -- Need N ... } SL-MappingToAddMod-r17 ::= SEQUENCE { sl-RemoteUE-RB-Identity-r17 SL-RemoteUE-RB-Identity-r17, sl-EgressRLC-ChannelUu-r17 Uu-RelayRLC-ChannelID-r17 OPTIONAL, -- Cond L2RelayUE sl-EgressRLC-ChannelPC5-r17 SL-RLC-ChannelID-r17 OPTIONAL, -- Need N ..., [[ sl-EgressRLC-Channel-UL-r19 SL-RLC-ChannelID-r17 OPTIONAL, -- Need N sl-EgressRLC-Channel-DL-r19 SL-RLC-ChannelID-r17 OPTIONAL, -- Need N ]] } SL-RemoteUE-RB-Identity-r17 ::= CHOICE { srb-Identity-r17 INTEGER (0..3), drb-Identity-r17 DRB-Identity, ... } -- TAG-SL-SRAP-CONFIG-STOP -- ASN1STOP

## 38.351

Below TP is based on current SRAP running CR by removing sl-SRAP-ConfigRelay-ToAddModList.

## 5.3 UL Data transfer #### ***** #### 5.3.3.2 Egress RLC channel determination For a SRAP Data PDU to be transmitted, the SRAP entity shall: - If there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU: - If the SRAP Data PDU is for SRB0: - Determine the egress Uu/PC5 Relay RLC channel corresponding to sl-EgressRLC-ChannelUu or sl-EgressRLC-Channel-UL configured for SRB0 for the concerned sl-LocalIdentity as specified in TS 38.331 [3]. - Else if the SRAP Data PDU is received from SL-RLC1 as specified in TS 38.331 [3]: - Determine the egress Uu/PC5 Relay RLC channel corresponding to sl-EgressRLC-ChannelUu or sl-EgressRLC-Channel-UL configured for SRB1 for the concerned sl-LocalIdentity as specified in TS 38.331 [3]. - Else if there is an entry in sl-RemoteUE-ToAddModList which includes an sl-RemoteUE-RB-Identity matches SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 or sl-EgressRLC-Channel-UL which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1): - Determine the egress Uu/PC5 Relay RLC channel corresponding to sl-EgressRLC-ChannelUu or sl-EgressRLC-Channel-UL configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

## 5.2 DL Data transfer #### **** #### 5.2.2.2 Egress RLC channel determination For a SRAP Data PDU to be transmitted, the SRAP entity shall: - If the SRAP Data PDU is for SRB0 of the Child UE (for single-hop U2N Relay case, the BEARER ID field is 0 and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received; or for multi-hop U2N Relay case, the UE ID field matches the sl-LocalIdentity configured in sl-SRAP-ConfigRelay, BEARER ID field is 0, and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-Channel-UL which matches the LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received): - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3]. - Else if there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU, which includes an sl-RemoteUE-RB-Identity that matches the SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel or sl-EgressRLC-Channel-UL which matches the LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1): - If the SRAP Data PDU is for SRB1 but the corresponding sl-EgressRLC-ChannelPC5/sl-EgressRLC-Channel-DL in sl-SRAP-ConfigRelay is absent: - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3]. - Else: - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-EgressRLC-ChannelPC5 or sl-EgressRLC-Channel-DL configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].