3GPP TSG-RAN WG2 Meeting #131

Bengaluru, India, August 25-29, 2025

Agenda item:	8.1.2.3

Source: 	Qualcomm Incorporated (Rapporteur)

Title: 	Unofficial offline - LPP-21: "Associated ID" for TRP Location Coordinates

Per Chairnotes:

LPP-21: "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19) [Unofficial offline – lead by Qualcomm] R2-2505765 Remaining LPP open issues for "AI/ML Positioning Case 1" Qualcomm Incorporated discussion NR_AIML_air-Core Proposal 4a (LPP-21): The new IE for the implicit TRP location information (NR-TRP-LocationInfo-Implicit-r19) has the same structure as the existing IE NR-TRP-LocationInfo-r16, but with the location coordinate information being replaced by an "Associated ID" at each level (TRP, ARPs level). Proposal 4b (LPP-21): In all NR AI/ML assistance data IEs where Cell IDs (NCGIs, PCIs) can optionally be included for a TRP, the Cell IDs (NCGIs, PCIs) are always present in the IEs if the TRP is associated to a cell. Proposal 4c (LPP-21): If Proposals 4a/4b are agreeable, the "Associated ID" can be defined with 8-bits. Proposal 4d (LPP-21): Whether the IEs NR-TRP-LocationInfo-r16 and NR-TRP-LocationInfo-Implicit-r19 can be both provided together or not is left to implementation/deployment and does not need to be specified. Proposal 4e (LPP-21): If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16) a UE must not mandatorily support also implicit location info (NR-TRP-LocationInfo-Implicit-r19) (and vice versa). R2-2505908 RAN1 Agreements impacting RAN2 and Addressing Open Issues Ericsson discussion Rel-19 NR_AIML_air-Core Proposal 5: (LPP-21) Associate ID for PRS only TP is not considered in Rel-20. The scope of Associate ID (to what granular level the associate ID needs to be updated) is left to NW implementation. Associate ID value range is 0 to 255. Explicit and Implicit TRP coordinates are mutually inclusive. In terms of UE capability, the UE supporting case1 AI/ML must support implicit location info.

Per Open Issues List:

Open issue LPP-21 (new): "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19) - Must the "Associated ID" for the TRP coordinates be associated with a "cell" (e.g., NCGI)? If so, how should PRS-only TPs (which are not associated with a cell) being handled? And how to implement RAN1 agreement that UE does not expect to receive different values of associated ID for TRPs belonging to the same cell? - Is the "Associated ID" only for the TRP location coordinates, or also for the associated DL-PRS Resource Set/DL-PRS Resource ARPs? - Is the value range of 0..255 for the "Associated ID" sufficient? In particular if the "Associated ID" should be "configured per cell"? - Are the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) mutually exclusive, or can a deployment provide both? - If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16), must a UE support also implicit location info (NR-TRP-ImplicitLocationInfo-r19) (and vice versa)?

Currently, only the following RAN1 agreement from RAN1#121 is available:

Agreement For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits)

Per RAN1 agreement above, "an associated ID is configured per-cell (e.g., NCGI-r15)". For NR positioning, PRS-only TPs are also supported.  Since a PRS-only TP has no NCGI, the "associated ID" would need to be indicated for a DL-PRS ID (i.e., TRP ID). Ericsson proposes above that "Associate ID for PRS only TP is not considered in Rel-20" (I assume this should be Rel-19).

Question 1: Should the NR-TRP-LocationInfo-Implicit-r19 assistance data support PRS-only TPs?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |

Per RAN1 agreement above, the "implicit indication of Info #7 is via associated ID". Info #7 refers to the following row in Table 8.12.2.1.0-1 of 38.305:

Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs)

According to the above, "Info#7" includes "transmission reference location for each DL-PRS Resource ID".

Per Ericsson Proposal above, "The scope of Associate ID (to what granular level the associate ID needs to be updated) is left to NW implementation.". However, a UE need to be aware of the scope of the Associated ID in order to make proper use of the assistance data.

Question 2a: Do you agree that the "Associated ID" for indicating the implicit TRP location coordinates in IE NR-TRP-LocationInfo-Implicit-r19 is also indicating the implicit DL-PRS Resource Set/DL-PRS Resource ARP coordinates?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |

Question 2b:

If your answer to question 2a is "yes", should it be possible to (optionally) indicate separate "Associated IDs" for the TRP and ARP coordinates (Qualcomm Proposal above), or should there be a single "Associated ID" for all ARPs under each TRP?

| Company | Single ID/Separate ID | Comments (if any) |
| --- | --- | --- |

In all NR positioning assistance data, a TRP is identified by the DL-PRS ID. However, given that there is usually some time elapsed between training and inference phases, the DL-PRS ID only may be ambiguous. To resolve this potential ambiguity, a cell-ID can be provided in addition. There is the following common structure in all existing NR positioning assistance data elements:

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15					OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,

For example, in the assistance data used for training, a particular value for the DL-PRS ID may be the same as in the assistance data for inference but may indicate different physical TRPs. A DL-PRS ID would be unambiguous only within the same LPP session, but there is no guarantee that a DL-PRS ID is unambiguous across different LPP sessions (and across different LMFs).

Therefore, Qualcomm proposed above that for DL AI/ML positioning, whenever assistance data are provided, a LMF must always include the Cell-IDs in all NR assistance data elements when the TRP is associated to a cell (in LPP, this could for example be implemented via a NOTE in NR-DL-AIML-ProvideAssistanceData).

Question 3a: Do you agree that for DL AI/ML positioning, when NR AI/ML assistance data are provided, the Cell IDs (NCGIs, PCIs) must always be included in the assistance data IEs when the TRP is associated to a cell?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |

Question 3b: Is the value range of 0..255 for the "Associated ID" considered being sufficient?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |

Currently, all assistance data for all positioning methods are optional (i.e., for each defined assistance data element there is also a separate UE capability indication). Ericsson proposes above that a UE supporting Case 1 must also support the NR-TRP-LocationInfo-Implicit-r19 assistance data.

Question 4a: Must the support for NR-TRP-LocationInfo-Implicit-r19 assistance data be mandatory for a UE supporting Case 1?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |

Question 4b: If a UE supports both, explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-LocationInfo-Implicit-r19) (independent of Question 4a) can a LMF provide both to a UE (when supported by an LMF and e.g., requested by a UE)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |