3GPP TSG-RAN WG3 Meeting #129	R3-255209

Bangalore, India, 25th – 29th May 2025

Agenda Item:		17.2

Source:		NEC

Title:			(TP to BLCR TS38.473) Support on-demand SSB

Document for:		Discussion and decision

- Introduction

In previous RAN3 meetings, we discussed whether NES gNB-CU can indicate NES gNB-DU the allowed cell list for on-demand SSB. Although majorities prefer the following but no agreement can be achieved:

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may take it into account to enable the on-demand SSB SCell operation for the indicated cells when they are operating as SCells.

Besides, there is still one open issue on Xn impact due to on-demand SSB:

- Normal cell to request NES cell to provide on-demand SSB over Xn for layer3 measurement?

In this contribution, we share our views on the above issues and analyse potential RAN3 impacts.

- Discussion

  - Cells allowed to be OD-SSB list from gNB-CU to gNB-DU

Considering gNB-CU is more knowledgeable on the SSB operation status (e.g. no always-on SSB, always-on SSB with a long periodicity or always-on SSB as in legacy) of the cells with all DUs connected to it and neighbour cells, gNB-CU can provide some guidance to gNB-DU for OD-SSB SCell operation, taking both energy saving and UE performance into account. For example, gNB-CU can suggest to gNB-DU e.g. OD-SSB allowed cells and, for each SCell, SSB operation mode, OD-SSB transmission periodicity, OD-SSB transmission duration, etc.

In legacy, gNB-CU can provide gNB-DU a Cells Allowed to be Deactivated List IE, and upon receiving this IE, gNB-DU shall consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy optimization purpose.

For OD-SSB, as defined in RAN1, there are two cases:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

A cell operating in Case #1 is similar as a cell with SSB beam deactivated, therefore the same logic as the existing cell deactivation in gNB-DU can be followed, i.e. gNB-CU can provide gNB-DU a Cells Allowed to be OD-SSB List. This newly proposed Cells Allowed to be OD-SSB List is an optional IE and it can be provided by gNB-CU to gNB-DU as assistance information. Once receiving this, gNB-DU can take it into account and still decide by itself whether to perform OD-SSB.

Proposal 1: gNB-CU can provide gNB-DU a Cells Allowed to be OD-SSB List to assist the OD-SSB operation in gNB-DU.

Regarding how to provide the assistance information from gNB-CU to gNB-DU, there are two options:

Option 1: Reuse Cells Allowed to be Deactivated List IE

In Rel-18 NES, a Cells Allowed to be Deactivated List IE is added in GNB-CU CONFIGURATION UPDATE message, and “If Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy optimization purpose”[1]. Some companies proposed that this IE can be reused for OD-SSB and, after receiving this, gNB-DU can decide whether to perform cell deactivation or OD-SSB activation. The procedure description of this IE needs to be updated to support OD-DDB case. Below is the detailed changes in TS38.473.

### 8.2.5	gNB-CU Configuration Update

<<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells or it is allowed to activate OD-SSB within the indicated SCells for network energy saving purpose.

Option 2: Add a new IE e.g. Cells Allowed to be OD-SSB List IE

The existing Cells Allowed to be Deactivated List IE is added as an assistance information for beam deactivation in a cell owned by gNB-DU, which is used for a different purpose than OD-SSB operation. In order not to impact the Rel-18 beam deactivation operation, it is more advantageous to use a separate IE, e.g. Cells Allowed to be OD-SSB List IE, to carry the assistance information for OD-SSB operation. This new IE can be added in the existing GNB-CU CONFIGURATION UPDATE message. Below is the detailed changes in TS38.473.

### 8.2.5	gNB-CU Configuration Update

<<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the SCells Allowed to be OD-SSB List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to activate OD-SSB within the indicated scells for network energy saving purpose.

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| <<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| SCells Allowed to be OD-SSB List |  | 0..1 |  |  | YES | ignore |
| >SCells Allowed to be OD-SSB List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

Comparing the two options, option 2 has no impact on legacy SSB beam deactivation and provide more accurate guidance to gNB-DU on whether the Cells Allowed List is for deactivation or OD-SSB. For example, some cells in a gNB-DU may allowed to be deactivated, while some other cells in the same gNB-DU may be better to only activate OD-SSB

However considering the standard impact, as a compromise, option 1 is also acceptable as a simplified solution.

Proposal 2: Reuse the existing Cells Allowed to be Deactivated List IE but update the relevant procedure texts as OD-SSB assistance information from gNB-CU to gNB-DU.

TP to BLCR TS38.473, mirroring proposal 2, is available in Annex.

Proposal 3: Agree the TP to BLCR TS38.473 in Annex.

  - Xn impacts

Considering the scenario needs to be analyzed is for UEs in connected mode configured with CA, for both intra-/inter-band CA [1], in which a PCell is always associated to a CD-SSB located on the synchronization raster, we do not see the need to use a neighbour gNB to tiger OD-SSB SCell operation and we think this should be an intra-gNB issue.

One issue was brought up about how to address L3 measurement of an OD-SSB Cell, whether NES cell can be triggered by a neighbor cell over Xn for OD-SSB activation? According to the existing RAN1 agreement, On-demand SSB on the cell is not located on synchronization raster; On-demand SSB on the cell is non-cell-defining SSB. And RAN2’s assumption is that UE does not expect to measure SSB when OD-SSB transmission is deactivated if no always-on SSB on the cell. Besides, RAN2 reached a new WA that “When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB”.

Therefore, from the analysis above, no Xn impact is foreseen for OD-SSB SCell operation and this open issue can be closed.

Proposal 4: No Xn impact for on-demand SSB SCell operation.

- Proposals

In this contribution, we summarized the latest RAN1 and RAN2 progress on OD-SSB and analysed the potential RAN3 impacts, and the following proposals were drawn from the analysis.

Proposal 1: gNB-CU can provide gNB-DU a Cells Allowed to be OD-SSB List to assist the OD-SSB operation in gNB-DU.

Proposal 2: Reuse the existing Cells Allowed to be Deactivated List IE but update the relevant procedure texts as OD-SSB assistance information from gNB-CU to gNB-DU.

Proposal 3: Agree the TP to BLCR TS38.473 in Annex.

Proposal 4: No Xn impact for on-demand SSB SCell operation.

- Reference

[1] TS38.473, NG-RAN; F1 application protocol (F1AP)

Annex TP to BLCR TS38.473

<Start of change>

### 8.2.5	gNB-CU Configuration Update

<<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells or it is allowed to activate OD-SSB within the indicated SCells for network energy saving purpose.

<End of change>