3GPP TSG-RAN WG3 Meeting #129                                                                  R3-255248

Bengaluru, India, August 25th – 29th, 2025

Source:	CATT

Title:	(TP to BLCR for TS 38.300) On remaining issues of OD-SIB1

Agenda Item:	17.3

Document for:	Discussion and Approval

1 Introduction

During RAN3#126 meetings[1], following agreements were achieved for on-demand SIB1:

RAN3#126 The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. Cell A stores the UL WUS configuration information after it has received it. A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

In the RAN3#127 meeting[2], agreements were made as below:

RAN3#127 It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

In the RAN3#127bis meeting[3], following agreements were achieved:

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that: - One Choice “Start with UL WUS Condiguration” - One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

In the last meeting[4], agreements on OD-SIB1 were made as below:

One “Provision Request message includes one “OD-SIB1 config R19” referring to the TS 38.331 definition, it is a RRC Container in octet string (presence M) + one NES Cell ID (presence M ) + one Cell-A ID (presence O ) Cell A gNB-CU encoding the SIBxx. The NES gNB-CU sends the indication to NES gNB-DU. The NES gNB-DU MAY go to OD-SIB 1 operation up to gNB-DU decision.

In this contribution, we will continue to discuss the remaining issues on OD-SIB1.

2 Discussion

2.1 XnAP signalling

In the last meeting[4], it has been agreed that the Provision Request message with codepoint “start” includes one UL WUS configuration corresponding to one NES cell and the NES cell ID. And this message can also optionally include one Cell A ID. If the Cell A ID is not present, the Cell A gNB can understand the UL WUS configuration being broadcasted in default Cell A(s) is requested (e.g., the default Cell A can be determined by OAM). If the Cell A ID is present, the UL WUS configuration is broadcasted in the indicated Cell A. The BLCR for XnAP[8] was updated based on the agreements made up to now.

In different scenarios, multiple UL WUS configurations may need to be broadcast by the same gNB, or one UL WUS configuration may need to be delivered in multiple cells of the same gNB. Thus, two types of request message (i.e., w or w/o Cell A ID) may be sent over Xn interface at the same instance, and multiple Provision Request messages with different Cell A IDs may also be sent. Then multiple Provision Response/Reject messages will be sent back from at the Xn instance respectively. For example, if one UL WUS configuration needs to be broadcast not only in Cell ID 1 (default Cell A) but also in Cell ID 2 under the same peer gNB, one Provision Request message without Cell A ID and another Provision Request message with Cell ID 2 are sent to the peer gNB. Therefore, discrimination for the received Provision Response/Reject messages to which different Provision Request messages correspond at the same Xn instance is necessary.

However, it’s not clear how to differentiate different Provision Response/Reject messages at the same Xn instance.

Observation: It’s not clear how to discriminate the received Provision Response/Reject messages to which different Provision Request messages correspond at the same Xn instance.

Several options are possible:

Option 1: Use NES Cell ID

NES cell ID should be included in the Provision Response/Reject messages for discrimination. This option restricts that only one ongoing UL WUS configuration provision procedure related to a certain NES cell is allowed at the same Xn instance, which means the Provision procedures for one NES cell should be carried out one by one towards the peer gNB at the same instance. Nevertheless, the NES cell ID IE is already captured in the XnAP BLCR.

This option requires no change to the messages in XnAP BLCR.

Option 2: Use Request ID

Include a Request ID into the Provision Request to identify a specific procedure. The corresponding Response/Reject message should include the same Request ID. So, a mandatory Request ID IE is added to the Provision Request/Response/Reject message, and the NES Cell ID in Provision Response/Reject message in BLCR can be removed.

This option supports more than one ongoing provision procedure at the same Xn instance for one NES cell, namely, multiple Provision Requests to the same peer gNB can be sent in parallel.

Option 3: Use {NES Cell ID, Cell A ID} pair

If a Cell A ID is included in the Provision Request message, the corresponding Response/Reject message should include the same Cell A ID. However, if the Provision Request message has no Cell A ID included, the Response/Reject message should not include Cell A ID. Thus, an optional IE of Cell A ID can be introduced to the Provision Response/Reject message.

This option also supports more than one ongoing procedure for one NES cell at the same Xn instance.

Proposal 1: RAN3 to discuss how to differentiate different Provision Response/Reject messages, and down-select from following options:

- Option 1: Use NES Cell ID

- Option 2: Use Request ID

- Option 3: Use {NES Cell ID, Cell A ID} pair

Which RRC IE is contained in the UL WUS configuration(naming FFS) IE in the Provision Request message is FFS now. RAN2#129bis meeting[6] agreed that one WUS configuration can be associated with a list of cells:

WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

According to the latest RRC Running CR[7], each OD-SIB1 configuration is delivered using the OD-SIB1-Config-r19 IE associated with a {ARFCN, PCI list} pair in SIBxx.

#### – SIBxx SIBxx contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells. SIBxx information element -- ASN1START -- TAG-SIBxx-START SIBxx-r19 ::= SEQUENCE { od-SIB1-ConfigList-r19 SEQUENCE (SIZE(1..maxNrofODSIB1-r19)) OF OD-SIB1-Config-r19 lateNonCriticalExtension OCTET STRING OPTIONAL, ... } OD-SIB1-Config-r19 ::= SEQUENCE { carrierFreq-r19 ARFCN-ValueNR, physCellIdList-r19 SEQUENCE (SIZE(1..maxPCI-r19)) OF PhysCellId, sib1-rsrp-ThresholdSSB-r19 RSRP-Range OPTIONAL, -- Need R prach-RootSequenceIndex-r19 CHOICE { l839 INTEGER (0..837), l139 INTEGER (0..137) } OPTIONAL, -- Need R msg1-SubcarrierSpacing-r19 ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2} OPTIONAL, -- Need R sib1-tdd-UL-DL-ConfigurationCommon-r19 TDD-UL-DL-ConfigCommon OPTIONAL, -- Cond TDD … k-ssb-r19 INTEGER (1..23), sib1-RequestConfig-r19 SIB1-RequestConfig-r19 } … -- TAG-SIBxx-STOP -- ASN1STOP

So, the UL WUS configuration (naming FFS) IE in the request message can contain the OD-SIB1-Config-r19 defined in RRC spec. At reception of the UL WUS configurations for multiple NES cells, the gNB would only broadcast one copy of configuration for these cells if they are all contained in the physCellIdList-r19 of OD-SIB1-Config-r19.

Proposal 2: The UL WUS configuration IE contains the OD-SIB1-Config-r19 in SIBxx-r19 IE defined by RRC spec.

2.2 Terminology alignment

RAN2 has decided to avoid using NES, Cell A, UL-WUS and UL WUS configuration in stage-2 and stage-3 specs from RAN2#129bis meeting[6]. Instead, the terminology SIB1 request and OD-SIB1 configuration are utilized.

Define features, not devices. That is, definition of OD-SIB1, OD-SSB, paging adaptation etc in 38.300 and in other specifications refer to 38.300: - Avoid NES, Cell A, UL-WUS - “UE supporting OD-SIB1” where/if needed, and to replace with the UE capability name - UL-WUS is replaced with SIB1 request in e.g. field descriptions - ul-WUS-Config renamed as od-sib1-Config

However, the terminologies NES, Cell A, UL-WUS and UL WUS configuration are still being used in RAN3 spec BLCRs. The terminologies used in RAN3 spec BLCRs should be updated to align with RAN2 specs.

Proposal 3: The terminologies used in RAN3 spec BLCRs should be updated to align with RAN2.

A TP to TS 38.300 BLCR is provided in Annex for terminology alignment and capturing the agreements in the last meeting.

3 Conclusion

In the previous sections we made the following observations and proposals:

Observation: It’s not clear how to discriminate the received Provision Response/Reject messages to which different Provision Request messages correspond at the same Xn instance.

Proposal 1: RAN3 to discuss how to differentiate different Provision Response/Reject messages, and down-select from following options:

- Option 1: Use NES Cell ID

- Option 2: Use Request ID

- Option 3: Use {NES Cell ID, Cell A ID} pair

Proposal 2: The UL WUS configuration IE contains the OD-SIB1-Config-r19 in SIBxx-r19 IE defined by RRC spec.

Proposal 3: The terminologies used in RAN3 spec BLCRs should be updated to align with RAN2.

4 Reference

- RAN3#126 Minutes, RAN3 chair

- RAN3#127 Minutes, RAN3 chair

- RAN3#127bis Minutes, RAN3 chair

- RAN3#127bis Minutes, RAN3 chair

- RAN3#128 Minutes, RAN3 chair

- Draft Report of 3GPP TSG RAN2 meeting #129bis, MCC

- R2-250xxxx, Running RRC CR NES Post130, Ericsson, RAN2#131

- R3-253974, Introduction of Network Energy Saving Enhancement, Ericsson, RAN3#128

5 Annex (TP to BLCR for TS 38.300)

-------------------------------------Start of TP---------------------------------------------

#### 15.4.2.x2	On-demand SIB1

To facilitate reducing gNB downlink transmissions, instead of always periodically transmitting SIB1, the gNB can provide on-demand SIB1, i.e., upon receiving an OD-SIB1 request from a UE supporting OD-SIB1. OD-SIB1 is supported for UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED when T311 is running. A request for SIB1 triggers a random access procedure, where MSG1 is used for indicating OD-SIB1 request and the gNB acknowledges the request in MSG2. OD-SIB1 request configurations of one or more cells which support OD-SIB1 are included in SIBxx, which can be broadcasted in any cell, including cell’s own OD-SIB1 request configuration. UE may request SIB1 based on the OD-SIB1 request configuration from SIBxx in order to determine the suitability of a cell during and after cell reselection.

A gNB may request over the Xn interface neighbour gNB(s) to broadcast the OD-SIB1 request configuration for the cell supporting OD-SIB1. The request may include identity of the cell to broadcast the OD-SIB1 request configuration. If the request is accepted, the gNB(s) receiving the OD-SIB1 request configuration should start the broadcast via SIBxx[FFS]. The gNB can also inform the gNB broadcasting the OD-SIB1 request configuration to stop the broadcast. If a gNB broadcasting the OD-SIB1 request configuration decides to stop broadcasting, it informs the gNB which has requested the broadcast over the Xn interface.

Editor’s Note: This paragraph may need to be refined in future.

-------------------------------------End of TP---------------------------------------------