3GPP TSG-RAN WG3 Meeting #129                                                              R3-255252

Bengaluru, India, 25 – 29 August 2025

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	Remaining issues for WAB

Document for:		Discussion

# 1	Introduction

This contribution discusses remaining issues for WAB.

# 2	Discussion

## 2.1	Obtaining TNL addresses of BH gNBs to be used for Xn

In the last meeting, RAN3 identified the following open issue [1]:

The BH-gNB can provide the TNL information of neighbour gNBs to the WAB node.

This issue only applies when access to WAB-gNB and BH use the same PLMN.

The WAB-gNB can obtain the TNL address of the co-located WAB-MT’s serving BH gNB via legacy TNL resolution via AMF. The question arises whether it needs to obtain the TNL addresses of other BH gNBs. Having these TNL addresses would allow the WAB-gNB to establish multiple Xn connection with surrounding gNBs. It could use these Xn connections, i.e., for Xn handover of UEs.

The following options can be considered:

Option 1: WAB-gNB only establishes Xn with the co-located WAB-MT’s serving BH gNB.

Option 2: WAB-gNB is OAM-configured with a mapping between TNL address and serving cell ID of BH gNBs.

Option 3: WAB-gNB uses mapping between TNL address and serving cell ID of BH gNBs stored from prior BH connections.

Option 4: WAB-gNB resolves TNL address of candidate served-cell based on WAB-MT’s report of candidate served-cell CGI.

Option 5: WAB-gNB obtains from its co-located WAB-MT’s BH gNB the TNL addresses of neighbor gNBs via Xn.

Options 1 to 4 do not require any signaling enhancements. For Option 5, the Neighbour NG-RAN Node IE in the Xn SETUP REQUEST/RESPONSE messages and the NG-RAN NODE CONFIGURATION UPDATE (ACK) messages needs to be enhanced with the NG-RAN-node’s TNL address.

Proposal 1: RAN3 to discuss whether to enhance Neighbour NG-RAN Node IE with TNL address in the Xn SETUP REQUEST/RESPONSE messages and the NG-RAN NODE CONFIGURATION UPDATE (ACK) messages for support of Xn establishment.

## 2.2	Issues for non-terrestrial link on WAB-gNB’s backhaul

In the last meeting, the following open issues were captured  in the Chair notes [1]:

If non-terrestrial link is used between WAB MT and BH gNB and/or between BH gNB and BH CN, the WAB-gNB informs UE’s CN that the BH includes a non-terrestrial link. FFS how a WAB node know the BH-gNB is using a non-terrestrial link. Possible options include BH-gNB informs WAB-gNB via Xn

There are two aspects that need to be addressed in this context:

Issue 1: The UE wants to take advantage of EAS or inter-UE communications via a UPF on a GEO satellite.

Issue 2: The backhaul link via satellite may have large latency, which adversely affects the CN part of the PDB.

### 2.2.1 UPF onboard of GEO satellite for EAS or inter-UE communications

TS 23.501 clause 5.43 defines the scenario, where the gNB uses GEO satellite backhaul and the satellite supports Edge Computing services or local inter-UE communications via an onboard UPF. This clause also describes the handling of UE mobility between such a gNB with GEO satellite backhaul and a gNB with TN backhaul.

In the last meeting, RAN3 discussed whether and how this scenario applied to WAB. The following two scenarios can be considered in the context of WAB:

- Scenario 1: The WAB-MT moves between NTN and TN, where the NTN satellite has onboard UPF for EAS.

- Scenario 2: The WAB-MT moves between two RAN-nodes, where one RAN-node uses satellite backhaul with onboard UPF for EAS, while the other RAN-node uses BH via TN.

Figure 1a-c below illustrate baseline scenario, scenario 1 and scenario 2.

Figure 1: Scenarios for backhauling via satellite with onboard UPF: a) baseline from TS 23.501 c. 5.43, b) WAB-MT using NTN, c) WAB-MT’s serving gNB using satellite-based backhaul.

#### Baseline: UE moves between gNB1 with satellite BH using onboard UPF for EAS and gNB2 with TN BH.

According to TS 23.501 clause 5.43, the UE’s CN needs to be aware whether the UE connects via the gNB using satellite BH with onboard UPF (gNB1) or via the gNB using TN (gNB2). In this manner, the UE’s CN can ensure that the UE is configured with the proper URSP to take advantage of EAS when it connects to gNB1. According to TS 23.501, the UE’s AMF needs to be locally configured with the gNB IDs for those gNBs that satellite BH with onboard UPF.

#### Scenario 1: The WAB-MT moves between NTN gNB1 with UPF onboard of satellite for EAS and TN gNB2.

The UE can only take advantage of EAS provided via UPF onboard of the satellite, if the MT’s UPF used for backhauling and the MT’s gNB (gNB1) are also onboard of the satellite. The MT’s AMF can establish the BH PDU session with the onboard UPF in the same manner as described by TS 23.501 for the baseline scenario. Placing the MT’s UPF for backhauling on the GEO satellite is an unrealistic scenario. It further raises the question of whether this onboard UPF is also used for the WAB-gNB’s NG-C traffic whose destination, the UE’s AMF, is not located on the satellite.

Observation 1: When WAB-MT uses NTN with EAS onboard of the satellite, the MT’s gNB, the MT’s UPF used for backhauling and the UE’s UPF need to reside on the satellite.

#### Scenario 2: The WAB-MT moves between gNB1 with satellite BH using onboard UPF for EAS and gNB2 with TN BH.

The UE can only take advantage of EAS provided via UPF onboard of the satellite, if the MT’s UPF used for backhauling is also onboard of the satellite. Scenario 2 therefore has the same implications as Scenario 1.

Observation 2: When WAB-MT connects to a gNB with BH via satellite providing onboard EAS, the MT’s UPF used for backhauling and the UE’s UPF need to reside onboard the satellite.

Based on this analysis, Scenario 1 and 2 should not be further pursued.

Proposal 2: EAS and inter-UE communications using UPF onboard of satellite are not supported for UEs that connect via WAB-gNB.

### 2.2.2 Impact of high latency from satellite-based backhaul

The use of a non-terrestrial link on the backhaul may introduce significant latency to the backhaul transport. This applies to scenarios, where the WAB-MT uses NTN or where the BH gNB uses a non-terrestrial link for its own backhauling.

The backhaul latency may further become very time-dependent under the following circumstances:

- The WAB-MT moves between TN and NTN.

- The WAB-MT moves between BH gNB with non-terrestrial BH and BH gNB with terrestrial BH.

It would be beneficial for the UE’s CN to know the backhaul latency so that it can assess whether a request for a specific 5QI can be accepted or not.

The following options can be considered:

Option 1: The UE’s CN and WAB-gNB perform QoS monitoring of packet delay as defined by TS 23.501 cl. 5.33.3. This approach is rather reactive rather than proactive.

Option 2: The UE’s CN is configured with the mapping between the MT’s serving cell’s TAC and the NTN type. This approach works well for NTN cells since they are expected to have different TAC than TN cells. It is likely to also apply to typical deployments of gNBs using satellite links for backhauling.

Option 3: The WAB-gNB informs the UE’s AMF about the satellite type used by the co-located WAB-MT (if any). This approach requires signaling enhancements and it is only applicable to NTN backhauling, but not to scenarios where the BH gNB uses satellite-based backhauling.

Option 1 and Option 2 can handle both scenarios, i.e., where the WAB-MT uses NTN and where the BH gNB uses a non-terrestrial link on its own backhaul. Both options further can be facilitated via implementation and do not require signaling enhancements.

Option 3 requires signaling enhancements and it does not work for scenarios, where the BH gNB uses satellite-based backhauling.

For these reasons, there is not reason to further pursue option 3.

Observation 3: The UE’s AMF can derive the backhaul latency from a non-terrestrial link via legacy QoS monitoring of packet delay defined by TS 23.501 cl. 5.33.3.

Observation 4: The UE’s AMF can be configured with a mapping between the TAC of the MT’s served cell and the backhaul satellite type.

Proposal 3: The UE’s AMF can use legacy- and implementation-based mechanisms to determine backhaul latency due to a non-terrestrial link. There is no need to introduce new signaling.

## 2.3	Additional ULI in UE-associated NG messages

RAN3#127 achieved the following agreements:

Include Additional ULI into the User Location Information IE in TS 38.413. Additional ULI contains a CGI and a TAI. In case the additional ULI has changed e.g. due to WAB-node movement, the WAB-gNB derives the new additional ULI and it reports it to the network, if required by the CN via legacy procedures. Add this description into TS38.401. In case of WAB-MT connects via NTN, the Additional ULI is determined based on WAB-node geo-location. The latter applies to intra PLMN and inter PLMN cases.

RAN3#127bis achieved the following agreement:

RAN3 to agree the WAB-gNB reports the additional ULI to the network within the NG Setup and RAN Configuration Update procedures.

The question arises whether it is necessary to keep the additional ULI in the UE-associated NG procedures if it is already included in the non-UE associated procedures. RAN3 should discuss this matter.

Proposal 4: RAN to discuss whether the additional ULI to be kept in the UE-associated NG procedures.

## 2.4	Update to stage-2

Based on the above proposals, the BL CR to TS 38.401 needs to be updated as shown in the Annex.

Further, TS 38.300 should introduce a very short section on WAB as provided in a separate contribution.

Proposal 5: Agree BL CR to TS 38.300 in R3-255254.

## 2.5	Closing of WI

For WAB, all objectives defined in the WID [2] have been addressed except for resource coordination. In last meeting, RAN3 has not achieved consensus on resource coordination. A separate contribution (R3-255253) proposes to deprioritize resource coordination of WAB for this release. With this, the WI can be closed with respect to WAB.

Proposal 6: From RAN3 perspective, the WI can be closed with respect to WAB.

# 3	Conclusion

This contribution discussed remaining issues related to WAB. The following observations and proposals have been made:

On obtaining TNL addresses of BH gNBs to be used for Xn:

Proposal 1: RAN3 to discuss whether to enhance Neighbour NG-RAN Node IE with TNL address in the Xn SETUP REQUEST/RESPONSE messages and the NG-RAN NODE CONFIGURATION UPDATE (ACK) messages for support of Xn establishment.

On issues for non-terrestrial link on WAB-gNB’s backhaul:

Observation 1: When WAB-MT uses NTN with EAS onboard of the satellite, the MT’s gNB, the MT’s UPF used for backhauling and the UE’s UPF need to reside on the satellite.

Observation 2: When WAB-MT connects to a gNB with BH via satellite providing onboard EAS, the MT’s UPF used for backhauling and the UE’s UPF need to reside onboard the satellite.

Proposal 2: EAS and inter-UE communications using UPF onboard of satellite are not supported for UEs that connect via WAB-gNB.

Observation 3: The UE’s AMF can derive the backhaul latency from a non-terrestrial link via legacy QoS monitoring of packet delay defined by TS 23.501 cl. 5.33.3.

Observation 4: The UE’s AMF can be configured with a mapping between the TAC of the MT’s served cell and the backhaul satellite type.

Proposal 3: The UE’s AMF can use legacy- and implementation-based mechanisms to determine backhaul latency due to a non-terrestrial link. There is no need to introduce new signaling.

On additional ULI in UE-associated NG messages

Proposal 4: RAN to discuss whether the additional ULI to be kept in the UE-associated NG procedures.

On update to stage-2

Proposal 5: Agree BL CR to TS 38.300 in R3-255254.

On closing the WI

Proposal 6: From RAN3 perspective, the WI can be closed with respect to WAB.

# 4	References

[1]  Chair notes, 3GPP TSG RAN WG3 Meeting #128, Wuhan China, April 2025

[2]  RP-242395, New WID on additional topological enhancements for NR, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 2024