3GPP TSG RAN3 meeting #129   	         	                                R3-25xxxx

25 – 29 August 2025   Bengaluru, India

Agenda item: 	    10.2

Source: 	China Unicom

Title: 	Discussion on MRO enhancements

Document for:	Discussion

1	Introduction

At RAN#102, the new Work Item on Data Collection for SON_MDT in NR standalone and MR-DC phase 4 was agreed [1] which introduced the following objectives in the core part:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure If needed, co-operate with RAN1, SA2, SA5, CT4.

This contribution provides our views on the scenarios of MRO enhancements which can be supported in Rel-19.

2	Discussion

2.1	LTM

UHI and ping-pong issue CU informs relevant information to the target DU via CU-DU Cell Switch Notification. Down selection of following solutions: 1a: CU provide full UHI list with L1/L3 type to the DU (maximum item number is 16). 1b: CU provide full UHI list (except PScell) with L1/L3 type to the DU (maximum item number is 16). 2a: CU provide last consecutive L1 entries to the DU. 2b: CU provide FFS number of last consecutive L1 entries to the DU. No stage2, work in stage3 with 1a. If the above 1a information is included in F1AP message from CU to DU, the DU may consider the potential mobility issue detected including ping-pong. FFS on the extra indication for ping-pong case.

At the RAN3#128 meeting, we have been made above agreements and discussed LTM ping-pong avoidance issue with potential solutions to be down selected.

As CU is the node to detect the ping pong issue, DU should be provided with only useful information to analyse the issue. L3 info should be included in the UHI list when L1-L3-L1 combined handover occurs.

PScell information can be removed irrelevant to DU. DU may be provided the UHI list when CU detects the ping-pong issue, then DU can perform the corresponding analysis.

Regarding other information provided to DU for other mobility event, the benefit should be further clarified.

Proposal 1: CU provide full UHI list (except PScell) with L1/L3 type to the DU when ping-pong issue detected.

BFR shortly after successful LTM cell switch to the wrong beam (Case 1):

Use DU-CU ACCESS AND MOBILITY INDICATION message for target DU send recovery beam information to the CU. Only one recovery beam information included.

Additional Information with recovery beam information from target DU to CU:

- CU UE F1AP ID

- Failure type (for case 1, case 2) in an implicit way or explicit way

- beam failure indicator in an implicit way or explicit way

Use ACCESS AND MOBILITY INDICATION message for CU forward recovery beam information to the source DU. Only one recovery beam information included.

Additional Information with recovery beam information from target CU to DU:

- Source DU UE F1AP ID

- Failure type (for case 1, case 2) in an implicit way or explicit way

- beam failure indicator in an implicit way or explicit way

No new failure type for beam failure recovery is needed for TS38.300.

The Timer configured in DU (e.g., Tstore_UE_cntxt) used for the DU to detect BFR shortly after successful LTM cell switch to the wrong beam is up to implementation.

Regarding the outdated TA issue, target DU can send the TA information used to CU and CU forwards it to source DU, then source DU can do analyse with the used TA information comparing its own configuration to identify the outdated TA issue.

Proposal 2: Target DU can send the TA information used to CU and CU forwards it to source DU.

3	Conclusion

Proposal 1: CU provide full UHI list (except PScell) with L1/L3 type to the DU when ping-pong issue detected.

Proposal 2: Target DU can send the TA information used to CU and CU forwards it to source DU.

4   References

- RP-234038: New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4; TSG RAN Meeting #102, Edinburgh, Scotland, Dec 11-15, 2 023