3GPP TSG-RAN WG3 Meeting #129				                                       R3-255372

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	20.2

Source:	Huawei

Title:	(TPs for AI/ML BLCR to TS 38.300/305/401/455/473) Remaining open issues for AI/ML-based positioning Case 3a and Case 3b

Document for:	Discussion and decision

# Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” – latest WID in [1]. To be specific, the discussion was on the specification support for AI/ML-based positioning accuracy enhancements.

This contribution focuses on remaining open issues for AI/ML assisted positioning Case 3a and Direct AI/ML positioning Case 3b, and provides proposals based on the latest progress in RAN3.

# Discussion

## 2.1 Case 3a

In the following we further discuss aspects related to AI/ML positioning Case 3a (i.e., NG-RAN node-assisted positioning with a gNB-side model), and below we recall the agreements and open issues as captured in the meeting minutes [2]:

Define the Positioning Data Collection Needed IE as a new bitmap. The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with optional LoS/NLoS indicator. FFS if UE location can be requested. The Positioning Data Collection Needed IE is introduced in the MEASUREMENT REPORT message. Introduce POSITIONING DATA COLLECTION REPORT message as class 2 procedure including Positioning Data Information IE and RAN/LMF Measurement IDs Introduce an optional Positioning Data Unavailable IE (FFS on encoding) when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. The codepoints indicating the reason. RAN3 confirms that Performance monitoring is done in the RAN in case 3a. RAN3 confirms the WA: For case 3a data collection, Part A is known internally by the gNB and is not signalled over any network interface. Introduce the new flag “Inferred measurement” in the TRP Measurement Result to align with RAN1 conclusion FFS: if LMF should specifically request for AIML inferred measurement.

Moreover, RAN3 agreed a bunch of TPs for the BL CRs towards affected specifications, i.e., TS 38.455, TS 38.413, TS 38.300, TS 38.401 and TS 38.305, some of which include open aspects that need to solved.

In order to address the open aspects for supporting AI/ML positioning Case 3a we will discuss model training and model inference remaining aspects separately.

### 2.1.1 Model training considerations for Case 3a

The latest version of the BLCR to TS 38.305 [3] captures, in clause 7.x, the signalling flow of the Positioning Data Collection procedure that can be used to collect data needed for training and monitoring of AI/ML models hosted in the gNB. Such procedure is also known as “opportunistic” data collection, where UEs being already positioned are exploited to gather data at the gNB side. RAN3 also discussed in past meetings another approach which is termed as “proactive” data collection at the gNB, where a positioning session may be triggered by the gNB for a specific UE to gather data associated to that UE. Due to potential SA2 impacts, RAN3 agreed in RAN3#127bis [4] to send the LS to SA2 in [5] asking for feedback on both solutions (“opportunistic” and “proactive” data collection, indicated as “Solution 1” and “Solution 2”, respectively, in the LS to SA2) for data collection for Case 3a, and whether the solution for the “proactive” data collection is feasible within the Rel-19 timeframe.

SA2 provided a reply LS to RAN3 in [6] with the following statements:

1. Overall Description: SA2 thanks RAN3 for their LS. SA2 would like to answer the questions raised by RAN3 as follows: *** skip unmodified text *** SA2 Answer: For solution 1 it may be desirable to document related LMF procedures in TS 23.273. There are SA2 impacts to support the solution but SA2 could address those impacts as alignment with RAN in Rel-19. For solution 2, SA2 concludes that this solution is not feasible in current frozen Rel-19. SA2 is currently discussing for data collection for training models to be used by the LMF for positioning (RAN case 3b) whether a user consent check and/or a LCS privacy profile check is required. SA2 is anticipating that similar checks could also be required for case 3a (both for solution 1 and solution 2). 2. Actions: To RAN3 group. ACTION: SA2 kindly asks RAN3 to take the provided answers into account and update SA2 on the solutions selected by RAN3.

Based on the above excerpt from [6], it is our understanding that only Solution 1, that is, the “opportunistic” data collection can be supported in Rel-19. Therefore we propose to align with the SA2 feedback pertaining to the Solution 2 (i.e., the “proactive” data collection).

Proposal 1: RAN3 to agree that the “proactive” data collection approach is not pursued in Rel-19.

Still concerning the Positioning Data Collection procedure as described in the BLCR to TS 38.305 [3], a new Positioning Data Collection Needed IE has been introduced in the existing NRPPa MEASUREMENT RESPONSE message (gNB  LMF) in order for the gNB needing data to train/monitor the AI/ML model to indicate to the LMF that, for the UE being positioned, data collection is needed. The same IE has also been introduced in the NRPPa MEASUREMENT REPORT message (gNB  LMF) for the same purpose, as captured in the latest version of the BLCR to NRPPa [7]. The encoding of the Positioning Data Collection Needed IE in the BLCR to NRPPa is as follows [7]:

### 9.2.Z1 Positioning Data Collection Needed This information element contains the requested positioning information. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Requested Positioning Data Collection; M; BIT STRING (SIZE(8)); Each position in the bitmap represents a requested positioning information: First bit: UL-RTOA Second bit: gNB Rx-Tx Time Difference Third bit: LoS/NLoS information Value ‘1’ indicates ‘needed’, Value ‘0’ indicates ‘not needed’. Other bits reserved for future use.. A

As can be seen from the above IE definition, the gNB may request to the LMF the reporting of UL-RTOA, gNB Rx-Tx Time Difference and LoS/NLoS information. There is a FFS from the last meeting on whether also the UE location can be requested to be reported by the LMF [2]:

FFS if UE location can be requested.

In our understanding, there could be privacy concerns in providing the actual UE location. Moreover, there is no RAN1 agreement stating that UE location can be provided as Part B information.

Proposal 2: RAN3 to agree that the UE location cannot be requested to be reported by the LMF.

For the reporting of Part B information from the LMF, it was agreed to introduce a new class 2 procedure in NRPPa termed as Positioning Data Collection Report, and this is reflected in the latest version of the NRPPa BLCR [7]. Within the POSITIONING DATA COLLECTION REPORT message the LMF may include the requested information encoded according to the Positioning Data Information IE. However, there could be the possibility for the LMF not being able to provide any Part B information to the gNB, and in the NRPPa BLCR this scenario is considered by means of the Positioning Data Unavailable IE whose actual encoding is FFS:

### 9.1.X Messages for Positioning Data Collection Information Transfer Procedures #### 9.1.X.1 POSITIONING DATA COLLECTION REPORT This message is sent to report positioning information for data collection. Direction: LMF NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3; YES; reject. NRPPa Transaction ID; M; 9.2.4; -. LMF Measurement ID; M; INTEGER (1..65536, …); YES; reject. RAN Measurement ID; M; INTEGER (1..65536, …); YES; reject. Positioning Data Information; O; 9.2.Z2; YES; ignore. Positioning Data Unavailable; O; ENUMERATED(true, …) [FFS]; YES; ignore. A

In our view, it would be appropriate to include more self-explanatory codepoints in the ENUMERATED IE type to provide more information to the gNB on the reason why the LMF was not able to provide the requested Part B information. Some useful codepoints could be “data temporarily not available”, which may allow the gNB to request for Part B in a later moment (via the MEASUREMENT REPORT message), as well as “no user consent” which could be useful especially if the request for Part B information pertains a non-PRU UE. We are open to discuss other additional codepoints if motivated properly.

Proposal 3: RAN3 to agree to introduce specific codepoints in the Positioning Data Unavailable IE of the POSITIONING DATA COLLECTION REPORT message indicating the reason why the LMF cannot provide Part B. At least the following codepoints should be considered: “data temporarily not available” and “no user consent”.

When introducing the concept of Part B information, RAN1 defined it as a set of information including the ground truth label (still timing information in Case 3a), the quality indicator of the label and the time stamp of the label:

******** RAN1#116bis ******** Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

In the latest version of the TS 38.305 BLCR in [3] it is mentioned that, to correlate Part A and Part B of the training data sample, some correlation information needs to be provided by the LMF when it provides information related to UE location (i.e., Part B) to the gNB in step 4. of the Positioning Data Collection procedure. The correlation information mentioned in step 4. includes the LMF Measurement ID and RAN Measurement ID. However, RAN1 agreed to use time stamp of Part B which, for Case 3a, is part of the information transmitted by the LMF to the gNB (while Part A and related information such as time stamp is already available at the gNB and remains internal to the gNB, i.e., Part A is never signalled over any network interface as agreed in the last RAN3 meeting):

******** RAN1#120 ******** Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above.

Proposal 4: RAN3 to agree that, for Case 3a training data collection, the correlation of Part A and Part B relies also on the time stamp of Part B (provided by the LMF to the gNB).

When pairing Part B with Part A at the gNB based on the time stamp provided within Part B, there is the risk that the time stamp of Part B may correspond to a time instance (say, T0) which is prior to the time when Part B is received (say, T1). This is shown in Figure 1.

Figure 1 - Correlation of Part A with "outdated" Part B at the gNB.

In this case, the gNB may not be able to collect a Part A with a time stamp around T0, or it may have already discarded any measurements collected around T0 when it receives the Part B at the later time instance T1, as the gNB is not aware about the time stamp of Part B before the gNB receives Part B. As one possibility, the gNB needs to store all past measurements during the data collection phase, but this may pose strict storage requirements. As another possibility, after receiving Part B (at T1) the gNB can generate Part A (say, at T2, where T2 > T1), such that Part A can only be paired with an outdated Part B having time stamp T0 which may result in an improper pairing and affect the model training at the gNB.

Due to the above issues, it would be beneficial for the gNB and LMF to align the time stamp of the Part B to be provided by the LMF: this could be achieved by allowing the gNB to request Part B information to be provided with a given timestamp to enable a proper pairing of Part A and Part B at the gNB.

This enables the gNB to pair Part A and Part B with an aligned time stamp.

Proposal 5: RAN3 to agree that the gNB may request Part B from the LMF to be provided with a given time stamp.

As previously anticipated, in the last meeting RAN3 agreed some Stage 2 TPs for TS 38.300 and TS 38.401 which became the BLCRs for these specifications in [8] and in [9], respectively. Both BLCRs have an FFS on whether the AI/ML model training function can also be located in the OAM:

### *** BLCR to TS 38.300 *** ### X.Y.4 AI/ML-based Positioning AI/ML-based Positioning is described in TS 38.305 [42]. For the deployment of AI/ML assisted positioning with gNB-side AI/ML model, AI/ML Model Training and AI/ML Model Inference are both located in the gNB. The gNB also does performance monitoring of its own AI/ML Model. The other principles for support of AI/ML for NG-RAN described in clause 16.20.2 apply for AI/ML assisted positioning with gNB-side AI/ML model. ### Editor’s note: AI/ML Model Training in OAM for AI/ML assisted positioning with gNB-side AI/ML model is FFS. Text can be further improved, if needed. ### *** BLCR to TS 38.401 *** ## 7.xx Support of AI/ML for NR Air Interface The support of AI/ML assisted positioning is specified in TS 38.300 [2]. In case of CU-DU split architecture, the following scenario may be supported: - AI/ML Model Training and Model Inference are both located in the gNB-CU. Editor’s Note: AI/ML Model Training located in the OAM is FFS.

In our view the training function may be located in the OAM, and this is based on what has been reported in TR 38.843 clause 7.2.4, where the training data generation and termination points are identified for the gNB-side model as follows [10]:

### 7.2.4 Positioning accuracy enhancements **** Skip non-relevant text **** For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: **** Skip non-relevant text **** - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.  Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress. **** Skip non-relevant text ****

Both possibilities concerning the termination point for training data – gNB or OAM – are aligned with the deployment scenarios also specified by RAN3 as part of the “AI/ML for NG-RAN” WI in Rel-18 (refer to TS 38.300 clause 16.20.2).

Proposal 6: RAN3 to agreed that, for AI/ML assisted positioning Case 3a, the AI/ML model training function can also be located in the OAM other than in the gNB.

TPs for the TS 38.300 BLCR and TS 38.401 BLCR reflecting the above Proposal 6 can be found in Annex 2 and Annex 3, respectively.

### 2.1.2 Model inference considerations for Case 3a

Inference aspects have been currently captured in the latest version of the NRPPa BLCR [7], where the TRP Measurement Result IE of the MEASUREMENT RESPONSE and MEASUREMENT REPORT messages sent by the gNB to LMF has been enhanced to include the new IE termed as Inferred measurement. The latter has been introduced to allow the LMF to distinguish whether the measurements reported by the gNB have been derived by means of an AI/ML model deployed in the gNB itself, and it is applicable at least to the UL-RTOA and gNB Rx-Tx Time Difference measurements:

EN: FFS if LoS/NLoS Information can also be indicated as inferred.

There is a FFS on whether the LoS/NLoS information can be inferred by the AI/ML model and reported to the LMF. To the best of our knowledge, there is no explicit RAN1 agreement preventing the LoS/NLoS information to be generated by the model, and eventually this is not a real issue from the LMF perspective because the meaning of the information is the same regardless of whether it is inferred or not. This is different from the timing information because, when inferred by the model, it refers to the direct path which in reality may not exist or not correspond to a LoS path.

Proposal 7: RAN3 to agree that there is no need to indicate that LoS/NLoS information are inferred by the AI/ML model.

Another FFS from the last meeting concerns whether (and how) the LMF should be allowed to request that the measurements reported by the gNB need to be generated via the A/ML model [2]:

FFS: if LMF should specifically request for AIML inferred measurement.

Logically, we should expect that it is the LMF that decides whether the gNB has to report inferred measurements or measurements derived by legacy means, it should not be the gNB which autonomously decides to send inferred measurements. This would allow wasting signalling resources in case the gNB sends inferred measurements which eventually may be discarded by the LMF if it finds out that such type of measurements are useless.

Proposal 8: RAN3 to agree that the LMF should explicitly request the gNB to report measurements derived via an AI/ML model.

On how to implement the above proposal, we think that one possibility is to introduce dedicated bits in the Measurement Characteristics Request Indicator IE of the MEASUREMENT REQUEST message sent by the LMF to request inferred UL-RTOA or inferred gNB Rx-Tx Time Difference.

Proposal 9: RAN3 to agree to introduce dedicated bits in the Measurement Characteristics Request Indicator IE of the MEASUREMENT REQUEST message to allow the LMF to request the gNB the reporting of inferred UL-RTOA or inferred gNB Rx-Tx Time Difference.

Still related to the request for inferred measurements by the LMF, we think that RAN3 should also discuss about whether the gNB should be able to revert back to reporting legacy timing measurements in case the gNB cannot provide the requested inferred measurements. In our understanding, currently the gNB should fail the measurement procedure if inferred measurements cannot be provided, and it should inform the LMF by sending the NRPPa MEASUREMENT FAILURE message with an appropriate cause value – refer to TS 38.455 clause 8.5.1.3. However, we think that the failure of the measurement procedure could be avoided if it would be allowed for the gNB to report the same timing measurements but derived via legacy means, hence allowing the gNB to fallback to legacy non-inferred measurement reporting. The reporting of the legacy measurements may still allow the LMF (which in principle does not know which measurements will be used eventually at the LMF for determining the UE location) to perform the positioning calculation at the UE, e.g., by combining the reported measurements with legacy measurements obtained from the same gNB or that have been requested from other gNBs.

Proposal 10: RAN3 to agree to allow the gNB to provide timing measurements derived via legacy means if the gNB is not able to provide the requested inferred measurements.

A TP for the NRPPa BLCR for Case 3a can be found in Annex 4.

## 2.2 Case 3b

In the last RAN3 meeting most of the discussion on Case 3b was on how to properly capture changes in NRPPa and F1AP specifications in order to support the sample-based version of the UL-RTOA measurements introduced in Rel-19 by RAN1.

In both the latest versions of the NRPPa and F1AP BLCRs in [7] and in [11], respectively, there is FFS on the encoding for requesting and reporting of the sample-based UL-RTOA measurements, where the term “sample-based” is used in multiple instances within the BLCRs. Take NRPPa as example:

In the Rel-19 version of TS 38.215 in [12] RAN1 specified two new measurements that should be used as reference for the reporting of sampled-based timing information as well as sample-based timing and power information. To be specific, RAN1 specified the UL SRS time domain channel timing (UL SRS-TDCT) in clause 5.2.9, and the UL SRS time domain channel power (UL SRS-TDCP) in clause 5.2.10. In order to allow proper alignment and reference to the above mentioned RAN1 specification we think that all sample-based instances in NRPPa and F1AP BLCRs should be revised to directly point to the new measurements as defined by RAN1.

Proposal 11: RAN3 to agree to align the encoding for requesting and reporting the sample-based UL-RTOA measurements in both NRPPa and F1AP BLCRs with the new UL SRS time domain channel timing (UL SRS-TDCT) and UL SRS time domain channel power (UL SRS-TDCP) measurements introduced in TS 38.215.

TPs for the NRPPa BLCR and F1AP BLCR for Case 3b can be found in Annex 5 and Annex 6, respectively.

# Conclusions

Case 3a

Proposal 1: RAN3 to agree that the “proactive” data collection approach is not pursued in Rel-19.

Proposal 2: RAN3 to agree that the UE location cannot be requested to be reported by the LMF.

Proposal 3: RAN3 to agree to introduce specific codepoints in the Positioning Data Unavailable IE of the POSITIONING DATA COLLECTION REPORT message indicating the reason why the LMF cannot provide Part B. At least the following codepoints should be considered: “data temporarily not available” and “no user consent”.

Proposal 4: RAN3 to agree that, for Case 3a training data collection, the correlation of Part A and Part B relies also on the time stamp of Part B (provided by the LMF to the gNB).

Proposal 5: RAN3 to agree that the gNB may request Part B from the LMF to be provided with a given time stamp.

Proposal 6: RAN3 to agreed that, for AI/ML assisted positioning Case 3a, the AI/ML model training function can also be located in the OAM other than in the gNB.

TPs for the TS 38.300 BLCR and TS 38.401 BLCR reflecting the above Proposal 6 can be found in Annex 2 and Annex 3, respectively.

Proposal 7: RAN3 to agree that there is no need to indicate that LoS/NLoS information are inferred by the AI/ML model.

Proposal 8: RAN3 to agree that the LMF should explicitly request the gNB to report measurements derived via an AI/ML model.

Proposal 9: RAN3 to agree to introduce dedicated bits in the Measurement Characteristics Request Indicator IE of the MEASUREMENT REQUEST message to allow the LMF to request the gNB the reporting of inferred UL-RTOA or inferred gNB Rx-Tx Time Difference.

Proposal 10: RAN3 to agree to allow the gNB to provide timing measurements derived via legacy means if the gNB is not able to provide the requested inferred measurements.

A TP for the NRPPa BLCR for Case 3a can be found in Annex 4.

Case 3b

Proposal 11: RAN3 to agree to align the encoding for requesting and reporting the sample-based UL-RTOA measurements in both NRPPa and F1AP BLCRs with the new UL SRS time domain channel timing (UL SRS-TDCT) and UL SRS time domain channel power (UL SRS-TDCP) measurements introduced in TS 38.215.

TPs for the NRPPa BLCR and F1AP BLCR for Case 3b can be found in Annex 5 and Annex 6, respectively.

# References

- RP-251186, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#108

- 3GPP RAN3#128 Chairman’s notes (RAN3_128_agenda_20250523_1530_EOM.doc)

- R3-255095, (BL CR to 38.305) Introduction of AI/ML air, CATT, Ericsson, Nokia, Huawei, Xiaomi, ZTE, CMCC, Samsung, CEWiT, Jio Platforms

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- R3-252487, LS on AI/ML Positioning Case 3b, RAN3 (Ericsson)

- R3-255023 (S2-2505943), Reply LS on AI/ML Positioning Case 3a, Response to LS on AI/ML Positioning Case 3a (S2-2504525 / R3-252487), Rel-19, SA2 (Nokia)

- R3-255098, (BL CR to 38.455) Introduction of AI/ML air, Ericsson, Nokia

- R3-255100, (BL CR to 38.300) Stage 2 for case 3a, Nokia, CATT, Xiaomi, Ericsson, CMCC, Huawei, ZTE, Lenovo, Jio Platforms

- R3-255096	, (BL CR to 38.401) Support of AI/ML assisted positioning, ZTE Corporation, Xiaomi, CATT, Ericsson, Huawei, NEC, Nokia, Jio Platforms

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Rel-18 V18.0.0

- R3-255099, (BL CR to 38.473) Support of Sample-based measurement (case 3b), Xiaomi, Ericsson, ZTE, CATT, Samsung, Nokia, Jio Platforms

- 3GPP TS 38.215, NR; Physical layer measurements, Rel-19 V19.0.0

# Annex 1 – TP for the AI/ML BLCR to TS 38.305 (based on R3-255095)

START OF CHANGES

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

*** skip unmodified text ***

UL-SRS	Uplink Sounding Reference Signal

UL SRS-TDCP	UL SRS Time Domain Channel Power

UL SRS-TDCT	UL SRS Time Domain Channel Timing

UL-TDOA	Uplink Time Difference of Arrival

UPF	User Plane Function

URA	User Range Accuracy

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

Z-AoA	Zenith Angles of Arrival

Next Change

## 6.3	NG-RAN Node terminated protocols

### 6.3.1	NR Positioning Protocol A (NRPPa)

The NR Positioning Protocol A (NRPPa) carries information between the NG-RAN Node and the LMF. It is used to support the following positioning functions:

-	E-CID for E-UTRA where measurements are transferred from the ng-eNB to the LMF.

-	Data collection from ng-eNB's and gNB's for support of OTDOA positioning for E-UTRA.

-	Cell-ID and Cell Portion ID retrieval from gNB's for support of NR Cell ID positioning method.

-	Exchange of information between LMF and NG-RAN node for the purpose of assistance data broadcasting.

-	NR E-CID where measurements are transferred from the gNB to the LMF.

-	NR Multi-RTT where measurements are transferred from the gNB to the LMF.

-	NR UL-AoA where measurements are transferred from the gNB to the LMF.

-	NR UL-TDOA where measurements are transferred from the gNB to the LMF.

-	Data collection from gNBs for support of DL-TDOA, DL-AoD, Multi-RTT, UL-TDOA, UL-AoA.

-	Measurement Preconfiguration Information Transfer which allows the LMF to request the NG-RAN node to pre-configure and activate/deactivate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer which allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-	Transfer of data collection information from LMF to gNB to facilitate training and performance monitoring of AI/ML models at the gNB.

The NRPPa protocol is transparent to the AMF. The AMF routes the NRPPa PDUs transparently based on a Routing ID corresponding to the involved LMF over NG-C interface without knowledge of the involved NRPPa transaction. It carries the NRPPa PDUs over NG-C interface either in UE associated mode or non-UE associated mode.

In case of a split gNB architecture, the NRPPa protocol is terminated at the gNB-CU.

Next Change

## 7.x	NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

### 7.x.1	General

Support for NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning requires inputs of ground truth and related data from LMF and measurements by the gNB.

### 7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve timing information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to UL related positioning procedures described in Clause 8.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

NOTE 1:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clause 8.

NOTE 2: The gNB(s) may also send the NRPPa MEASUREMENT REPORT message to the LMF to indicate that data collection is needed for the UE being positioned.

4.	The LMF sends a NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated in step 3 that positioning data collection is needed. The message includes UE’s timing information and correlation information that enables the gNB to correlate the UE’s timing information with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID, time stamp of the UE’s timing information).

Next Change

## 8.10	Multi-RTT positioning

### 8.10.1	General

In the Multi-RTT positioning method, the UE position is estimated based on measurements performed at both, UE and TRPs. The measurements performed at the UE and TRPs are UE/gNB Rx-Tx time difference measurements (and optionally DL-PRS-RSRP, DL-PRS-RSRPP, UL-SRS-RSRP, UL-SRS-RSRPP, and/or DL-RSCP/UL-RSCP) of DL-PRS and UL-SRS, which are used by an LMF to determine the RTTs.

The gNB Rx-Tx time difference measurements may also be inferred by using a trained AI/ML model hosted by the gNB.

For network verification of UE location in NTN, the measurements can be performed at a single TRP at different time instances. The additional measurements performed at UE are the UE Rx – Tx time difference subframe offset in unit of subframe and the DL timing drift due to Doppler in service link between UE and satellite as defined in TS 38.215 [37].

The UE may require measurement gaps to perform the Multi-RTT measurements from NR TRPs. The UE may request measurement gaps from a gNB using the procedure described in clause 7.4.1.1. The UE may also request to activate pre-configured measurement gaps as described in clause 7.7.2.

NOTE: Multi-RTT positioning with aperiodic or semi-persistent SRS is not supported for a U2N Remote UE.

[…]

#### 8.10.2.3	Information that may be transferred from the gNB to LMF

[…]

The measurement results that may be signalled from gNBs to the LMF is listed in Table 8.10.2.3-3.

Table 8.10.2.3-3: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

gNB Rx-Tx time difference measurement NOTE 2

UL-SRS-RSRP

UL-SRS-RSRPP

UL-RSCP measurement

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type

Time stamp of the measurement

Quality for each measurement

Beam Information of the measurement

LoS/NLoS information for each measurement NOTE 2

ARP ID of the measurement

Mobile TRP Location Information

Measured frequency hops

Aggregated positioning SRS resource ID list

Measurement based on aggregated resources indication

NOTE 1: When used with UL-AoA for hybrid positioning. NOTE 2: These measurements may also be inferred by using a trained AI/ML model hosted by the gNB.

[…]

### 8.10.3	Multi-RTT Positioning Procedures

#### 8.10.3.0	General

The procedures described in this clause support Multi-RTT positioning measurements obtained by the UE and TRPs/gNB.

The procedures for data collection to support training and performance monitoring of the AI/ML model at the gNB for Multi-RTT positioning measurement inference are described in Clause 7.x.

[…]

Next Change

## 8.13	UL-TDOA positioning

### 8.13.1	General

In the UL-TDOA positioning method, the UE position is estimated based on UL-RTOA (and optionally UL-SRS-RSRP and/or UL-SRS-RSRPP and/or UL-RSCP) measurements taken at different TRPs of uplink radio signals from UE, along with other configuration information.

The UL-RTOA measurements of uplink radio signals from UE taken at different TRPs may also be inferred by using a trained AI/ML model hosted by the gNB.

The LMF may also use a trained AI/ML model hosted by the LMF to infer directly the UE location using the TRP measurements.

In order to obtain uplink measurements, the TRPs need to know the characteristics of the SRS signal transmitted by the UE for the time period required to perform uplink measurement. These characteristics should be static over the periodic transmission of SRS during the uplink measurements. Hence, the LMF will indicate to the serving gNB the need to direct the UE to transmit SRS signals for uplink positioning. It is up to the serving gNB to make the final decision on resources to be assigned and to communicate this SRS configuration information back to the LMF so that LMF can forward the SRS configuration to the TRPs. The gNB may decide (e.g., in case no resources are available) to configure no resources for the UE and report the empty resource configuration to the LMF.

NOTE: UL-TDOA positioning with aperiodic or semi-persistent SRS is not supported for a U2N Remote UE.

[…]

### 8.13.2	Information to be transferred between NG-RAN/5GC Elements

This clause defines the information that may be transferred between LMF and gNB/TRPs.

[…]

#### 8.13.2.2	Location Information that may be transferred from the gNBs to LMF

The information that may be transferred from gNBs to the LMF include measurement results listed in Table 8.13.2.2-1. The individual measurements are defined in TS 38.215 [37].

Table 8.13.2.2-1: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

UL-RTOA NOTE 2

UL-SRS-RSRP

UL-SRS-RSRPP

UL-RSCP measurement

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type

Time stamp of the measurement

Quality for each measurement

Beam Information for each measurement

LoS/NLoS information for each measurement NOTE 2

ARP ID of the measurement

Mobile TRP Location Information

Measured frequency hops

Aggregated positioning SRS resource ID list

Measurement based on aggregated resources indication

UL SRS-TDCT NOTE 3

UL SRS-TDCP NOTE3

NOTE 1: When used with UL-AoA for hybrid positioning. NOTE 2: These measurements may also be inferred by using a trained AI/ML model hosted by the gNB. NOTE 3: These measurements may be used by an LMF to directly infer the UE location using a trained AI/ML model hosted by the LMF. UL SRS-TDCP can only be reported jointly with UL SRS-TDCT.

#### 8.13.2.3	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-1.

Table 8.13.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

SSB Information

Periodicity of the SRS for each SRS resource set

Carrier frequency of SRS transmission bandwidth

Bandwidth aggregation request indication

Positioning validity area cell list

Validity area specific SRS information

The TRP measurement request information that may be signalled from the LMF to the gNB is listed in table 8.13.2.3-2.

Table 8.13.2.3-2: TRP Measurement request information that may be transferred from LMF to gNB.

Information

TRP ID, cell ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity and amount

Measurement beam information request

Search window information

Expected UL AoA/ZoA and uncertainty range

Number of TRP Rx TEGs

Response time

Measurement characteristics request indicator

Measurement time occasions for a measurement instance

Time window information for measurements

Information on time window size and number of estimated channel response values for channel measurements in either time domain or time and power domains

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-3.

Table 8.13.2.3-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger List - Activation time

UL-SRS: - Release all

### 8.13.3	UL-TDOA Positioning Procedures

#### 8.13.3.0	General

The procedures described in this clause support UL-TDOA positioning measurements obtained by the gNB and provided to the LMF using NRPPa.

The procedures for data collection to support training and performance monitoring of the AI/ML model at the gNB for UL-TDOA measurement inference are described in Clause 7.x.

The procedures for data collection to support training and performance monitoring of the AI/ML model at the LMF for direct UE location inference are described in TS 23.273 [35].

[…]

END OF CHANGES

# Annex 2 – TP for the AI/ML BLCR to TS 38.300 (based on R3-255100)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

-----------------------------------------------------------Skip Unchanged-----------------------------------------------------------

## X.Y	Support of AI/ML for NR Air Interface

### X.Y.1	Overview

The objective of AI/ML for NR air interface is to improve network performance and user experience, through AI/ML-enabled enhancements to the following features: beam management, CSI prediction and positioning accuracy.

-----------------------------------------------------------Skip Unchanged-----------------------------------------------------------

### X.Y.4	AI/ML-based Positioning

AI/ML-based Positioning is described in TS 38.305 [42].

For the deployment of AI/ML assisted positioning with gNB-side AI/ML model, AI/ML Model Training and AI/ML Model Inference are both located in the gNB. AI/ML Model Training may also be located in OAM. The gNB may also do performance monitoring of its own AI/ML Model. Additional principles for support of AI/ML for NG-RAN as specified in clause 16.20.2 apply for AI/ML assisted positioning with gNB-side AI/ML model.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex 3 – TP for the AI/ML BLCR to TS 38.401 (based on R3-255096)

<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>

## 7.xx	Support of AI/ML for NR Air Interface

The support of AI/ML assisted positioning with gNB-side AI/ML model is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training and Model Inference are both located in the gNB-CU;

- 	AI/ML Model Training is located in OAM and Model Inference is located in the gNB-CU.

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>

# Annex 4 – TP for the AI/ML BLCR to TS 38.455 for Case 3a (based on R3-255098)

<Start of changes>

### 9.1.X	Messages for Positioning Data Collection Information Transfer Procedures

#### 9.1.X.1	POSITIONING DATA COLLECTION REPORT

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Data Information | O |  | 9.2.Z2 |  | YES | ignore |
| Positioning Data Unavailable | O |  | ENUMERATED(data temporarily not available, no user consent, …) |  | YES | ignore |

Next change

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >>Sample-based UL-RTOA |  |  | 9.2.X |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >Inferred measurement | O |  | ENUMERATED(true, …) | This IE is only valid for UL-RTOA and gNB Rx-Tx Time Difference. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

Next change

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information *** skip unmodified text *** Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Thirteen bit: UL SRS-RSRPP in sample-based UL-RTOA. Fourteen bit: Inferred UL-RTOA. Fifteen bit: Inferred gNB Rx-Tx Time Difference. Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

Next change

### 9.2.Z1	Positioning Data Collection Needed

This information element contains the requested positioning information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested Positioning Data Collection | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents a requested positioning information: First bit: UL-RTOA Second bit: gNB Rx-Tx Time Difference Third bit: LoS/NLoS information Value ‘1’ indicates ‘needed’, Value ‘0’ indicates ‘not needed’. Other bits reserved for future use. |
| Time Stamp of Positioning Data Information | O |  | Time Stamp 9.2.42 | Indicates the requested time stamp of Positioning Data Information |

Next change

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

*** skip unmodified text ***

maxnoAggregatedPosSRSCombinations,

maxnoofChannelRes,

id-Cell-ID,

*** skip unmodified text ***

id-ValidityAreaSpecificSRSInformationExtended,

id-ChannelResponseInformation,

id-sample-based-UL-RTOA,

id-Inferred-measurement

*** skip unmodified text ***

PositioningDataUnavailable ::= ENUMERATED {

data-temporarily-not-available,

no-user-consent,

...}

PositioningDataCollectionNeeded ::= SEQUENCE {

Requested-PosDataCollection 		BIT STRING (SIZE(8)),

timeStamp-of-PosDataInformation		TimeStamp		OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { PositioningDataCollectionNeeded-ExtIEs } } OPTIONAL,

...

}

PositioningDataCollectionNeeded-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

*** skip unmodified text ***

END

-- ASN1STOP

*** skip unmodified text ***

<End of changes>

# Annex 5 – TP for the AI/ML BLCR to TS 38.455 for Case 3b (based on R3-255098)

<Start of changes>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ARP	Antenna Reference Point

*** skip unmodified text ***

UL SRS-RSRPP	UL SRS reference signal received path power

UL SRS-TDCP	UL SRS Time Domain Channel Power

UL SRS-TDCT	UL SRS Time Domain Channel Timing

WAAS	Wide Area Augmentation System

Z-AoA	Zenith Angles of Arrival

Next change

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| *** skip unmodified parts *** |  |  |  |  |  |  |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP, UL SRS-TDCT ) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>UL SRS-TDCT Information | O |  | 9.2.Y | Applicable to UL SRS-TDCT only. |  |  |
| *** skip unmodified parts *** |  |  |  |  |  |  |

Next change

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >> UL SRS-TDCT |  |  | 9.2.X |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| *** skip unmodified parts *** |  |  |  |  |  |  |

Next change

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information *** skip unmodified text *** Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Thirteen bit: UL SRS-TDCP. Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

Next change

### 9.2.X	UL SRS-TDCT

This information element contains the UL SRS time domain channel timing (UL SRS-TDCT) measurement with optional UL SRS time domain channel power (UL SRS-TDCP) measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL SRS-TDCT List |  | 1 |  |  |
| >UL SRS-TDCT Item |  | 1..<maxnoofChannelRes> |  |  |
| >>CHOICE Timing Information | M |  |  |  |
| >>>k0 |  |  | INTEGER (0.. 1970049) | TS 38.133 [16] |
| >>>k1 |  |  | INTEGER (0.. 985025) | TS 38.133 [16] |
| >>>k2 |  |  | INTEGER (0.. 492513) | TS 38.133 [16] |
| >>>k3 |  |  | INTEGER (0.. 246257) | TS 38.133 [16] |
| >>>k4 |  |  | INTEGER (0.. 123129) | TS 38.133 [16] |
| >>>k5 |  |  | INTEGER (0.. 61565) | TS 38.133 [16] |
| UL SRS-TDCP | O |  | 9.2.Z |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofChannelRes | Maximum no of timing information of the estimated channel response in time domain. Value is 24 |

### 9.2.Y	UL SRS-TDCT Information

This IE contains information about the requested UL SRS-TDCT measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Channel Response Window Size | M |  | ENUMERATED (32, 64, 128, …) | Represents the window size over which channel response values are selected. |
| Channel Response Number | M |  | ENUMERATED (8, 16, 24, …) | Represents the number of channel response values selected over the channel response window size. |

### 9.2.Z  UL SRS-TDCP

This information element contains the UL SRS time domain channel power measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL SRS-TDCP List |  | 1 |  |  |
| > UL SRS-TDCP Item |  | 1..<maxnoofChannelRes> |  |  |
| >>Power Information | M |  | UL SRS-RSRPP 9.2.72 |  |

Next change

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

*** skip unmodified parts ***

maxnoAggregatedPosSRSCombinations,

maxnoofChannelRes,

id-Cell-ID,

*** skip unmodified parts ***

id-ValidityAreaSpecificSRSInformationExtended,

id-UL-SRS-TDCT-Information,

id-UL-SRS-TDCT,

id-Inferred-measurement

FROM NRPPA-Constants

*** skip unmodified parts ***

#### -- C

*** skip unmodified parts ***

ChannelResponseWindowSize ::= ENUMERATED { 32, 64, 128, ... }

ChannelResponseNumber ::= ENUMERATED { 8, 16, 24, ... }

ChannelTimingInformation ::= CHOICE {

k0					INTEGER (0..1970049),

k1					INTEGER (0..985025),

k2					INTEGER (0..492513),

k3					INTEGER (0..246257),

k4					INTEGER (0..123129),

k5					INTEGER (0..61565),

choice-Extension		ProtocolIE-Single-Container {{ ChannelTimingInformation-ExtIEs}}

}

ChannelTimingInformation-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

*** skip unmodified parts ***

#### -- S

*** skip unmodified parts ***

#### -- T

*** skip unmodified parts ***

TRPMeasurementQuantitiesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ID id-TimingReportingGranularityFactorExtended	CRITICALITY ignore EXTENSION TimingReportingGranularityFactorExtended PRESENCE optional}|

{ID id-UL-SRS-TDCT-Information				CRITICALITY ignore EXTENSION UL-SRS-TDCT-Information 				PRESENCE optional},

...

}

TRPMeasurementType::= ENUMERATED {

gNB-RxTxTimeDiff,

uL-SRS-RSRP,

uL-AoA,

uL-RTOA,

...,

multiple-UL-AoA,

uL-SRS-RSRPP,

ul-RSCP,

uL-SRS-TDCT

}

*** skip unmodified parts ***

TrpMeasuredResultsValue-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-ZoA		CRITICALITY reject TYPE ZoA PRESENCE mandatory}|

{ ID id-MultipleULAoA	CRITICALITY reject TYPE MultipleULAoA PRESENCE mandatory}|

{ ID id-UL-SRS-RSRPP	CRITICALITY reject TYPE UL-SRS-RSRPP 	PRESENCE mandatory}|

{ ID id-UL-RSCPMeas		CRITICALITY reject TYPE UL-RSCPMeas	PRESENCE mandatory}|

{ ID id-UL-SRS-TDCT	CRITICALITY reject TYPE UL-SRS-TDCTUL	PRESENCE mandatory},

...

}

*** skip unmodified parts ***

#### -- U

*** skip unmodified parts ***

UL-SRS-TDCP-List ::= SEQUENCE (SIZE(1..maxnoofChannelRes)) OF UL-SRS-TDCP-Item

UL-SRS-TDCP-Item ::= SEQUENCE {

powerInformation	UL-SRS-RSRPP,

iE-Extensions		ProtocolExtensionContainer { { UL-SRS-TDCP-Item-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-TDCP-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

UL-SRS-TDCT ::= SEQUENCE {

uL-SRS-TDCT-List			UL-SRS-TDCT-List,

iE-Extensions				ProtocolExtensionContainer { { UL-SRS-TDCT-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-TDCT-ExtIEs 	NRPPA-PROTOCOL-EXTENSION ::= {

...

}

UL-SRS-TDCT-List ::= SEQUENCE (SIZE(1..maxnoofChannelRes)) OF UL-SRS-TDCT-Item

UL-SRS-TDCT-Item ::= SEQUENCE {

timingInformation	ChannelTimingInformation,

uL-TDCP-List		UL-SRS-TDCP-List				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { UL-SRS-TDCT-Item-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-TDCT-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

UL-SRS-TDCT-Information ::= SEQUENCE {

channelResponseWindowSize	ChannelResponseWindowSize,

channelResponseNumber		ChannelResponseNumber,

iE-Extensions				ProtocolExtensionContainer { { UL-SRS-TDCT-Information-ExtIEs} } OPTIONAL,

...

}

UL-SRS-TDCT-Information-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

*** skip unmodified parts ***

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

*** skip unmodified parts ***

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

*** skip unmodified parts ***

id-ValidityAreaSpecificSRSInformationExtended					ProtocolIE-ID ::= 166

id- UL-SRS-TDCT-Informationid									ProtocolIE-ID ::= XX1

id- UL-SRS-TDCT											ProtocolIE-ID ::= XX2

id-PositioningDataInformation									ProtocolIE-ID ::= XX3

id-PositioningDataUnavailable									ProtocolIE-ID ::= XX4

id-PositioningDataCollectionNeeded								ProtocolIE-ID ::= XX5

id-Inferred-measurement											ProtocolIE-ID ::= XX6

END

-- ASN1STOP

End of Changes

# Annex 6 – TP for the AI/ML BLCR to TS 38.473 for Case 3b (based on R3-255099)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Change starts >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

*** skip unmodified parts ***

UL-AoA	Uplink Angle of Arrival

UL-RSCP	UL Reference Signal Carrier Phase

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL SRS-TDCP	UL SRS Time Domain Channel Power

UL SRS-TDCT	UL SRS Time Domain Channel Timing

V2X	Vehicle-to-Everything

Z-AoA	Zenith Angles of Arrival

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.2.12.3	POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request the gNB-DU to configure a positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| *** skip unmodified parts *** |  |  |  |  |  |  |
| Positioning Measurement Quantities |  | 1 |  |  | YES | reject |
| >Positioning Measurement Quantities Item |  | 1..<maxnoofPosMeas> |  |  | EACH |  |
| >>Positioning Measurement Type | M |  | ENUMERATED (gNB RX-TX, UL-SRS-RSRP, UL AoA, UL RTOA, …, Multiple UL AoA, UL SRS-RSRPP, UL-RSCP, UL SRS-TDCT) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | TS 38.133 [38] This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5, and so on, see TS 38.133 [38]. | YES | ignore |
| >>UL SRS-TDCT Information | O |  | 9.3.1.x2 | Applicable to UL SRS-TDCT only. | Yes | Ignore |
| *** skip unmodified parts *** |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.3.1.166	Positioning Measurement Result

The purpose of this information element is to provide the measurement result(s).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning Measured Result Item |  | 1 .. <maxnoofPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  |  |  |  |  |
| >>>UL Angle of Arrival | M |  | 9.3.1.167 |  | - |  |
| >>UL SRS-RSRP |  |  |  |  |  |  |
| >>>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA |  |  |  |  |  |  |
| >>>UL RTOA | M |  | UL RTOA Measurement 9.3.1.168 |  | - |  |
| >>gNB Rx-Tx Time Difference |  |  |  |  |  |  |
| >>>gNB Rx-Tx Time Difference | M |  | 9.3.1.170 |  | - |  |
| >>Zenith Angle of Arrival Information |  |  |  |  | YES | reject |
| >>>Zenith Angle of Arrival Information | M |  | 9.3.1.239 |  | - |  |
| >>Multiple UL AoA |  |  |  |  | YES | reject |
| >>>Multiple UL AoA | M |  | 9.3.1.245 |  | - |  |
| >>UL SRS-RSRPP |  |  |  |  | YES | reject |
| >>>UL SRS-RSRPP | M |  | 9.3.1.246 |  | - |  |
| >>UL RSCP |  |  |  |  | YES | reject |
| >>>UL RSCP | M |  | 9.3.1.335 |  | - |  |
| >>UL SRS-TDCT |  |  |  |  | YES | reject |
| >>>UL SRS-TDCT | M |  | 9.3.1.x1 |  | - |  |
| >Time Stamp | M |  | 9.3.1.171 |  | - |  |
| >Measurement Quality | O |  | TRP Measurement Quality 9.3.1.172 |  | - |  |
| >Measurement Beam Information | O |  | 9.3.1.173 |  | - |  |
| >ARP ID | O |  | 9.3.1.244 |  | YES | ignore |
| >SRS Resource type | O |  | 9.3.1.247 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.3.1.249 |  | YES | ignore |
| *** skip unmodified parts *** |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.3.1.254	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by the gNB-CU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information *** skip unmodified parts *** Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Thirteenth bit: UL SRS-TDCP Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x1	UL SRS-TDCT

This information element contains the UL SRS time domain channel timing (UL SRS-TDCT) measurement with optional UL SRS time domain channel power (UL SRS-TDCP) measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL SRS-TDCT List |  |  |  |  |
| >UL SRS TDCT Item |  | 1..<maxnoofChannelRes> |  |  |
| >>CHOICE Timing Information | M |  |  |  |
| >>>k0 |  |  |  |  |
| >>>>k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [38] |
| >>>k1 |  |  |  |  |
| >>>>k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [38] |
| >>>k2 |  |  |  |  |
| >>>>k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [38] |
| >>>k3 |  |  |  |  |
| >>>>k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [38] |
| >>>k4 |  |  |  |  |
| >>>>k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [38] |
| >>>k5 |  |  |  |  |
| >>>>k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [38] |
| UL SRS-TDCP | O |  | 9.3.1.x3 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofChannelRes | Maximum no of timing information of the estimated channel response in time domain. Value is 24 |

#### 9.3.1.x2	UL SRS-TDCT Information

This IE contains information about the requested UL SRS-TDCT measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Channel Response Window Size | M |  | ENUMERATED (32, 64, 128, …) | Represents the window size over which channel response values are selected. |
| Channel Response Number | M |  | ENUMERATED (8, 16, 24, …) | Represents the number of channel responses selected over the channel response window size. |

#### 9.2.x3  UL SRS-TDCP

This information element contains the UL SRS time domain channel power measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL SRS-TDCP List |  | 1 |  |  |
| > UL SRS-TDCP Item |  | 1..<maxnoofChannelRes> |  |  |
| >>Power Information | M |  | UL SRS-RSRPP 9.3.1.246 |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<< next change (ASN.1)>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

*** skip unmodified parts ***

id-SRSPosPeriodicConfigHyperSFNIndex,

id-UL-SRS-TDCT-Information,

id-UL-SRS-TDCT,

maxNRARFCN,

*** skip unmodified parts ***

maxnoAggregatedPosSRSCombinations,

maxnoofChannelRes

*** skip unmodified parts ***

*** skip unmodified parts ***

*** skip unmodified parts ***

#### -- M

*** skip unmodified parts ***

MeasuredResultsValue-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-ZoAInformation	CRITICALITY reject TYPE ZoAInformation	PRESENCE mandatory}|

{ ID id-MultipleULAoA	CRITICALITY reject TYPE MultipleULAoA	PRESENCE mandatory}|

{ ID id-UL-SRS-RSRPP	CRITICALITY reject TYPE UL-SRS-RSRPP	PRESENCE mandatory}|

{ ID id-UL-RSCP			CRITICALITY reject TYPE UL-RSCP			PRESENCE mandatory}|

{ ID id-UL-SRS-TDCT	CRITICALITY reject TYPE UL-SRS-TDCT	PRESENCE mandatory},

...

}

*** skip unmodified parts ***

#### -- P

*** skip unmodified parts ***

PosMeasurementQuantities-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-TimingReportingGranularityFactorExtended	CRITICALITY ignore EXTENSION TimingReportingGranularityFactorExtended PRESENCE optional}|

{ID id-UL-SRS-TDCT-Information				CRITICALITY ignore EXTENSION UL-SRS-TDCT-Information	 				PRESENCE optional},

...

}

*** skip unmodified parts ***

PosMeasurementType ::= ENUMERATED {

gnb-rx-tx,

ul-srs-rsrp,

ul-aoa,

ul-rtoa,

... ,

multiple-ul-aoa,

ul-srs-rsrpp,

ul-rscp,

ul-srs-tdct

}

*** skip unmodified parts ***

*** skip unmodified parts ***

#### -- T

*** skip unmodified parts ***

TimingInformation ::= CHOICE {

k0					INTEGER (0..1970049),

k1					INTEGER (0..985025),

k2					INTEGER (0..492513),

k3					INTEGER (0..246257),

k4					INTEGER (0..123129),

k5					INTEGER (0..61565),

choice-extension			ProtocolIE-SingleContainer { { TimingInformation-ExtIEs } }

}

TimingInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

*** skip unmodified parts ***

#### -- U

*** skip unmodified parts ***

UL-SRS-TDCP-List ::= SEQUENCE (SIZE(1..maxnoofChannelRes)) OF UL-SRS-TDCP-Item

UL-SRS-TDCP-Item ::= SEQUENCE {

powerInformation	UL-SRS-RSRPP,

iE-Extensions		ProtocolExtensionContainer { { UL-SRS-TDCP-Item-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-TDCP-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-SRS-TDCT ::= SEQUENCE {

uL-SRS-TDCT-List			UL-SRS-TDCT-List,

iE-Extensions				ProtocolExtensionContainer { { UL-SRS-TDCT-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-TDCT-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-SRS-TDCT-List::= SEQUENCE (SIZE(1..maxnoofChannelRes)) OF UL-SRS-TDCT-Item

UL-SRS-TDCT-Item ::=SEQUENCE {

timingInformation	TimingInformation,

uL-TDCP-List		UL-SRS-TDCP-List	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { UL-SRS-TDCT-Item-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-TDCT-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-SRS-TDCT-Information ::= SEQUENCE {

channelResponseWindowSize		ENUMERATED {ws32, ws64, ws128, ...},

channelResponseNumber			ENUMERATED {n8, n16, n24, ...},

iE-Extension			ProtocolExtensionContainer { { UL-SRS-TDCT-Information-ExtIEs} }	OPTIONAL,

...

}

UL-SRS-TDCT-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

*** skip unmodified parts ***

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

*** skip unmodified parts ***

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-UL-SRS-TDCT-Information						ProtocolIE-ID ::= xx1

id-UL-SRS-TDCT									ProtocolIE-ID ::= xx2