3GPP TSG-RAN WG3 Meeting #129	R3-255435

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	17.2

Source:	Nokia

Title:	(TP for BLCR to TS 38.473) On assistance information for OD-SSB over F1

Document for:	Text Proposal

# 1	Introduction

In this paper we provide further discussion on whether and how to introduce assistance information for OD-SSB over the F1 interface.

# 2	Discussion

RAN3 has discussed at previous meetings whether and how the gNB-CU should provide some assistance information to the gNB-DU for activation/deactivation of the OD-SSB functionality (see e.g. summary of discussion from RAN3#127bis in [1]). E.g. it was discussed reuse of the existing Cells Allowed to be Deactivated List IE to indicate the allowed cells for on-demand SSB operation to the DU, without agreement.

We here remind background for our view that such assistance information is not needed.

RAN1 has agreed two cases for OD-SSB operation:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

Here are some examples of OD-SSB operation:

Figure 1: Examples of OD-SSB operation case #2 (always-on SSB is periodically transmitted on the cell) and case #1 (only on-demand SSB is transmitted on the cell)

For case #2, the cell will keep always-on SSB operation in addition to the OD-SSB operation. Case #2 operation therefore doesn’t alter the capability of the cell to operate as a suitable or acceptable cell for idle-mode UEs, or to serve UEs as PCell or PSCell.

Observation 1:  OD-SSB case #2 operation doesn’t prevent the cell to operate as a suitable or acceptable cell for idle-mode UEs, or to serve UEs as PCell or PSCell.

It therefore seems clear that there is no need for the gNB-CU to be involved in activation and deactivation of OD-SSB operation in case #2.

Proposal 1: For OD-SSB operation case #2 (always-on SSB is periodically transmitted on the cell), it is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”. gNB-CU assistance is not needed.

Also for OD-SSB operation case #1 (no always-on SSB on the cell) we believe that no such assistance information is required over F1 for the following reasons:

- RAN1 and RAN2 have been discussing the various aspects of the functionality of OD-SSB operation – the only agreement which could involve gNB-CU is the one where RRC signalling could be used to activate the functionality (if it is done along with the configuration for the first time). Even this agreement doesn’t require any authorization or assistance information from the gNB-CU but rather reuse existing RRC/F1 signalling. There are no other aspects that may have any gNB-CU relevance, and

- The concept of SSB-less SCell operation for intra-band CA was introduced in Rel-15. It was enhanced to inter-band CA also in Rel-18. In these legacy functionalities, no such assistance information over F1 was considered required – and we therefore don’t see why it should be required for the Rel-19 functionality under discussion.

Proposal 2: For OD-SSB operation case #1 (no always-on SSB on the cell), the gNB-DU has sufficient information to determine whether it can switch from regular SSB transmission to OD-SSB SCell operation. Additional gNB-CU assistance is therefore not needed.

In case our proposal 2 above is not agreeable by RAN3, we next discuss alternative solutions for providing additional assistance information to the gNB-DU.

One option (option 1) is to reuse the Rel-18 F1AP Cells Allowed to be Deactivated List IE (in the GNB-CU CONFIGURATION UPDATE message).

| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

The corresponding procedural text is:

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

It is understood that deactivation of all SSB beams within a cell corresponds to deactivating the cell itself. In deployments where this IE is implemented, the gNB-CU sending this IE can expect that the gNB-DU receiving such information will take the information into account in decisions to deactivate the cell. And although the gNB-DU has no mandate to keep other cells operational, a gNB-CU implementation can also expect that cells that are not included in the list would remain operational if possible.

However, proponents for option 1 have argued that OD-SSB case #1 operation is similar to cell switch off, and for this reason cells included in the Cells Allowed to be Deactivated List IE can be considered allowed to either switch off, or by extension in Rel-19, to start OD-SSB operation.

The other option (option 2) would be to introduce a separate list, e.g. Cells Allowed for OD-SSB Operation List IE.

In order to provide background for choice between option 1 and option 2, we look into more detail about similarities and differences between OD-SSB case #1 operation and cell switch off.

OD-SSB case #1 operation vs. cell switch off

For UEs in RRC_IDLE and RRC_INACTIVE, SCell with OD-SSB case #1 operation is the same as cell switch-off: The cell is off and “invisible” (not detectable) to the UE, and the gNB-DU will not have any UE contexts for this cell.

Observation 2: For UEs in RRC_IDLE and RRC_INACTIVE, a cell with OD-SSB case #1 operation and a switched off cell will be the same.

For UEs in RRC_CONNECTED, some companies reminded in offline discussions about the following agreement from RAN1#119bis:

Agreement

For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell)

- UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

Based on this agreement we assume that a gNB will transmit OD-SSB over the air as long as the SCell is activated for one or more UEs served by the cell. Only when the SCell has been deactivated for all UEs served using the OD-SSB feature, the gNB will stop transmitting OD-SSB. If additionally the SCell is deactivated for UEs served using R15/R18 SSB-less operation, or R15/R18 SSB-less operation is not supported by the cell, there will be

- no user plane activity in the cell,

- no UEs would monitor PDCCH,

- and no UEs would be required to do CSI-RS related operations like CQI reporting (and hence not monitor CSI-RS if configured)

In this case the OD-SSB case #1 operation is similar to cell switch off, but only in terms of the instantaneous radio transmission. For several other aspects OD-SSB case #1 operation and cell switch off are very different:

- From UE perspective: UEs may know the OD-SSB cell and have it configured as (deactivated) SCell. These UEs will be prepared for reactivation of the cell according to timing requirements provided by RAN4 (in the order of a few ms). While in case of cell switch off impact on UEs is very high, e.g. UEs will no longer be able to keep the cell configured as SCell and off, and will have to rediscover the cell and get it reconfigured as SCell when the network needs the additional transmission capacity offered by this cell. This is a slow process, and also costly in terms of UE processing and energy consumption.

- From network perspective: The gNB-DU will have UE contexts for the OD-SSB cell, with DRBs being set up, and it is the traffic on these DRBs that are intended to trigger activation of SSB transmission in case of OD-SSB operation. While in case of cell switch off there will be no UE contexts for the cell in the gNB-DU. The gNB-CU may have to first request reactivation of the cell and then set up UE contexts after the UEs have discovered the newly reactivated cell.

From the above, we see that both impact on UEs in RRC_CONNECTED and impact on the network from OD-SSB case #1 operation vs. cell switch off are very different. In particular, reactivation of the cell after cell switch off has high impact on both UEs and the network. Still, energy saving gains may be higher in case of cell switch off because implementations may be able to switch off a higher part of the electronic circuitry than in case of OD-SSB operation. Also from service (capacity) point of view these two features are not comparable.

Observation 3: Impact on UEs in RRC_CONNECTED and impact on the network from OD-SSB case #1 operation vs. cell switch off are very different, and also from service (capacity) point of view these two features are not comparable.

Therefore, in case RAN3 decides that it is beneficial that the gNB-CU indicates to the gNB-DU which cells are allowed to enter OD-SSB operation, such indication needs to be separate from the Rel-18 F1AP Cells Allowed to be Deactivated List IE, and we therefore propose to introduce a new F1AP Cells Allowed for OD-SSB Operation List IE (option 2 above).

Proposal 3: If RAN3 decides that it is beneficial that the gNB-CU indicates to the gNB-DU which cells are allowed to enter OD-SSB operation, introduce a new F1AP Cells Allowed for OD-SSB Operation List IE.

In annex of this paper we also propose F1AP procedural text associated with this new IE.  The proposed text refers to OD-SSB stage 2 description currently being elaborated by RAN2, and we based our F1AP text proposal on the following version of RAN2’s running CR to TS 38.300:

#### 15.4.2.x1	On-demand SSB SCell

On-demand SSB-based SCell operations are supported for UEs in RRC_CONNECTED configured with carrier aggregation (CA), applicable to both intra-band and inter-band CA configurations for FR1 and FR2 in non-shared spectrum. The OD-SSB transmission activation/deactivation command can only be transmitted to a UE configured with an SCell prior to or when receiving the SCell activation command. Both RRC and MAC-CE can be used for signalling the activation/deactivation state of OD-SSB transmissions. Additionally, the same MAC-CE can also update the transmission parameter of an activated OD-SSB after the SCell activation completion. The OD-SSB transmission deactivation can also be achieved implicitly based on the number of OD-SSB bursts to be transmitted configured by RRC. When there is no SSB on the SCell, the OD-SSB transmission is maintained while the SCell is activated. When SSB and OD-SSB have different centre frequencies in the SCell, only a single OD-SSB on a different centre frequency is supported. L3 measurement on OD-SSB is supported as specified in TS 38.331 [12].

Proposal 4: Agree the F1AP TP in annex of this paper

# 4	Conclusion

We have made the following observations and proposals:

Observation 1:  OD-SSB case #2 operation doesn’t prevent the cell to operate as a suitable or acceptable cell for idle-mode UEs, or to serve UEs as PCell or PSCell.

Proposal 1: For OD-SSB operation case #2 (always-on SSB is periodically transmitted on the cell), it is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”. gNB-CU assistance is not needed.

Proposal 2: For OD-SSB operation case #1 (no always-on SSB on the cell), the gNB-DU has sufficient information to determine whether it can switch from regular SSB transmission to OD-SSB SCell operation. Additional gNB-CU assistance is therefore not needed.

Observation 2: For UEs in RRC_IDLE and RRC_INACTIVE, a cell with OD-SSB case #1 operation and a switched off cell will be the same.

Observation 3: Impact on UEs in RRC_CONNECTED and impact on the network from OD-SSB case #1 operation vs. cell switch off are very different, and also from service (capacity) point of view these two features are not comparable.

Proposal 3: If RAN3 decides that it is beneficial that the gNB-CU indicates to the gNB-DU which cells are allowed to enter OD-SSB operation, introduce a new F1AP Cells Allowed for OD-SSB Operation List IE.

Proposal 4: Agree the F1AP TP in annex of this paper

# References

[1]	R3-252357, SoD – OD-SSB, RAN3# 127-bis

[2]	R3-255081, NES F1AP BLCR

# Annex	- TP for BLCR to TS 38.473

The TP is based on [2].

<<< start of changes >>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells Allowed for OD-SSB Operation List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that the indicated cells are eligible for on-demand SSB-based SCell operations as described in TS 38.300 [6].

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the On-Demand SIB1 Cell IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider the cell indicated by the NR CGI IE for On-demand SIB1 operation.

Editor’s note: FFS how to describe the behaviour relative to the On-Demand SIB1 Indicator IE.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| On-Demand SIB1 Cell |  | 0..1 |  |  | YES | ignore |
| >NR CGI | M |  | 9.3.1.12 |  | - |  |
| >On-Demand SIB1 Indicator | M |  | ENUMERATED (start, stop, ...) |  | - |  |
| Cells Allowed for OD-SSB Operation List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed for OD-SSB Operation Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<< end of changes >>>