3GPP TSG-RAN WG3 Meeting #129	R3-255446

# Bengaluru, India, 25th – 29th Aug, 2025

Title: 	Collection of rejected slice information for slice coverage optimisation

Source: 	Ericsson, BT, Deutsche Telekom, Jio Platforms (JPL), FiberCop, InterDigital

Agenda item:	10.3.2

Document Type:	Discussion and decision

# 1	Introduction

In meetings leading up to RAN3#128, discussion on proposals regarding enhancements to log NAS-level rejected slice service requests, due to slice not being supported in the cell from which the UE initiated the request have been discussed without much consensus. During RAN3#128, as a solution to the above issue, an enhancement consisting of reporting to the NG-RAN node of the NAS level rejected slices for a given UE was further discussed and the following FFS was captured in the chair notes:

Whether to do enhancement on reporting rejected slice from CN to NG-RAN based on the request from NG-RAN?

In this contribution, we provide further details about the problem and potential solutions to solve it.

# 2	Discussion

##### 2.1 Slice support within a tracking area

In NR slices are uniformly supported across a Tracking Area within a Registration Area, as specified in [2] the list of slices supported per Tracking Area, i.e., TAI Slice Support List IE defined in section 9.3.1.17 of [2] is first defined by the OAM in the gNB which later on transfers the information to the AMF in the NG SETUP REQUEST or in the RAN CONFIGURATION UPDATE message.

9.2.6.1	NG SETUP REQUEST (excerpt from 3GPP TS 38.413[2])

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| --irrelevant parts omitted-- |  |  |  |  |  |  |

9.2.6.4	RAN CONFIGURATION UPDATE (excerpt from 3GPP TS 38.413[2])

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| --irrelevant parts omitted-- |  |  |  |  |  |  |

Section 8.7.2.2 in [2] describes the behaviour of the AMF upon receiving the TAI Slice Support List IE from the gNB as follows:

If the Supported TA List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE.

Observation 1:  The list of supported slices per tracking area, i.e., the TAI Slice Support List, is configured by the OAM in the gNB which sends this list to the AMF via NGAP signaling.

##### 2.2 Requesting services over a slice

In Sec. 2.1, we established that the slice support per Tracking Area is configured by the OAM in a gNB and this information is then passed to the AMF/core entities by the RAN node during NG setup.

During RAN3#128, one of the main issues blocking progress was the lack of clarity on whether the UE may request for a service over a slice that the UE had not requested for during the initial registration at a later point in time. Specifically, if the UE may request services from a slice that is not currently part of the allowed, rejected, or partially allowed NSSAI lists.

In this section we further clarify the process the UE goes through in order to be able to request a service over a given slice.

For each PLMN a UE is configured with a list of Configured NSSAI and if the UE want to use a service provided by a network slice from the list of Configured NSSAI, the UE has to first register to that network slice. TS 23.501 specifies procedures for UE registration and other associated procedures. Clause 5.15.5.2.1, which specifies the initial registration mentions the following:

… the UE shall provide to the network, in AS layer under the conditions described in clause 5.15.9 and in NAS layer, a Requested NSSAI containing the S-NSSAI(s) corresponding to the Network Slice(s) to which the UE wishes to register, unless they are stored in the UE in the Pending NSSAI.

Regarding the Requested NSSAI IE, the following is specified in clause 5.15.4.1.1 in TS 23.501:

When providing a Requested NSSAI to the network upon registration, the UE in a given PLMN only includes and uses S-NSSAIs applying to this PLMN. … The S-NSSAIs in the Requested NSSAI are part of the Configured and/or Allowed NSSAIs applicable for this PLMN, when they are available. The S-NSSAIs that the UE provides in the Requested NSSAI which are neither in the Allowed NSSAI nor in the Partially Allowed NSSAI, nor provided as a rejected S-NSSAI, shall, by the UE, not be regarded as rejected, i.e. the UE may request to register these S-NSSAIs again next time the UE sends a Requested NSSAI.

Furthermore, in TS 23.501 Sec. 5.15.5.2.2, where the UE initiated aspects are discussed, the following is specified:

In order to change the set of S-NSSAIs the UE is registered to over an Access Type, the UE shall initiate a Registration procedure over this Access Type as specified in clause 5.15.5.2.1.

The above is also specified in TS 23.502, section 4.2.2.2.2 as follows:

The Registration type indicates if the UE wants to perform an Initial Registration (i.e. the UE is in RMDEREGISTERED state), a Mobility Registration Update (i.e. the UE is in RM-REGISTERED state and initiates a Registration procedure due to mobility or due to the UE needs to update its capabilities or protocol parameters, or to request a change of the set of network slices it is allowed to use), …

In context of the discussions in R3#128, and based on the above, we conclude on the following:

Observation 2: The UE is not mandated to register on all slices that the UE was configured with and from which the UE will request services in the current Registration Area.

Observation 3: The UE may trigger a registration procedure at any point in time after the initial Registration to request access for services over a slice that is currently not in the Allowed/Rejected/Partially allowed NSSAI.

In response to UE’s Registration Request the AMF may use the INITIAL CONTEXT SETUP REQUEST or the DOWNLINK NAS TRANSPORT to send the Registration Response to the UE via the gNB. As presented below, besides the NAS PDU IE which contains the Registration Response, both messages contains the following slice related information sent from the AMF to the gNB for the UE at the origin of the Registration Request:

- Allowed NSSAI,

- Target NSSAI,

- Partially Allowed NSSAI.

It has to be noted here that the Rejected NSSAI list is not sent from the AMF to the gNB along with the above NSSAI lists and therefore the gNB is unaware of the network slices that where requested by the UE and rejected by the AMF because, e.g. the slices are not supported in the Tracking area from which the UE has initiated the Registration Request.

Observation 4: As the Rejected NSSAI is not sent by the AMF to the gNB in the INITIAL CONTEXT SETUP REQUEST nor in the DOWNLINK NAS TRANSPORT message, the gNB is not aware of the network slices that were requested by the UE and rejected by the AMF.

9.2.2.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| --irrelevant parts omitted-- |  |  |  |  |  |  |
| >>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.3.1.94 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network | YES | reject |
| --irrelevant parts omitted-- |  |  |  |  |  |  |
| QMC Configuration Information | O |  | 9.3.1.223 |  | YES | ignore |
| Target NSSAI Information | O |  | 9.3.1.229 |  | YES | ignore |
| --irrelevant parts omitted-- |  |  |  |  |  |  |
| Mobile IAB Authorized | O |  | 9.3.1.259 |  | YES | ignore |
| Partially Allowed NSSAI | O |  | 9.3.1.261 | Indicates the S-NSSAIs partially permitted by the network. | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.269 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

According to the current state of the art, when a UE registers on a network, it may request for access to a set of slices as part of the registration process. The UE may then be granted access to a (sub-)set of the requested slices. According to the current state-of-the-art, the RAN is unaware of the slices that the UE was not allowed to access to, i.e., the Rejected NSSAIs. The core network, depending on UE subscription, current registration area / tracking area, and other existing policies, may either allow or reject the use of certain slices by the UE.

On one hand, the AMF/core network has knowledge of the slices that the UE requested, and which slices among the requested were accepted or rejected. However, the AMF/core does not know, by design, the specific cell from which the request originated.  On the other hand, the gNB which is configured by the OAM with the list of network slices supported per tracking area, and which has the knowledge of the cell from which the UE initiated the NAS procedure, receives from the AMF the list of  Allowed/Partially Allowed/Target NSSAI for the UE that has initiated the NAS procedure without receiving also the list of Rejected NSSAI for the UE. As a consequence of this, the RAN OAM, which defines the mapping of lice supported slices per tracking area in the gNB is also unaware of the slices rejected by the AMF for the UE and the correlation between the cell from which the request was made.

The above-described missing piece of information at  gNB, and therefore the OAM, where both are unaware of the cell-level slice rejections prevents the RAN OAM to establish a feedback loop for reviewing / adjusting the planning of supported slices per tracking area and/or available slices within the cells of a tracking area. Mare specifically such a feedback loop would enable the RAN OAM to determine which slice should be made available in an area due to a high number of UEs registering to use a particular slice. In such cases, if the RAN/OAM were made aware of the rejected NSSAIs during registration, they could adjust slice deployment in specific cells or TAs to meet the demand.

Based on the above discussion and observations, we make the following proposal:

Proposal 1: RAN3 to discuss and agree on mechanisms to enable the gNB to request and obtain from the AMF information about the slices requested by the UE and rejected by the AMF.

With access to the information described in this enhancement, the gNB now may:

- know the list of slices that were rejected for a UE at the time of registration

- associate the rejected slice to a cell from which the registration request came from and provide this information to the RAN OAM. (This is the central reason to why this information should be exchanged by the AMF to the RAN instead of AMF sending this directly to the OAM)

The OAM based on the enhanced information from the gNB may observe the statistical occurrences of rejections for the different slices and depending on the slice and its associated use-case, the OAM may decide to modify the slice deployment in the cell/TA/RA.

##### 2.3 Scenarios for proposed enhancement

The above-described advantages and capabilities are in general applicable to any network/deployment. However, below we explain specific use-cases which are either made possible by or are significantly simplified by the discussed mechanism to enable the gNB to obtain information about slices requested by the UE but rejected by the AMF.

Slice deployment for office / remote working locations

Let us consider the case of an operator providing services over a certain slice for a customer such that the employees of the customer can access the slice services, e.g., the enterprise slice, for work-related purposes.  The users may request access to slice services while working from the office premises of their employer but also while working remotely from home. While configuring a good overlap between the slice coverage and the customer’s office premises is rather easy, it may be comparatively more difficult to configure a good slice coverage overlapping with the remote working location of each employee.

Slice coverage for progressing construction sites

Another use case which requires a continuous monitoring and adjustment of the mapping between a slice coverage and a geographical area is the case of “moving as progressing” construction sites such as roads or railways construction sites.

In this case, as the work progresses, the construction site is moving across a geographical area, and any network slice providing coverage for the construction site has to follow the progression of the working place. As the construction site moves over an area, the support of the slice becomes obsolete in some “old” tracking areas and the same slice is “required” in some other “new” tracking areas covering the geographical area towards which the construction site is moving.

The easiest way to follow and adjust the support of the slice in the ‘old” and “new” tracking area, is to monitor the location from which the service of the slice is requested, i.e. monitor both the allowed and the rejected registration request for the slice.

### 2.4 Proposed solution

In order to obtain per-UE information about NAS rejected slices from the AMF, we propose to send an additional IE, in the Initial UE Message (gNB->AMF), to request the AMF to signal the set of rejected slices for the UE as presented below.

9.2.5.1 INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node → AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| --Irrelevant parts omitted-- |  |  |  |  |  |  |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.269 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | Ignore |
| Rejected Slice Information Request | O |  | BOOLEAN | When TRUE, this IE indicates a request from the gNB for the Rejected NSSAI. | YES | Ignore |

Proposal 2: RAN3 to agree extending the INITIAL UE MESSAGE to enable the gNB to request from the AMF the list of slices requested by the UE and rejected by the AMF due to slice(s) being unsupported in the area from which UE initiated the request.

Having received the request, and after the UE performs the Service Request, two possibilities may arise for a UE depending on the number of accepted/rejected slices. In the first scenario, one or more of the service requests on certain slices by the UE are accepted. In such a scenario, the AMF can communicate the list of accepted and rejected slices (new IE) to the gNB via the Initial Context Setup Request message.

9.2.2.1 INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF → NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| --Irrelevant parts omitted-- |  |  |  |  |  |  |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.269 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | Ignore |
| Rejected NSSAI | O |  | 9.3.1.xxx | Indicates the S-NSSAIs rejected by the network | YES | reject |

In the second scenario, where all the slices requested by the UE are rejected, there is no Initial Context Setup Request message that is signalled by the AMF to the gNB as there is no need to setup a UE context. The list of rejected slices, in this case, can be communicated to the gNB from the AMF via the Downlink NAS Transport message.

9.2.5.2 DOWNLINK NAS TRANSPORT

This message is sent by the AMF and is used for carrying NAS information over the NG interface.

Direction: AMF → NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Old AMF | O |  | AMF Name 9.3.3.21 |  | YES | reject |
| --Irrelevant parts omitted-- |  |  |  |  |  |  |
| Mobile IAB Authorized | O |  | 9.3.1.259 |  | YES | Ignore |
| Rejected NSSAI | O |  | 9.3.1.xxx | Indicates the S-NSSAIs rejected by the network | YES | Ignore |

Proposal 3: RAN3 to agree extending the INITIAL CONTEXT SETUP REQUEST and the DOWNLINK NAS TRANSPORT messages to enable the AMF to signal to the gNB the list of slices which were requested by the UE and rejected by the AMF due to slices being unsupported in the area from which the UE initiated the request.

Proposal 4: RAN3 to agree the TP in R3-25xxxx and send an LS to inform SA2 about these agreements.

# 3	Conclusion

Based on the discussion in the previous sections we made the following proposals:

Observation 1:  The list of supported slices per tracking area, i.e., the TAI Slice Support List, is configured by the OAM in the gNB which sends this list to the AMF via NGAP signaling.

Observation 2: The UE is not mandated to register on all slices that the UE was configured with and from which the UE will request services in the current Registration Area.

Observation 3: The UE may trigger a registration procedure at any point in time after the initial Registration to request access for services over a slice that is currently not in the Allowed/Rejected/Partially allowed NSSAI.

Observation 4: As the Rejected NSSAI is not sent by the AMF to the gNB in the INITIAL CONTEXT SETUP REQUEST nor in the DOWNLINK NAS TRANSPORT message, the gNB is not aware of the network slices that were requested by the UE and rejected by the AMF.

Proposal 1: RAN3 to discuss and agree on mechanisms to enable the gNB to request and obtain from the AMF information about the slices requested by the UE and rejected by the AMF.

Proposal 2: RAN3 to agree extending the INITIAL UE MESSAGE to enable the gNB to request from the AMF the list of slices requested by the UE and rejected by the AMF due to slice(s) being unsupported in the area from which UE initiated the request.

Proposal 3: RAN3 to agree extending the INITIAL CONTEXT SETUP REQUEST and the DOWNLINK NAS TRANSPORT messages to enable the AMF to signal to the gNB the list of slices which were requested by the UE and rejected by the AMF due to slices being unsupported in the area from which the UE initiated the request.

Proposal 4: RAN3 to agree the TP in R3-255447 and send an LS to inform SA2 about these agreements.

# References

3GPP TS 38.413 V18.6.0 NG-RAN; NG Application Protocol (NGAP) (2025-06)