3GPP TSG-RAN WG3 Meeting #129															R3-255511

Bengaluru, India, 25th - 29th Aug, 2025

Agenda Item:	14.3

Source:	Huawei

Title:	(TP to BL CR for TS 36.300) Removal of the Suspend/Resume part of TS 36.300

Document for:	discussion

# 1	Introduction

In this paper, we discuss a agree TP, objected by several companies because it is related to Suspend/Resume which was never agree to implement yet in IoT NTN.

# 2	Discussion

In meeting 127-bis there was a compromised to introduce the Suspend/Resume via Editor’s Note, due to further check, as resulting of the implementation of the TP in R3-252454 as below.

# Introduction This is a TP to TS 38.300 to suspend/resume the downlink NG transmission before/after hard FLSO for NR NTN Regenerative Payload. # TP for TS 38.300 16.14.4 Switchover 16.14.4.1 Definitions A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload. Both hard and soft feeder link switchover are supported in NTN. 16.14.4.2 Assumptions A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs. For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links. For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption and/or NG link interruption may occur during the transition between the feeder links. 16.14.4.3 Procedures The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function. For Regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the NGAP Configuration Update message when the Hard Feeder Link switch occurs. Editor’s Note: FFS whether this text is needed.

To us it is obvious that TP is fully and only related to the suspend/Resume, and The Editor’s Note is covering also subclause 16.14.4.2 even thoughts, there is no Editor’s Note in this section. Obviously, Editor’s Note are not everywhere when related TP is introduced, as example in same related TP for TS 38.413 (R3-252409) there is no Editor’s note in ASN.1 neither in procedural text.

#### 8.7.2.2 Successful Operation <<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>> If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9]. If the Downlink NG Transmission Control IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, suspend or resume the downlink NG transmission accordingly. -----------------Next Changes------------------- >>TNL Association Transport Layer Address at AMF; O; CP Transport Layer Information 9.3.2.6; Transport layer address of the AMF.; -. NB-IoT Default Paging DRX; O; 9.3.1.137; YES; ignore. Extended RAN Node Name; O; 9.3.1.193; YES; ignore. Downlink NG Transmission Control; O; ENUMERATED (suspend, resume, …); Indicates the CN to suspend or resume the downlink NG Transmission; YES; ignore. Editor’s Note: FFS whether this IE is needed. -----------------Next Changes------------------- ### 9.4.4 PDU Definitions -- ASN1START -- ************************************************************** -- -- PDU definitions for NGAP. -- -- ************************************************************** NGAP-PDU-Contents { itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) } DEFINITIONS AUTOMATIC TAGS ::= <<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>> WUS-Assistance-Information, XrDeviceWith2Rx, DownlinkNGTransmissionControl FROM NGAP-IEs <<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>> id-XrDeviceWith2Rx, id-SLPositioningRangingServiceInfo, id-DownlinkNGTransmissionControl FROM NGAP-Constants;

Then in meeting RAN3 128, some company force an early inclusion of a part of this NR in IoT BL CR by agreed R3-253923 against objection of other companies:

#### 23.21.5.2 Assumptions A feeder link switchover may result in transferring the established connection for the affected UEs between two eNBs. For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links. For hard feeder link switchover, an NTN payload only connects to one NTN Gateway at any given time i.e. a radio link interruption and/or S1 link interruption may occur during the transition between the feeder links.

We should also comment that for us a hard feeder link switchover result of radio link interruption between over the feeder link not necessarily over Uu or S1. In any case the S1 interruption meaning in such case need further dedicated clarification, discussion for any TP agreement….

Above going against an objection, the companies bringing this CR also break the RAN3 agreement to not discuss implement same topic in LTE before the conclusion in NR.

Proposal: We propose then to remove all related implementation of the suspend/resume from BL CR TS 36.300. And agreed the TP in Annex A.

# 2	References

- R3-25xxxx Downlink NG Transmission Suspend/Resume; Ericsson, Thales, Huawei and all

# Annex A: for TS 36.300 Remove the Editor’s Notes related to the suspend/resume

Note: Part to remove is highlighted in cyan

-----------------Start of the Changes-------------------

#### 23.21.5.2	Assumptions

A feeder link switchover may result in transferring the established connection for the affected UEs between two eNBs.

For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switchover, an NTN payload only connects to one NTN Gateway at any given time i.e. a radio link interruption may occur during the transition between the feeder links.

#### 23.21.5.3	Procedures

-----------------End of the Change-------------------