3GPP TSG RAN WG3 Meeting #129	R3-255559

Bengaluru, India, 25 - 29 August, 2025

Agenda item:	9.2

Source:	Ericsson

Title:	Discussion on emergency call back and paging with PEIPS

Document for:	Discussion

# 1	Introduction

This paper discusses the issue of PEIPS restriction during emergency PDU session.

# 2	Discussion

## PEIPS issue during emergency PDU session

Under the topic of emergency PDU session and paging, there is a problem related to the usage of Paging Early Indication (PEI) when RAN releases the UE to RRC_INACTIVE. The problem stems from SA2 agreement where they have removed the restriction of PEIPS usage during emergency PDU session from Rel-19, because the emergency callback is considered as a normal call. Specifically, the following sentence was removed from 5.4.12.2 Core Network Assistance for PEIPS in TS 23.501 Rel-19.

- When the UE has an active emergency PDU Session:

  - The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

However, the PEIPS usage restriction still exists in RAN spec during an emergency PDU session, see TS 38.300 section 9.2.2.1:

- The NG-RAN node takes into account the PEI with Paging Subgrouping assistance information for subgroup paging in RRC_INACTIVE except when the UE context contains an emergency PDU session in which case the PEI with Paging Subgrouping assistance information shall not be used according to TS 24.501 [28].

One identified issue is if a Rel-18 gNB releases a Rel-19 UE to RRC_INACTIVE during the emergency PDU session, then according to SA2 specification the Rel-19 UE would still monitor PEI with Paging subgroup ID determined according to PEIPS assistance information. But Rel-18 RAN will not use the PEIPS assistance information and would not send the PEI for which the UE is waiting. The PEI based on PEIPS subgroup ID would hence be lost.

Observation 1: For interworking between Rel-19 UE with Rel-18 network, if the UE has used PEIPS assistance information then the UE released to RRC_INACTIVE would wait for PEI based on Paging subgroup ID. The Rel-18 only network would not use the PEIPS assistance information and would not send the PEI for which the UE is waiting. The PEI based on PEIPS subgroup ID would be lost.

The issue in observation 1 may however not be critical, as the gNB can still send the UE-ID based PEI (the restriction in TS 38.300 does not concern UE-ID based subgrouping) or legacy paging. In that case no paging loss can occur.

Observation 2: In the case of co-existence between Rel-19 UE with legacy Rel-18 network, the gNB can send UE-ID based PEI to UE or performs legacy paging. Thus there would be no paging loss.

In previous meetings, it was proposed to remove the restriction from TS 38.300 [1] to align with SA2. If the restriction is removed from Rel-17/18 TS 38.300 specs, this means that the PEIPS assistance information could be used during RAN paging on the UE that was released to RRC_INACTIVE during an emergency PDU session.

An issue however is that the UE would still not use PEIPS assistance information while emergency PDU session exists, due to possible interworking with existing Rel-18 network which does not use the PEIPS assistance information during emergency PDU session.

Observation 3: Even if the PEIPS usage restriction during emergency PDU session is lifted from RAN spec, a UE which has been released to RRC_INACTIVE during an emergency PDU session would still not monitor PEIPS assistance information while emergency PDU session exists, due to possible interworking with Rel-18 network where the restriction is in place.

Considering that there are already deployed UEs that would not monitor PEI with PEIPS assistance information during emergency PDU session due to the existing restriction, these UEs may only receive legacy RAN paging or PEI with UE_ID based subgrouping from the RAN during an emergency PDU session, but not PEI based on PEIPS subgroup ID. If the RAN restriction is lifted from Rel-17/18, for backward compatibility, the RAN needs to know if PEIPS should be used during RAN paging; and the PEIPS information is signalled from the CN during initial context setup request. Hence the CN needs to know first the UE capability indicating whether the UE monitors PEI based on PEIPS subgroup ID during emergency PDU session or not, so that CN decides if PEIPS should be sent to RAN for PEIPS-based paging this UE during an emergency PDU session.

Observation 4: In case the restriction is removed, the RAN needs to know if PEIPS should be used, which is communicated from the CN. The CN needs to know of UE capability of monitoring PEI during emergency PDU session, if  based on PEIPS subgrouping or not, before sending the PEIPS assistance information to RAN and avoid UE/NW mismatch.

Below a summary of the situations:

| UE is Rel-19, gNB is Rel-19 with no restriction of PEIPS usage during emergency PDU session | UE is Rel-19, gNB is Rel-18 with restriction of PEIPS usage during emergency PDU session | UE is Rel-19, gNB is Rel-18 with no restriction of PEIPS usage during emergency PDU session | UE is Rel-18, gNB is Rel-19 with no restriction of PEIPS usage during emergency PDU session |
| --- | --- | --- | --- |
| UE uses PEIPS NW uses PEIPS UE receives PEI based on PEIPS subgrouping | UE uses PEIPS NW can only page using UE_ID based PEI or legacy paging. UE does not receive PEI based on PEIPS. | UE uses PEIPS NW uses PEIPS UE receives PEI based on PEIPS subgrouping | UE may not use PEIPS during emergency PDU session. CN does not know if UE will monitor PEI based on PEIPS or not. NW may or may not use PEIPS depending on CN’s provided information |
| No issue | Mismatch issue (but no paging loss). | No issue | BC issue, CN needs to know UE capabilities |

Therefore, based on observations above, RAN3 should discuss the following:

- Whether to remove the restriction from RAN specs on PEIPS usage during an emergency PDU session to align with SA2.

- in this case (bullet 1 is approved) discuss how should new network handle interworking with Rel-18 UEs, which do NOT use PEI with PEI assistance information while emergency PDU session exists

Proposal 1: Discuss how can a gNB with no restriction on PEIPS based paging during an emergency PDU session distinguish between a UE expecting PEIPS-based paging during an emergency PDU session and not (e.g., can receive PEI with UE_ID based subgroup or legacy paging, while emergency PDU session exists).

Another solution is to discuss whether only Rel-19 specs needs to be updated, considering that SA2 removed the restriction only from Rel-19 TS 23.501.

# 3	Conclusion

This contribution has provided the following observations and conclusions:

Observation 1: For interworking between Rel-19 UE with Rel-18 network, if the UE has used PEIPS assistance information then the UE released to RRC_INACTIVE would wait for PEI based on Paging subgroup ID. The Rel-18 only network would not use the PEIPS assistance information and would not send the PEI for which the UE is waiting. The PEI based on PEIPS subgroup ID would be lost.

Observation 2: In the case of co-existence between Rel-19 UE with legacy Rel-18 network, the gNB can send UE-ID based PEI to UE or performs legacy paging. Thus there would be no paging loss.

Observation 3: Even if the PEIPS usage restriction during emergency PDU session is lifted from RAN spec, a UE which has been released to RRC_INACTIVE during an emergency PDU session would still not monitor PEIPS assistance information while emergency PDU session exists, due to possible interworking with Rel-18 network where the restriction is in place.

Observation 4: In case the restriction is removed, the RAN needs to know if PEIPS should be used, which is communicated from the CN. The CN needs to know of UE capability of monitoring PEI during emergency PDU session, if  based on PEIPS subgrouping or not, before sending the PEIPS assistance information to RANand avoid UE/NW mismatch.

Proposal 1: Discuss how can a gNB with no restriction on PEIPS based paging during an emergency PDU session distinguish between a UE expecting PEIPS-based paging during an emergency PDU session and not (e.g., can receive PEI with UE_ID based subgroup or legacy paging, while emergency PDU session exists).

Another solution is to discuss whether only Rel-19 specs needs to be updated, considering that SA2 removed the restriction only from Rel-19 TS 23.501.

# 4	Reference

- R3-253408, Correction of PEI and emergency PDU session (Huawei, Qualcomm, Ericsson, China Telecom, Nokia, CATT)