3GPP TSG-RAN WG3 Meeting #129	R3-255578

Bengaluru, India, August 25th–29th, 2025

Agenda Item:	17.3

Source:	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms

Title:	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Document for:	Other

# Introduction

The revised Work Item Description (WID) for “Enhancements of network energy savings for NR” [1] in Rel-19 states the following objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

In this contribution, we discuss issues and solutions regarding Objective 2 of the work item (see highlighted text above), focusing on the impact on the Xn interface.

In particular, we discuss the main principles of the inter NG-RAN node signaling to enable on-demand SIB1 (OD-SIB1) transmission, considering non-split architecture.

# General background for discussion

## General scenario

Figure 1 illustrates an example deployment scenario for OD-SIB1 transmission, where the “NES Cell” of gNB1 is meant to operate in OD-SIB1 transmission mode and the “Cell A” of gNB2 provides the UL Wake-Up Signal (WUS) configuration (that is needed to request SIB1 transmission in the NES Cell) to UEs. The scenario corresponds to Case 2 as defined by RAN1, assumed for normative work according to the WID.

Figure 1: An example deployment scenario for OD-SIB1 transmission (corresponding to Case 2).

We assume that, before the NES Cell operates in OD-SIB1 transmission mode, gNB1 provides the UL WUS configuration of its NES Cell to gNB2 in step 1. In step 2, gNB2 provides the UL WUS configuration of the NES Cell to UEs camping on (or connected to) Cell A. In step 3, when needed, a UE (in RRC Idle/Inactive mode) sends the UL WUS to the NES Cell and, in response, receives SIB1 from the NES Cell in step 4.

## Previous agreements

In relation to the impact on the Xn interface, the following agreements have been captured:

- RAN3#125 meeting [2]:

- NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

- RAN3#125-bis meeting [3]:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- RAN3#126 meeting [4]:

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

The procedure uses non UE-associated signaling.

Cell A stores the UL WUS configuration information after it has received it.

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

- RAN3#127 meeting [5]:

It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity.

It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx.

It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

- RAN3#127-bis meeting [6]:

Step4 is not needed.

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that:

- One Choice “Start with UL WUS Condiguration”

- One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

- RAN3#128 meeting [7]:

One “Provision Request message includes one “OD-SIB1 config R19” referring to the TS 38.331 definition, it is a RRC Container in octet string (presence M)  +  one NES Cell ID (presence M ) +  one Cell-A ID (presence O )

These agreements can be summarized as follows:

- New class 1 Xn procedure for signaling the UL WUS configuration from NES-Cell gNB to Cell-A gNB

  - The initiating (request) message has the choice “start”, which carries the UL WUS configuration for the NES Cell, the NES Cell ID, and optionally the Cell A ID, and the reception of the message triggers Cell-A gNB to start broadcasting the UL WUS configuration in SIBx of Cell A.

  - After accepting the request, Cell-A gNB stores the UL WUS configuration for the NES Cell and broadcasts the UL WUS configuration in SIBx at the first opportunity.

  - The response message (successful or unsuccessful outcome) from Cell-A gNB indicates if the UL WUS configuration is to be broadcasted in Cell A or not.

  - The initiating (request) message has the choice “stop”, which triggers Cell-A gNB to stop broadcasting the UL WUS configuration in SIBx. All related information is removed from local memory in Cell-A gNB.

- New class 2 Xn procedure for signaling the interruption of UL WUS configuration broadcast by Cell-A gNB

  - Cell-A gNB signals to NES-Cell gNB that it stops the UL WUS configuration broadcast in SIBx.

In the following, we address some open issues relating to both the new class 1 and the new class 2 procedures.

# Discussion on the new XnAP procedures

## General aspects

As this is the final meeting of the Work Item, RAN3 should prioritize the resolution of all remaining open issues and finalize outstanding details. In this sense, we would like to reiterate the following guiding principles and observations from our previous paper [8]:

- It is essential to prioritize simple and robust procedures for enabling UL WUS configuration provisioning to minimize the potential for errors.

- The NES Cell is expected to be either in OD-SIB1 mode or periodic SIB1 transmission for very long periods of time, i.e., very few interactions between NES Cell and Cell A.

- The size of the UL WUS configuration is around 30 octets (240 bits) according to RAN1. This is a negligible amount of data to signal over RAN3 interfaces.

- The UL WUS configuration is semi static, and due to the small size of UL WUS Configuration, it is no issue to always send it when needed.

An initial step toward concluding the WI is to reach agreement on the naming of the new procedures and their messages. In our view, the names used so far are largely appropriate, with only minor adjustments needed in the Class 2 procedure for consistency.

RAN3 to agree that the name of the Class 1 procedure is “UL WUS Configuration Provision” and has the following messages: “UL WUS CONFIGURATION PROVISION REQUEST”, “UL WUS CONFIGURATION PROVISION RESPONSE”, and “UL WUS CONFIGURATION PROVISION FAILURE”.

RAN3 to agree that the name of the Class 2 procedure is “UL WUS Configuration Provision Status” and has the following message: “UL WUS CONFIGURATION PROVISION STATUS”.

## Design of the new Class 1 procedure as start/stop request

### Agreed structure of the new Class 1 procedure

As already agreed, the new Class 1 procedure over Xn has two options: start and stop.

- The “start” option carries the UL WUS configuration information, the NES Cell ID, and optionally the Cell A ID, and is used when NES-Cell gNB intends to put the NES Cell into OD-SIB1 operation and a cell of another gNB (Cell-A gNB) needs to assist.

- The “stop” option is used by NES-Cell gNB to signal Cell-A gNB that it can stop providing the UL WUS configuration for a previously accepted request.

### Further discussion related to the new Class 1 procedure

In summary, we propose to design the procedure as a simple start/stop request, sent by NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The Cell-A gNB can accept or reject the “start” request. The Cell-A gNB cannot reject the “stop” request, which is similar to, e.g., the PDU session resource release procedure (where the “release” will never fail).

RAN3 to agree to the design of the new Class 1 procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The “start” request can be accepted or rejected by the Cell-A gNB. The “stop” request is always successful.

We further believe, it is sufficient and most appropriate, that Cell-A gNB responds with “accept” if, and only if, it can provide the UL WUS configuration in the cell requested (by NES-Cell gNB) to be Cell A, if included in the request, and “reject” otherwise. The Cell-A gNB cannot judge if broadcasting the UL WUS configuration in another cell than the requested cell will provide good Cell-A coverage for the NES Cell to be in OD-SIB1 mode; thus, the procedure should fail in this situation.

RAN3 to agree that the new Class 1 procedure fails if Cell-A gNB cannot provide the UL WUS configuration in the requested Cell A, if included in the request message.

### Resolving the IE Naming FFSs related to the new Class 1 procedure

The following tabular is the latest agreed version of the UL WUS CONFIGURATION PROVISION REQUEST message, and it includes all the remaining FFSs related to the new Class 1 procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >>UL WUS configuration (FFS to align to RAN2 spec) | M |  | OCTET STRING | Includes the XX IE (FFS) as specified in TS 38.331 [10]. | – |  |
| >>NR CGI at NG-RAN node1 (Naming FFS) | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the UL WUS configuration/On Demand SIB1(Naming FFS) | – |  |
| >>NR CGI at NG-RAN node2 (Naming FFS) | O |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 requested to broadcast the UL WUS configuration (Naming FFS). | – |  |
| >Stop |  |  |  |  |  |  |
| >> NR CGI at NG-RAN node1 (Naming FFS) | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the UL WUS configuration/On Demand SIB1(Naming FFS) | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

In relation to “UL WUS configuration (FFS to align to RAN2 spec)”, we propose to keep the IE name and only refer to the IE naming in TS 38.331 in the semantics description. For the other IE naming FFSs, which relate to cell naming, we prefer to choose a meaningful naming approach for the IEs. For example, “NR CGI at NG-RAN node1 (Naming FFS)” is changed to “OD-SIB1 Cell” to quickly convey that the Cell ID is for the one initiating the NES On Demand SIB1 operation. Similarly, “NR CGI at NG-RAN node2 (Naming FFS)” is changed to “Assisting Cell”, meaning it is the Cell that will broadcast the OD-SIB1 Configuration

RAN3 to agree to the new IE naming to solve the IE naming FFS in the new Class 1 messages as in the below TP.

## Design of the new Class 2 procedure to signal the feedback regarding the UL WUS configuration provisioning status

### Further discussion related to Cell-A gNB stopping the broadcasting

RAN3 has already agreed to introduce a new Class 2 XnAP procedure so that Cell-A gNB can provide feedback to NES-Cell gNB that it stops the UL WUS configuration provisioning in its SIBx. This is highlighted in the agreement below:

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

It is of paramount importance that there is no significant time gap between the moment Cell-A gNB stops broadcasting the UL WUS configuration in its SIBx and the moment the NES Cell reverts to periodic SIB1 transmission (if needed); otherwise, the NES Cell might not be accessible. For this reason, Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting. This will give NES-Cell gNB enough time to revert the NES Cell to periodic SIB1 broadcasting if needed.

RAN3 to clarify that Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting in Cell-A’s SIBx.

A final point to consider is the consequence of Cell-A gNB signaling that it stops the broadcasting of the UL WUS configuration in its SIBx. In our view, and in line with our statement in Section 3.1 and the past meeting agreements, if Cell-A gNB signals that it stops the broadcasting of the UL WUS configuration, the procedure should terminate.

In other words, the effect of signaling the Class 2 procedure from Cell-A gNB to NES-Cell gNB is the same as signaling the Class 1 procedure from NES-Cell gNB to Cell-A gNB using the option “stop”, i.e., Cell-A gNB removes the UL WUS Configuration information and stops the broadcasting.

RAN3 to clarify that the Class 2 procedure (to signal that Cell-A gNB stops the UL WUS configuration provisioning) terminates the UL WUS configuration provisioning.

### Resolving the Editor’s Note related to the new Class 2 procedure

The following tabular is the latest agreed version of the Class 2 message, and it includes an Editor’s Note about a possible refinement for the message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NR CGI at NG-RAN node2 (FFS) | M (FFS) |  |  | RAN node1 supports the UL WUS configuration/On Demand SIB1(Naming FFS) |  |  |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the status of the UL WUS configuration transmission. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

Editor’s Note: This message will be refined at the next meeting.

Two main issues were raised during the last RAN3 meeting: the cell identity indicated in this message and the presence type. To address these, we must first clarify the purpose of the message.

In our view, the Cell-A gNB signals this message to the NES-Cell gNB to indicate that it cannot provide a sufficient Cell-A coverage for the UL WUS configuration provision. The “sufficient Cell-A coverage” is determined either from the UL WUS CONFIGURATION PROVISION REQUEST message (via the Assisting Cell IE) or from OAM configuration.

For example, consider a scenario where OAM configures the Cell-A gNB to broadcast the UL WUS configuration for the NES Cell across three overlapping cells. If the Cell-A gNB intends to stop broadcasting in one of these cells, but the remaining two still provide adequate coverage, there is no need to notify the NES-Cell gNB. This illustrates that the message in Class 2 procedure should not include the identity of a Cell A, as seen in the agreed tabular above.

Instead, the message in Class 2 procedure should clearly identify the NES Cell that has lost sufficient coverage of the SIB broadcasting. This information must be mandatory, as the NES-Cell gNB may operate multiple cells in on-demand SIB1 mode, all assisted by the same Cell-A gNB.

RAN3 to agree that the message in the new Class 2 procedure should include the identity of the NES Cell and that this IE is mandatory.

A final proposal concerns the name of the Class 2 procedure. As stated in Proposal 2, we propose to keep the names of the procedure and its message more general, rather than focusing solely on the current functionality of indicating the stopping of UL WUS configuration broadcasting. This can allow for future extensions of the procedure.

RAN3 to agree to keep the name of the Class 2 procedure and its message more general. For example, “UL WUS Configuration Provision Status” for the procedure and “UL WUS CONFIGURATION PROVISION STATUS” for the message.

A TP reflecting the above proposals is provided in Appendix A.

RAN3 to agree on the TP for BL CR for TS 38.423 provided in Appendix A.

# Conclusion

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN3 to agree that the name of the Class 1 procedure is “UL WUS Configuration Provision” and has the following messages: “UL WUS CONFIGURATION PROVISION REQUEST”, “UL WUS CONFIGURATION PROVISION RESPONSE”, and “UL WUS CONFIGURATION PROVISION FAILURE”.

Proposal 2	RAN3 to agree that the name of the Class 2 procedure is “UL WUS Configuration Provision Status” and has the following message: “UL WUS CONFIGURATION PROVISION STATUS”.

Proposal 3	RAN3 to agree to the design of the new Class 1 procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The “start” request can be accepted or rejected by the Cell-A gNB. The “stop” request is always successful.

Proposal 4	RAN3 to agree that the new Class 1 procedure fails if Cell-A gNB cannot provide the UL WUS configuration in the requested Cell A, if included in the request message.

Proposal 5	RAN3 to agree to the new IE naming to solve the IE naming FFS in the new Class 1 messages as in the below TP.

Proposal 6	RAN3 to clarify that Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting in Cell-A’s SIBx.

Proposal 7	RAN3 to clarify that the Class 2 procedure (to signal that Cell-A gNB stops the UL WUS configuration provisioning) terminates the UL WUS configuration provisioning.

Proposal 8	RAN3 to agree that the message in the new Class 2 procedure should include the identity of the NES Cell and that this IE is mandatory.

Proposal 9	RAN3 to agree to keep the name of the Class 2 procedure and its message more general. For example, “UL WUS Configuration Provision Status” for the procedure and “UL WUS CONFIGURATION PROVISION STATUS” for the message.

Proposal 10	RAN3 to agree on the TP for BL CR for TS 38.423 provided in Appendix A.

# References

RP-251678, Revised WID on enhancements of network energy savings for NR

R3-244708, Summary of offline disc at RAN3#125, Ericsson (Moderator)

R3-245737, Summary of offline disc at RAN3#125-bis, Ericsson (Moderator)

R3-247806, Summary of offline disc at RAN3#126, Ericsson (Moderator)

R3-250801, Summary of offline disc at RAN3#127, Ericsson (Moderator)

R3-252387, Summary of offline disc at RAN3#127-bis, Ericsson (Moderator)

R3-253811, Summary of offline disc at RAN3#128, Ericsson (Moderator)

R3-253539, Xn impact of On-demand SIB1 for UEs in idle/inactive mode, Ericsson et al.

Text Proposal for BL CR for TS 38.423

*************** Start of TP ***************

### 8.x.x	UL WUS Configuration Provision

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to an NG-RAN node2 and to request NG-RAN node2 to transmit the provided UL WUS configuration information.

The procedure is also used to enable the NG-RAN node1 to request the NG-RAN node2 to stop transmission of the UL WUS configuration.

The procedure uses non-UE -associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision procedure, successful operation

If the start is included in the Request Type IE in the UL WUS CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, broadcast the provided UL WUS Configuration in SIBx (SIB naming FFS) and reply with the UL WUS CONFIGURATION PROVISION RESPONSE message. The NG-RAN node2 stores the UL WUS Configuration information.

If the Assisting Cell IE is included in the UL WUS CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, broadcast the UL WUS Configuration in the cell indicated by the Assisting Cell IE.

If the start is included in the Request Type IE in the UL WUS CONFIGURATION PROVISION REQUEST message and the OD-SIB1 Cell IE indicates a cell for which the NG-RAN node2 is already broadcasting the UL WUS Configuration:

-	If the request is for the existing Assisting Cell(s), the NG-RAN node2 shall, if supported, stop the ongoing broadcasting and broadcast the newly provided UL WUS Configuration in SIBx (SIB naming FFS). The NG-RAN node2 removes the stored UL WUS Configuration information and stores the newly received UL WUS Configuration information.

-	If the request is for a new Assisting Cell(s), the NG-RAN node2 shall, if supported, start to broadcast the provided UL WUS Configuration in SIBx (SIB naming FFS) in the indicated cell.

If the stop is included in the Request Type IE in the UL WUS CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall stop broadcasting the UL WUS Configuration for the given OD-SIB1 Cell in SIBx (SIB naming FFS). NG-RAN node2 removes the stored UL WUS Configuration information.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision procedure, unsuccessful operation

If the NG-RAN node2 cannot broadcast the provided UL WUS Configuration in SIBx (Naming FFS), or a failure occurs during UL WUS Configuration Provision, the NG-RAN node2 shall send the UL WUS CONFIGURATION PROVISION FAILURE message. The message shall contain the Cause IE with an appropriate value.

If the Assisting Cell IE is included in the UL WUS CONFIGURATION PROVISION REQUEST message and the NG-RAN node2 cannot broadcast the provided UL WUS Configuration in SIBx (Naming FFS) of the cell indicated in the Assisting Cell IE, or a failure occurs during UL WUS Configuration Provision, the NG-RAN node2 shall send the UL WUS CONFIGURATION PROVISION FAILURE message. The message shall contain the Cause IE with an appropriate value.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration Provision Status

#### 8.x.y.1	General

This procedure is initiated by an NG-RAN node2 to inform a neighbouring NG-RAN node1 about a previously admitted UL WUS configuration provision being stopped.

The procedure uses non-UE associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Provision Status procedure, successful operation

The NG-RAN node2 shall indicate that a previously admitted UL WUS configuration provision in the UL WUS CONFIGURATION PROVISION STATUS message is being stopped. The NG-RAN node2 removes the stored UL WUS Configuration information after signalling this message.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

*************** Next change ***************

#### 9.x.x.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request to start or to stop the provisioning of an UL WUS configuration in one assisting cell.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >>UL WUS Configuration | M |  | OCTET STRING | Includes the OD-SIB1-Config IE as specified in SIBxx (FFS) in TS 38.331 [10]. | – |  |
| >>OD-SIB1 Cell | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 which supports On-Demand SIB1 | – |  |
| >>Assisting Cell | O |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 requested to broadcast the OD-SIB1 configuration. | – |  |
| >Stop |  |  |  |  |  |  |
| >>OD-SIB1 Cell | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 which supports On-Demand SIB1 | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.y	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration will be provided by the NG-RAN node2 as requested by the NG-RAN node1.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| OD-SIB1 Cell | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 which supports On-Demand SIB1 | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.z	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration cannot be provided by the NG-RAN node2 as requested by the NG-RAN node1.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| OD-SIB1 Cell | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 which supports On-Demand SIB1 | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.t	UL WUS CONFIGURATION PROVISION STATUS

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted UL WUS configuration provision is being stopped.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| OD-SIB1 Cell | M |  |  | Indicates the NR cell of the NG-RAN node1 which supports On-Demand SIB1 |  |  |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the status of the UL WUS configuration transmission. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

*************** Next change ***************

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextConfirm,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart,

HandoverSuccess,

ConditionalHandoverCancel,

EarlyStatusTransfer,

FailureIndication,

HandoverReport,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

AccessAndMobilityIndication,

CellTrafficTrace,

RANMulticastGroupPaging,

ScgFailureInformationReport,

ScgFailureTransfer,

F1CTrafficTransfer,

IABTransportMigrationManagementRequest,

IABTransportMigrationManagementResponse,

IABTransportMigrationManagementReject,

IABTransportMigrationModificationRequest,

IABTransportMigrationModificationResponse,

IABResourceCoordinationRequest,

IABResourceCoordinationResponse,

CPCCancel,

PartialUEContextTransfer,

PartialUEContextTransferAcknowledge,

PartialUEContextTransferFailure,

RachIndication,

DataCollectionRequest,

DataCollectionResponse,

DataCollectionFailure,

DataCollectionUpdate,

ULWUSConfigurationProvisionRequest,

ULWUSConfigurationProvisionResponse,

ULWUSConfigurationProvisionFailure,

ULWUSConfigurationProvisionStatus

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-cellTrafficTrace,

id-RANMulticastGroupPaging,

id-scgFailureInformationReport,

id-scgFailureTransfer,

id-f1CTrafficTransfer,

id-iABTransportMigrationManagement,

id-iABTransportMigrationModification,

id-iABResourceCoordination,

id-retrieveUEContextConfirm,

id-cPCCancel,

id-partialUEContextTransfer,

id-rachIndication,

id-dataCollectionReportingInitiation,

id-dataCollectionReporting,

id-uLWUSConfigurationProvision,

id-uLWUSConfigurationProvisionStatus

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

XnAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnRemoval												|

xnSetup													|

nGRANnodeConfigurationUpdate							|

e-UTRA-NR-CellResourceCoordination						|

cellActivation											|

reset													|

resourceStatusReportingInitiation						|

mobilitySettingsChange									|

iABTransportMigrationManagement							|

iABTransportMigrationModification						|

iABResourceCoordination									|

partialUEContextTransfer								|

dataCollectionReportingInitiation						|

uLWUSConfigurationProvision,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport				|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				|

cellTrafficTrace						|

rANMulticastGroupPaging					|

scgFailureInformationReport				|

scgFailureTransfer						|

f1CTrafficTransfer						|

retrieveUEContextConfirm				|

cPCCancel								|

rachIndication							|

dataCollectionReporting					|

uLWUSConfigurationProvisionStatus,

...

}

*************** Next change ***************

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

sNStatusTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-sNStatusTransfer

CRITICALITY				ignore

}

*************** Next change ***************

dataCollectionReportingInitiation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				DataCollectionRequest

SUCCESSFUL OUTCOME				DataCollectionResponse

UNSUCCESSFUL OUTCOME			DataCollectionFailure

PROCEDURE CODE					id-dataCollectionReportingInitiation

CRITICALITY						reject

}

dataCollectionReporting	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataCollectionUpdate

PROCEDURE CODE			id-dataCollectionReporting

CRITICALITY				ignore

}

uLWUSConfigurationProvision		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ULWUSConfigurationProvisionRequest

SUCCESSFUL OUTCOME				ULWUSConfigurationProvisionResponse

UNSUCCESSFUL OUTCOME			ULWUSConfigurationProvisionFailure

PROCEDURE CODE					id-uLWUSConfigurationProvision

CRITICALITY						reject

}

uLWUSConfigurationProvisionStatus	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULWUSConfigurationProvisionStatus

PROCEDURE CODE			id-uLWUSConfigurationProvisionStatus

CRITICALITY				ignore

}

END

-- ASN1STOP

*************** Next change ***************

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

*************** Next change ***************

id-S-CPAC-Request,

id-sk-Counter,

id-Source-M-NG-RANnodeID,

id-SourceSN-to-TargetSN-QMCInfo,

id-RegistrationRequestForDataCollection,

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection,

id-NodeAssociatedInfoResult,

id-SLPositioning-Ranging-Services-Info,

id-PDUSessionsListToBeReleased-UPError,

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-RequestType,

id-OD-SIB1-Cell,

id-ProvisionStatus,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells,

maxnoofSSBAreas

FROM XnAP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

*************** Next change ***************

-- **************************************************************

--

#### -- DATA COLLECTION UPDATE

--

-- **************************************************************

DataCollectionUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionUpdate-IEs}},

...

}

DataCollectionUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-CellMeasurementResultForDataCollection-List	CRITICALITY ignore	TYPE CellMeasurementResultForDataCollection-List		PRESENCE optional }|

{ ID id-UEAssociatedInfoResult-List					CRITICALITY ignore	TYPE UEAssociatedInfoResult-List							PRESENCE optional }|

{ ID id-NodeAssociatedInfoResult					CRITICALITY ignore	TYPE NodeAssociatedInfoResult								PRESENCE optional },

...

}

-- **************************************************************

--

#### -- UL WUS CONFIGURATION PROVISION REQUEST

--

-- **************************************************************

ULWUSConfigurationProvisionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ULWUSConfigurationProvisionRequest-IEs}},

...

}

ULWUSConfigurationProvisionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RequestType								CRITICALITY reject	TYPE RequestTypeChoice								PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

RequestTypeChoice ::= CHOICE {

Start							ULWUSConfigurationInformation,

Stop							NR-CGI,

choice-extension				ProtocolIE-Single-Container { {RequestTypeChoice-ExtIEs} }

}

RequestTypeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ULWUSConfigurationInformation ::= SEQUENCE {

uLWUSConfiguration					OCTET STRING,

oD-SIB1-Cell						NR-CGI,

assisting-Cell						NR-CGI						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ULWUSConfigurationInformation-ExtIEs} }	OPTIONAL,

...

}

ULWUSConfigurationInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- UL WUS CONFIGURATION PROVISION RESPONSE

--

-- **************************************************************

ULWUSConfigurationProvisionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ULWUSConfigurationProvisionResponse-IEs}},

...

}

ULWUSConfigurationProvisionResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-OD-SIB1-Cell							CRITICALITY reject	TYPE NR-CGI									PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- UL WUS CONFIGURATION PROVISION FAILURE

--

-- **************************************************************

ULWUSConfigurationProvisionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ ULWUSConfigurationProvisionFailure-IEs}},

...

}

ULWUSConfigurationProvisionFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-OD-SIB1-Cell							CRITICALITY ignore	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- UL WUS CONFIGURATION PROVISION STATUS

--

-- **************************************************************

ULWUSConfigurationProvisionStatus ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ULWUSConfigurationProvisionStatus-IEs}},

...

}

ULWUSConfigurationProvisionStatus-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-OD-SIB1-Cell							CRITICALITY ignore	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-ProvisionStatus							CRITICALITY ignore	TYPE ProvisionStatus							PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

ProvisionStatus ::= ENUMERATED {stopped, ...}

END

-- ASN1STOP

*************** Next change ***************

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication											ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-cellTrafficTrace												ProcedureCode ::= 38

id-RANMulticastGroupPaging										ProcedureCode ::= 39

id-scgFailureInformationReport									ProcedureCode ::= 40

id-ProcedureCode41-NotToBeUsed									ProcedureCode ::= 41

id-scgFailureTransfer											ProcedureCode ::= 42

id-f1CTrafficTransfer											ProcedureCode ::= 43

id-iABTransportMigrationManagement								ProcedureCode ::= 44

id-iABTransportMigrationModification							ProcedureCode ::= 45

id-iABResourceCoordination										ProcedureCode ::= 46

id-retrieveUEContextConfirm										ProcedureCode ::= 47

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

id-rachIndication												ProcedureCode ::= 50

id-dataCollectionReportingInitiation							ProcedureCode ::= 51

id-dataCollectionReporting										ProcedureCode ::= 52

id-uLWUSConfigurationProvision									ProcedureCode ::= 5x

id-uLWUSConfigurationProvisionStatus							ProcedureCode ::= 5y

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

*************** Next change ***************

id-CPAC-Preparation-Type																			ProtocolIE-ID ::= 468

id-UserPlaneFailureIndication																		ProtocolIE-ID ::= 469

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-RequestType																						ProtocolIE-ID ::= 4xx

id-OD-SIB1-Cell																						ProtocolIE-ID ::= 4xx+1

id-ProvisionStatus																					ProtocolIE-ID ::= 4xx+3

END

-- ASN1STOP

*************** Start of TP ***************