3GPP TSG-RAN WG3 Meeting #129	R3-255701

Bengaluru, IN, August 25th – 29th, 2025

Title: 	(TP to BL CR for TS38.401) UE Performance Requirement for AI/ML in Split Architecture

Source: 	CMCC

Agenda item: 	11.4

Document Type:	Other

- Introduction

In RAN3#128 meeting, it has been agreed that:

The PDCP level UE performance are feasible in R19.

- Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec

- DL/UL UE Throughput Measurement will not be supported in split architecture in R19.

Add the corresponding description in stage 2 for split architecture.

- TP to BL CR to TS 38.401

<< Start of the change>>

7.11	Support of AI/ML for NG-RAN

7.11.1	General

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

For a split gNB, the Energy Cost (EC) of the gNB is the sum of the Energy Cost of its gNB-DUs.

For a split gNB, the UE Performance(UL/DL Delay) measurements are first collected on gNB-DU at DRB level, then sent to CU-UP for average through F1-U. And gNB-CU-UP shall report the UE Performance Feedback (Average Packet Delay UL/DL)  to gNB-CU-CP. The UE Performance(UL/DL Packet Loss Rate) represent the PDCP level packet loss, which is measured on gNB-CU.

<< end of change>>

- Reference

- R3-253985 (BL CR to 38.401) Enhancement of AI/ML Energy Saving for NGRAN split architecture, CMCC

- TS 38.401,  NG-RAN Architecture Description

- RAN3_128_agenda_20250523_1530_EOM, chairnote