3GPP TSG-RAN WG3 Meeting #129	             R3-255708

Bengaluru, India, Aug 25th- Aug 29th, 2025

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on RAN Architecture aspects for Ambient IoT

Document for:	Other

# 1	Introduction

In RAN3#128 meeting, the discussion on RAN architecture aspects for Ambient IoT has made following agreements and remaining FFSs.

Architecture aspects: The new A-IoT Area is encoded as an A-IoT Area ID. IoT Area ID = PLMN ID +NID(optional) + A-IoT Area Code (OCTET STRING (SIZE(3))) One reader only belongs to one gNB. One reader can map to one or multiple A-IoT Area ID(s). One A-IoT Area may include readers belong to the same or different gNBs. One gNB may serve multiple readers which belong to the same or different A-IoT Areas. OAM configures in the AIOTF the mapping relationships among gNBs, readers and A-IoT areas, as needed. Details of the Reader Location is out the scope of RAN3. Reader Selection refers to SA2 TS 23.369 clause 5.3.3. Further check is needed. RAN A-IoT Information will not be provided over NGAP from NG-RAN node to AIoT CN. WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …) Whether to have A-IoT Support Indicator in the NG SETUP RESPONSE message?

In this contribution, we will provide our views on RAN architecture aspects for interface management and A-IoT reader selection mechanism.

# 2	Discussion

### 2.1	Interface Management

As agreed in previous meeting, RAN3 already decided to enhance the NG Setup procedure in both direct and indirect path deployment. Since some mandatory IEs used in the existing NG Setup procedure may not be suitable for A-IoT services, and to mitigate backward-compatibility risks caused by large-scale changes to mandatory IE presence, RAN3 made a work assumption in last meeting to define the A-IoT Support IE in the NG Setup Request message as ENUMERATED (A-IoT only, A-IoT and NR Uu, …). Considering that an A-IoT RAN node needs to meet the operator’s requirements for supporting both "A-IoT only" and "A-IoT and NR Uu" deployments, the current design of the A-IoT Support IE is well qualified for the above two deployments. Therefore, we believe this work assumption can now be formalized to an agreement.

Proposal 1: Convert the WA to agreement: Define the A-IoT Support IE in the NG SETUP REQUEST message as ENUMERATED (A-IoT only, A-IoT and NR Uu, …).

As the A-IoT Support IE is agreed to be included in the NG SETUP REQUEST message, the similar way should be applied in the NG SETUP RESPONSE message. Currently, the NG SETUP RESPONSE message contains useless non-A-IoT related mandatory IEs, i.e., AMF name, Served GUAMI List, Relative AMF Capacity and Slice Support List. Regarding the A-IoT only deployment, the A-IoT CN node may include the random values in those non-A-IoT related mandatory IEs. Once receiving the A-IoT Support IE in the NG SETUP RESPONSE message with the "A-IoT only" value, the A-IoT RAN node shall ignore those IEs provided by the A-IoT CN node.

Proposal 2: Introduce the A-IoT Support IE in the NG SETUP RESPONSE message as ENUMERATED (A-IoT only, A-IoT and NR Uu, …).

### 2.2 A-IoT Reader Selection

In the previous meeting, RAN3 held extensive discussions on the A-IoT reader selection mechanism. To avoid duplicate discussion on reader selection between SA2 and RAN3, RAN3 reached a common understanding that the reader selection refers to the SA2 specification. However, the description of RAN reader selection in SA2 specification, as excerpted below, does not comprehensively specify all scenarios, i.e., the combinations of the presence or absence of the Requested A-IoT Area List and Requested Reader List carried in the INVENTORY REQUEST message.

### 5.3.3 NG-RAN Node and RAN Reader Selection The AIOTF selects NG-RAN node(s) and optionally RAN readers based on the configured NG-RAN node selection information. The AIOTF may also select the RAN reader based on the stored last known RAN Reader information. The AIOTF obtains the NG-RAN selection information (AIoT Area list, RAN Reader list, and, optionally, the location served by each RAN Reader and each AIoT Area) via OAM or local configuration. When the AIOTF receives an AIoT service request, based on the received Target Area information in the request and the NG-RAN selection information, the AIOTF selects the NG-RAN node(s) to handle the request, and determines a Requested Service Area Information for each selected NG-RAN node. If the RAN Readers within an AIoT Area is configured in AIOTF, the AIOTF can directly select the RAN Reader(s). The Requested Service Area Information provided to each NG-RAN node can be empty, a list of AIoT Areas, a list of RAN Readers, or both a list of AIoT Areas and a list of RAN Readers. NOTE 1: The Target Area information in a received request can span multiple NG-RAN node(s) or can be a subset of the supported AIoT areas of a single NG-RAN node. The AIOTF sends the AIoT service request to each selected NG-RAN node, either directly or through the selected AMF, the AIOTF sends the AIoT service request to each selected NG-RAN node along with the corresponding Requested Service Area Information for the NG-RAN node. If the AIOTF provides an empty Requested Service Area Information in the AIoT service request to the NG-RAN node, then the NG-RAN node may use all available RAN Readers under its coverage. If an AIoT service request includes information about individual target AIoT Device(s), the AIOTF may consider the last known serving RAN Reader(s) from the AIoT Device context to determine the NG-RAN node and RAN Reader(s) for the request. NOTE 2: The RAN Reader ID is not exposed to the AF. NOTE 3: An AIOTF receives the Inventory Report from an NG-RAN node includes a RAN Reader ID that represents the AIoT Device’s location at Reader granularity. NOTE 4: The AIOTF uses the RAN Reader ID and the NG-RAN node to update the last known serving RAN reader information in the local AIoT Device context.

As highlighted in yellow, we already agreed in previous meeting that Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers. Two cases remain for further discussion:

Case 1: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

Case 2: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

For Case 1, the gNB does not have rights to select the readers outside the reader list indicated by the A-IoT CN, and need to choose the reader from the reader list received by A-IoT CN. As the reader list provided by the A-IoT CN is based on the reader’s configured locations with the selection principle being to ensure full coverage of the service area, the gNB should take it into account when selecting the readers.

Proposal 3: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list from the A-IoT CN.

For Case 2, we think this scenario is unlikely to exist as it would be impossible for the A-IoT CN to provide both information at the same time in Inventory Request message which will make the gNB confused if the readers within indicated area and the reader list provided by CN is not consistent.

Proposal 4: The A-IoT CN does not provide both the area and the reader list in Inventory Request at the same time.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide following observations and proposals on RAN architecture aspects for Ambient IoT:

Proposal 1: Convert the WA to agreement: Define the A-IoT Support IE in the NG SETUP REQUEST message as ENUMERATED (A-IoT only, A-IoT and NR Uu, …).

Proposal 2: Introduce the A-IoT Support IE in the NG SETUP RESPONSE message as ENUMERATED (A-IoT only, A-IoT and NR Uu, …).

Proposal 3: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list from the A-IoT CN.

Proposal 4: The A-IoT CN does not provide both the area and the reader list in Inventory Request at the same time.

# 4	Reference

[1] RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TS 23.369 v0.3.0, Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19);