3GPP TSG-RAN3 Meeting #129bis																R3-256548

Prague, CZ, 13-17 October, 2025

Agenda Item:	12.2.1

Source:	ZTE Corporation

Title:	(TP to TR 38.745) Consideration on Multiple-hop UE trajectory

Document for:	other

- Introduction

In Rel-18, cell-based UE trajectory prediction was limited to the first-hop target NG-RAN node. Multi-hop UE trajectory prediction across gNBs was identified as one of the objectives of AI/ML for NG-RAN enhancements phase 3. At the early stage of the Rel-19 phase, the topic of multi-hop UE trajectory prediction across gNBs was discussed, and several solutions were proposed.

In this contribution, we further provide our understanding and proposed solutions for multi-hop UE trajectory prediction across gNBs, along with the corresponding TP to TR 38.745.

- Discussion

Multi-hop UE trajectory can be divided into two categories: predicted information and measured information. For single-hop predicted information, it is currently delivered to the target NG-RAN node via the Handover Request message. For single-hop measured UE trajectory, the measured information is requested and reported through the Data Collection Initiation procedure and Data Collection Update procedure.

In TR38.743, the use case description of multiple-hop UE trajectory prediction was captured:

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node. Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

- Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

- The definition of multi-hop predicted UE trajectory can be captured in TR 38.745.

Based on the mechanism for predicted UE trajectory, the predicted multi-hop UE trajectory could also be delivered to the target gNB via the Handover Request message. When the target node (e.g., gNB2) receives the predicted UE trajectory from source node (e.g., Gnb1), it can, according to this trajectory information, forward the remaining predicted UE trajectory to its own target node (e.g., gNB3) during the next handover.

- The delivery of predicted multi-hop UE trajectory can continue to be carried in the Handover Request message to the next target gNB.

For the measured multi-hop UE trajectory across gNBs, the collected measurement results should be forwarded to the initial gNB. This allows the initial gNB to evaluate and analyze the end-to-end performance of the trajectory prediction and mobility optimization.

The same mechanism can be applied to collect measurements. The initial gNB initiates the Data Collection procedure and configures the duration required for the measurements. Subsequent gNBs, based on the actual UE residence time, also initiate the data collection procedure towards their neighbor gNBs and adjust the configured measurement duration as needed. Upon receiving the Data Collection Update message, the gNB integrates the locally measured UE trajectory with the reported measured UE trajectory and forwards the combined information to the previous requesting gNB.

Following is the example of signaling procedures for multiple hop UE trajectory:

Figure. Signalling procedure for multiple-hop UE trajectory across gNBs

Step 1&2. The initial gNB (gNB1) initiates the Data Collection procedure and configures a 30-second measurement duration.

Step 3. gNB1 sends a HANDOVER REQUEST carrying the predicted UE trajectory.

Step 4. UE handover to next gNB (gNB2), and stay for 20 seconds.

Step 5&6. After the UE hands over to gNB2 and remains for 20 seconds, gNB2—based on the UE’s actual residence time—initiates its own Data Collection procedure toward the neighbor cell, configuring a 10-second duration for gNB3.

Step 7. gNB2 sends a HANDOVER REQUEST with the remaining predicted trajectory to next gNB.

Step 8. The UE then hands over to gNB3 and stays for 10 seconds, during which gNB3 performs the configured measurement.

Step 9&10. gNB3 reports the measured trajectory for 20–30 seconds to gNB2; Upon receiving a Data Collection Update message, the gNB integrates its locally measured UE trajectory with the reported results and forwards the combined information to the previous gNB.

Step 11. gNB2 integrates and relays the aggregated 20–30-second results to gNB1; and gNB1 finally consolidates the end-to-end measured multi-hop UE trajectory over the configured window.

- The collection of multi-hop UE trajectory measurements can be supported by the existing mechanisms with minimal standard impacts. The intermediate measurement gNB integrates its locally measured UE trajectory with the reported results received from the next gNB, and then forwards the combined information to the previous gNB.

- Conclusion

Following is the observations and proposals:

- Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

- The definition of multi-hop predicted UE trajectory can be captured in TR 38.745.

- The delivery of predicted multi-hop UE trajectory can continue to be carried in the Handover Request message to the next target gNB.

- The collection of multi-hop UE trajectory measurements can be supported by the existing mechanisms with minimal standard impacts. The intermediate measurement gNB integrates its locally measured UE trajectory with the reported results received from the next gNB, and then forwards the combined information to the previous gNB.

- Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

For the measured multi-hop UE trajectory across gNBs, the collected measurement results should be forwarded to the initial gNB. This allows the initial gNB to evaluate and analyze the end-to-end performance of the trajectory prediction and mobility optimization.

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

The delivery of predicted multi-hop UE trajectory can continue to be carried in the Handover Request message to the next target gNB. If the remaining predicted UE trajectory information is useful, the next gNB can rely this assistance information, and forwards to the subsequent gNB. The process continues in a similar manner.

The gNBs in this process can request the measured UE trajectory that span across multiple gNBs. The intermediate measurement gNB integrates its locally measured UE trajectory with the reported results received from the next gNB, and then forwards the combined information to the previous gNB.

Following is the signaling example to support multiple hop UE trajectory across gNBs.

Figure. Signalling procedure for multiple-hop UE trajectory across gNBs

Step 1&2. The initial gNB (gNB1) initiates the Data Collection procedure and configures a 30-second measurement duration.

Step 3. gNB1 sends a HANDOVER REQUEST carrying the predicted UE trajectory.

Step 4. UE handover to next gNB (gNB2), and stay for 20 seconds.

Step 5&6. After the UE hands over to gNB2 and remains for 20 seconds, gNB2—based on the UE’s actual residence time—initiates its own Data Collection procedure toward the neighbor cell, configuring a 10-second duration for gNB3.

Step 7. gNB2 sends a HANDOVER REQUEST with the remaining predicted trajectory to next gNB.

Step 8. The UE then hands over to gNB3 and stays for 10 seconds, during which gNB3 performs the configured measurement.

Step 9&10. gNB3 reports the measured trajectory for 20–30 seconds to gNB2; Upon receiving a Data Collection Update message, the gNB integrates its locally measured UE trajectory with the reported results and forwards the combined information to the previous gNB.

Step 11. gNB2 integrates and relays the aggregated 20–30-second results to gNB1; and gNB1 finally consolidates the end-to-end measured multi-hop UE trajectory over the configured window.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>