3GPP TSG-RAN WG3 Meeting #129bis	R3-256556

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	10.2.1

Source:	Nokia

Title:	Initial discussion on 6G study in RAN3 and 6G RAN general principles and requirements

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

# 1	Introduction

We here provide an initial discussion for the RAN3-led part of the Study on 6G Radio [1]. As part of this study, RAN3 will define principles, requirements and potential solutions for the general 6G RAN architecture to be standardized in Rel-21 and beyond. This corresponds to objective 6a of [1]:

(6)	Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

We describe the background and context leading to the 6G study definition. Based on guidance from RAN in [1], we formulate principles that RAN3 may use in study of the 6G RAN architecture. We also propose problem statements that may serve in RAN3’s analysis of potential solutions for core 6G features.

# 2	Discussion

## 2.1	Lessons learned from 5G Context and background leading to the 6G study definition

The present study [1] was approved at RAN#108 (June 2025) with some updates agreed at RAN#109. It is scheduled for Rel-20 time frame until RAN3#136 with the resulting TR 38.760-3 to be approved at RAN#116 (June 2027). An interim milestone is planned in June 2026 (RAN#112), with the following interim study results from RAN3:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

The study in RAN3 runs in parallel with studies in other RAN WGs with common Study Item Description. The initial part of the study also runs in parallel with the Study on 6G Scenarios and requirements conducted by RAN plenary (SID in [2], outcome in TR 38.914, with target RAN#112 (June 2026)). The outcome of RAN’s study is expected to be shared with ITU-R in the form of official liaisons, contributing to ITU-R’s work on requirements for IMT-2030 radio interface technologies. RAN’s study will also serve as guidance for further work in RAN3 and other WGs.

Among the parallel 6G studies in WGs under other TSGs can be mentioned:

- SA2: Study on Architecture for 6G System (TR 23.801-01)

- SA5: Study on 6G Management and Orchestration (TR 28.xxx)

- SA1: Study on 6G Use Cases and Service Requirements (TR 22.870)

Being at the beginning of the 6G study phase, we can draw a parallel between the 5G study phase which was conducted during Rel-14-time frame starting in RAN in 2016 and the 6G study. The 5G RAN study item, named Study on Scenarios and Requirements for Next Generation Access Technologies, resulted in a description of 5G usage scenarios (e.g. eMBB, URLLC), deployment scenarios, key performance indicators, requirements for architecture and migration, and operational requirements. The outcome was published in TR 38.913. It involved parallel studies in all WGs, including RAN WGs and e.g. in SA2 (TR 23.799).

Starting from usage scenarios and deployment scenarios described by RAN, RAN3 defined during the 5G study (TR 38.801) support for tight integration of the legacy E-UTRA RAT and the new 5G radio access technology (NR – New Radio). Different options for tight integration enabled multiple paths for migration from 4G to 5G. These were extensively studied by RAN3 and described in the TR.  One well-known example is the possibility to deploy 5G radio coverage without 5G core network. This was solved via a setup enabling a 5G UE to simultaneously connect to an eNB (with EPC connection) and a new 5G NB where the latter did not have control plane connection to any core network. This setup was given the name non-standalone deployment (NSA), or EN-DC in the normative phase that followed in Rel-15. The options studied for migration involved the definition of three different logical nodes for the 5G RAN:

- the en-gNB: a 5G RAN node supporting 5G radio access (NR) and X2 interface, without control plane connection to any core network. This node is part of the E-UTRAN.

- the ng-eNB: a 5G RAN node supporting 4G radio access (E-UTRA) and Xn interface, connecting to a 5G core network

- the gNB: a 5G RAN node supporting 5G radio access (NR) and Xn interface, connecting to a 5G core network

Among these options the en-gNB and the gNB became widely deployed, however this lead to market fragmentation.

Observation 1: Different options in 5G, lead to market fragmentation and slow uptake of 5G SA and new features.

Another important part of RAN3’s study for 5G was related to the definition of the logical NG-RAN architecture. In particular two aspects of this architecture did not have any precedent from earlier 3GPP studies or work. The first aspect was the support of split deployments where parts of the functionality of the RAN node are handled in a central location e.g. in a data center (the gNB-CU), and the other part being deployed geographically closer to the served cell (gNB-DU). The study in this way laid the grounds for the F1 interface (TS 38.473) applicable for the gNB and the en-gNB, and the W1 interface (TS 37.473) applicable for the ng-eNB.

This represented a significant standardization effort, but did not result in multivendor deployments.

Observation 2: 5G work on CU-DU lead to extra standardization effort with no multivendor deployments.

The second aspect of logical NG-RAN architecture definition related to the separation between control plane and user plane within the RAN. Hence, the CU described above became separated into a CUCP (CU Control Plane) and a CUUP (CU User Plane). The interface between the CUCP and the CUUP became the E1 interface (TS 37.483), applicable to the gNB and the ng-eNB.

RAN3 additionally described in TR 38.801 the outcome of the study of different features led by RAN3 or led by other groups with RAN3 impact: Network Slicing, QoS, SON, Wireless Relay, Interworking with non-3GPP systems.

Observation 3: 5G covered several features (IAB, relays etc.) with no clear commercial viability and which took away focus from features of true commercial needs.

## 2.2	Principles and requirements for the 6G architecture

### 2.2.1	Overall principles

RAN has provided guidance directly applicable for the 6G RAN architecture in the Study Item Description (SID) [1] as follows:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

[…]

- System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

Based on the above and other details provided in [1], the 6G RAN architecture shall support a single, non-backward compatible radio access technology named 6GR (6G Radio).

Furthermore, as per the justification section of the SID, concerns about implementation complexity, energy consumption, and UE burden are especially relevant given the expected wide adoption of 6G across both consumer and industrial verticals, where cost-efficiency and performance trade-offs are critical. The SID therefore provides guidance to e.g. avoid excessive configurations and minimizing multiple options for the same functionality. Based on this we propose:

Proposal 1: RAN3 to ensure a simple 6G RAN architecture design and elimination of support of multiple options.

The following requirement is captured in TR 38.914 [3]:

-	The 6G RAN architecture shall support standalone RAN architecture.

We propose to define the standalone 6G RAN architecture as an architecture supporting the 6GR RAT only, and which is connected to the 6G CN only.

Proposal 2: A standalone 6G RAN supports the 6GR only and is connected to the 6G CN only.

### 2.2.2	Migration, RAT support, mobility

As we saw in section 2.1 above, an important part of RAN3’s study for 5G is related to migration paths from 4G to 5G. Similarly, for the introduction of 6G, analysis of migration scenarios is also required. RAN provides the following guidance for migration paths from 5G to 6G:

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

The following further requirements are captured in TR 38.914 [3]:

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

At current stage RAN3 shall therefore consider 5G to 6G migration based on Multi-RAT Spectrum Sharing (MRSS), in addition to 5G-6G mobility. This represents a significant simplification compared to the 4G to 5G case.

The principle of single RAT 6G framework (from objective 1 in [1]) also means that inter-RAT mobility involving 6G implies inter-system mobility. E.g. handover from 5G to 6G will then from the 6G RAN perspective correspond to an inter-system handover from 5G system (5GS) to 6G system (6GS). However, these aspects will be handled under a separate agenda item 10.7 in later meetings.

When it comes to MRSS, we believe that support of complex coordination for MRSS should be avoided because this would have architectural impacts and go against simplicity requirements discussed in section 2.2.1 above. We therefore foresee a smooth MRSS-based migration mechanism where radio aspects and architecture aspects are decoupled, hence with no RAN3 impact from MRSS.

Proposal 3: RAN3 to agree that there is no RAN3 impact from MRSS.

### 2.2.3	Resiliency

Resiliency is a critical topic for networks in general and it impacts greatly costumer’s experience. The following requirement is captured in TR 38.914 [3]:

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

In 5G, SA2 introduced support for resiliency in the CN based on the adoption of NF sets whereby compute and storage resources were separated. This has proven highly beneficial in the CN from field experience of commercial 5GC resiliency. However, for RAN in 5G, resiliency solutions were only left to network implementation. Yet, given that RAN is a very critical component of the network infrastructure, it is crucial that a more detailed analysis is performed to assess whether more standardized solutions need to be adopted for enabling end-to-end resilient system in 6G, encompassing RAN.

The resiliency in RAN is aimed at minimizing interruption time of connectivity and increased service continuity in case of RAN node outages and maintenance as well as UE radio link failures as proposed in [7].

Observation 4: While CN resiliency was standardized in 5G, RAN resiliency still lacks standard support although it is one of the most critical components of the network infrastructure.

Proposal 4: RAN3 to study solutions for a robust and resilient 6G RAN.

### 2.2.4	Service Awareness in RAN

As we embark into the 6G era, new services and requirements from new applications need to be accommodated into 6G networks. One example of such new services could be evolution of XR.

Another example could be network for AI according to RAN’s assumption in [1]:

Note: NW for AI is assumed to be covered by new services

According to this assumption, network for AI could from the perspective of the 6G RAN represent a service enabling the UE to transfer data over the user plane, e.g. to an external application or AI/ML model training server.

New applications and services such as XR and AI exhibit different traffic patterns compared to eMBB and other traditional traffic types. In recent 5G-A releases, XR has been explored in 3GPP resulting in the introduction of PDU Sets and additional traffic assistance information as well as Rel.20 RAN2 WID was agreed in [6] to study transmission characteristics of mobile AI traffic and specify latency measurements coordination between RAN and CN. XR services are a clear example of how application awareness can help optimizing QoS flow provisioning, and other RAN operations like UE power saving configurations. Still, for 6G, XR use cases continue to evolve further, bringing more challenges to RAN. On the other hand, AI services are expected to become more relevant for 6G, with an explosive growth of AI applications and its integration in different types of devices. Currently in SA1 TR 22.870, different use cases and service requirements for 6G are studied, where AI is included as one of the key technologies.

Given that RAN has scarce resources, it is crucial that RAN obtains the proper means to adequately manage its resources. Thus, to enable efficient scheduling and spectrum efficiency it is crucial that RAN is service/application aware. Service/Application awareness refers to methods for RAN obtaining/being provided relevant information to handle an application or service.

Observation 5: Service/Application awareness refers to methods for RAN obtaining/being provided relevant information to handle an application or service.

For that, there are already some mechanisms in 5G standardization to capture means for Service/Application awareness in RAN such as:

- TSC traffic characteristics (TSCAI) describes the TSC traffic characteristics that may be provided for use by the RAN.

- QoS framework for XR traffic (e.g. PDU Set QoS handling)

- QoE Measurement framework to make QoE metrics visible at RAN (i.e., RAN-visible QoE).

- UE Assistance Information (UAI) with UL traffic information.

However, it is observed that the visibility of the application requirements and characteristics in RAN is limited. To that end, in RAN#109, RAN plenary captured the below requirement in TR 38.914:

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

Given the above requirement from RAN plenary and the clear need for enhanced visibility of RAN for new applications, a study is needed in RAN3 for mechanisms that further enhance RAN application awareness.

Proposal 5: RAN3 to study mechanisms for 6G RAN to be made more service/application aware and allow real time provisioning of application/service characteristics.

Similarly, monitoring of connectivity metrics is also important for enabling services with advanced requirements such as immersive communications and AI traffic. Therefore, 3GPP specifications should enable mechanisms to allow better observability in terms of tracking latency requirements between 3GPP end points (e.g. radio layer) or RAN-CN domains and enabling enhanced reporting (e.g. from UE) with different frequency (per packet, per interval etc.) as well as granularity (per QoS flow, per slice etc.).

Proposal 6: RAN3 to study mechanism for 6G RAN to be allow enhanced observability to ensure service assurance.

### 2.2.5	Network optimization, AI/ML and data collection for 6G RAN

RAN has provided the following requirement for the 6G RAN framework [1]:

- System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

We believe that while this requirement is intended to be taken into account in the design of all RAN features, it also implies that dedicated features for network optimization are needed. Distributed SON features, under the responsibility of RAN3, have reached a high level of maturity in 5G. The above requirement implies in our view that at least a minimum set of distributed SON features is supported in the initial 6G release, and among them can be mentioned e.g. the ANR feature (Automatic Neighbour Relationships). The set of SON features to be supported may be studied in a later phase of the study item.

A further step for 5G network optimization came with the introduction of AI/ML for NG-RAN in Rel-18 and Rel-19, with support for several use cases related to automated network optimization where predictions enable preventive actions. AI/ML for RAN is handled under separate agenda item 10.5 for 6G study and we provide our detailed view about AI/ML for RAN use cases in [5], but here we describe the general principles and requirements.

In RAN study item description, it was agreed that:

- 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

Therefore, it is important that RAN Architecture is designed agnostic to the support of AI/ML use cases. That means that any specific architecture design for a particular use case should be carefully studied and justified, and it should ensure smooth inter-operability with the baseline RAN Architecture. Furthermore, we propose that 5G-A AI/ML framework is to be taken as baseline to support to support the current and new use cases. Evolution of this framework can then be evaluated based on new use cases and conclusions for the AI/ML framework in a later phase of this study.

Proposal 7: 5G-A framework for AI/ML support in RAN to be taken as baseline.

In our view, data collection and signaling enhancements remain the main enabler for AI/ML in RAN. MDT was introduced in Rel-10 in order to collect radio measurement information for use by centralized SON functions. To this end, the OAM system configures MDT measurements in the RAN, and the corresponding MDT measurement reports are sent by the RAN to the OAM system. From Rel-18, MDT may also be used in order to collect data for the purpose of AI/ML model training, e.g. for the use cases standardized as part of AI/ML for NG-RAN (cf. TR 37.817 (Rel-17), TR 38.743 (Rel-19)). Therefore, RAN3 should study how data should be collected and transported for the support of AI/ML (as well as SON) for 6G RAN.

Proposal 8: RAN3 studies efficient data collection for centralized SON and AI/ML for RAN use cases, considering 5G-A mechanisms such as MDT as a baseline.

# 3	Conclusion

This paper has discussed key principles for overall RAN architecture and has made the following observations and proposed the following requirements:

Observation 1: Different options in 5G, lead to market fragmentation and slow uptake of 5G SA and new features.

Observation 2: 5G work on CU-DU lead to extra standardization effort with no multivendor deployments.

Observation 3: 5G covered several features (IAB, relays etc.) with no clear commercial viability and which took away focus from features of true commercial needs.

Proposal 1: RAN3 to ensure a simple 6G RAN architecture design and elimination of support of multiple options.

Proposal 2: A standalone 6G RAN supports the 6GR RAT only and is connected to the 6G CN only.

Proposal 3: RAN3 to agree that there is no RAN3 impact from MRSS.

Observation 4: While CN resiliency was standardized in 5G, RAN resiliency still lacks standard support although it is one of the most critical components of the network infrastructure.

Proposal 4: RAN3 to study solutions for a robust and resilient 6G RAN.

Observation 5: Service/Application awareness refers to methods for RAN obtaining/being provided relevant information to handle an application or service.

Proposal 5: RAN3 to study mechanisms for 6G RAN to be made more service/application aware and allow real time provisioning of application/service characteristics.

Proposal 6: RAN3 to study mechanism for 6G RAN to be allow enhanced observability to ensure service assurance.

Proposal 7: 5G-A framework for AI/ML support in RAN to be taken as baseline.

Proposal 8: RAN3 studies efficient data collection for centralized SON and AI/ML for RAN use cases, considering 5G-A mechanisms such as MDT as a baseline.

# 4	References

- RP-252912, 6G RAN study

- RP-250810, Study Item Description: Study on 6G Scenarios and requirements, RAN#107 (April 2025)

- TR 38.914, Study on 6G Scenarios and requirements, v0.2.0 (RP-252882)

- R3-256557, Discussion on 6G RAN Functions and Logical Architecture, Nokia

- R3-257102, AI/ML Use Cases for 6G, Nokia

- RP-251866, New WID on XR (eXtended Reality) for NR Phase 4, RAN#108

- R3-256557, “Resilient 6G RAN Architecture,” Nokia