3GPP TSG-RAN WG3 Meeting #129bis	R3-256712

Prague, Czech Republic, 13th – 17th October, 2025

Agenda Item:	13.3

Source: 	OPPO

Title:  	Discussion on RAN3 impact of gNB-based mono-static sensing

Document for:	Discussion and Decision

- Introduction

In Rel-20 5G-A ISAC SID [1], the objectives led by RAN3 are captured as follows.

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution would like to provide our considerations on these objectives.

- Discussion

UAV use cases

According to the Rel-20 5G-A ISAC SID [1], the scopes of Rel-20 5G-A ISAC only include gNB-based mono-static sensing for the use case of UAV sensing target.

In practical scenario, UAV regulation is essential, such as identifying unauthorized UAVs, where the sensing tasks include 1/ to detect the UAV existence (e.g., for intrusion detection), and 2/ to track the UAV flying (e.g., for collision detection and warning).

- The use cases of UAV sensing target include 1/ to detect the UAV existence (e.g., for intrusion detection), and 2/ to track the UAV flying (e.g., for collision detection and warning).

Based on TR 22.837 and TS 22.137, the requirements of detection and/or tracking of UAV are quite different from each other, which may lead to different design on the network architecture and its procedures and signalling.

For example,

1/ For the use case of UAV detection, we only expect to identify the presence of targets, and optionally determine the number of existing targets in multi-target scenario. In this case, event-based or periodical reporting of sensing measurement results from gNB to Sensing Function is sufficient, and it seems no significant issue from each WGs.

2/ However, for the use case of UAV tracking, on top of UAV detection, we further expect to keep tracking the UAV flying continuously, and identify and distinguish a certain UAV target uniquely in multi-target scenario. In this case, even though the identification of non-3GPP target could be left to application layer via target feature exposure, the support of sensing service continuity might be required further, where all these issues need to coordinate with SA2. Besides, the feasibility of UAV tracking also depends on the evaluation of RAN1.

- The requirements of UAV detection and UAV tracking are quite different from each other, which may lead to different design on the network architecture and its procedures and signalling.

Therefore, we understand that the UAV use case in SID should be further clarified clearer.

- RAN3 assume that the UAV use cases at least include UAV detection. The support of UAV tracking is up to further progress of RAN1/SA2.

How to transfer sensing signalling between RAN and SF

As for the measurements and relevant measurement quantization, sensing measurements can be systematically acquired at fourth hierarchical stages of the signal processing pipeline, which are discussing in RAN1.

- First, full channel state information (CSI) is estimated as the foundational input for sensing operations.

- Second, delay/doppler/angle profiles are generated via transform-domain processing (e.g., FFT/IFFT).

- Third, intermediate sensing results, e.g., point cloud information, are obtained by further processing such as filter/compression.

- Fourth, detection algorithms extract refined target parameters or final sensing results.

All of the measurement results obtained from these fourth stages can be listed as candidate measurements and reporting quantities, but these measurement quantities may cause significant signalling overhead, especially for the first three types of measurement quantities, necessitating efficient compression or reporting methods.

- Significant signalling overhead for sensing measurement reporting is foreseen, especially when full channel state information, delay/doppler/angle profiles, or point cloud information are reported.

Therefore, one open issue is how to transfer sensing signalling between RAN node and SF considering the potential heavy signalling overhead due to sensing measurement reporting.

Considering that sensing and positioning are somehow similar, where positioning aims to obtain the location/velocity/direction of UE, sensing aims to obtain the sensing info at least including location/velocity/direction of non-UE object (i.e., UAV sensing target), NRPPa protocol designed for positioning signalling between gNB and LMF could be used to analyse when we start to design sensing procedures and signalling between RAN and SF.

In legacy, NRPPa signalling is carried by NGAP signalling via the NG control plane interface, i.e., relying on AMF to forward the NRPPa signalling from/to RAN node to/from LMF. The protocol stack is shown as below.

The transfer of NRPPa signalling for positioning does not consider the enhancement of heavy signalling overhead. For sensing, if we still rely on AMF to forward sensing signalling, AMF would be a bottleneck for signalling transfer between RAN node and SF, since it would cause significant signalling overhead for AMF. So legacy manner to transfer NRPPa signalling via AMF over CP is not applicable for sensing.

- If relying on AMF to forward sensing signalling over CP, AMF would be a bottleneck for heavy signalling transfer between RAN node and SF.

As captured in SA2 Rel-20 ISAC TR 23.700-14 [2], some UP-like solutions are proposed, as shown below, where all of them aim to directly transfer sensing signalling between RAN node and SF without UPF in-between, and the difference is which protocol stack the sensing signalling is carried through for transfer.

- Based on SA2 Rel-20 ISAC TR 23.700-14, it seems that common view of SA2 is that sensing data and the associated information is transported via direct connection between RAN node and SF in the core network.

1/ GTP-U protocol

In this solution, sensing signalling is transferred between RAN node and SF via direct UP data tunnel, i.e., GTP-U protocol. It is similar to NG UP interface between RAN node and UPF, which is designed especially for large volume UP data transfer.

However, GTP tunnel is designed for UP data transfer from/to UE. For example, for uplink UP data transfer, RAN node relies on the mapping between QoS flow and GTP tunnel to encapsulate the uplink UP data received from UE into the corresponding GTP tunnel to send to the correct UPF, where RAN node obtains the mapping between QoS flow and GTP tunnel from SMF via AMF, i.e., via NGAP PDU SESSION RESOURCE SETUP REQUEST message. If we extend the GTP-U protocol stack to transfer sensing signalling generated by RAN node without association of UE, more RAN3 effort would be needed to design the new process procedures to allow this.

2/ sensing UP protocol (like LCS-UPP)

In this solution, sensing signalling is transferred between RAN node and SF via a new sensing UP layer, which is like LCS-UPP protocol. The advantage is that consistent protocol stack for data collection from UE and data collection from gNB, thus it is more future-proof for 6G when UE based sensing is developed.

Similar to LCS-UPP, the main work for this solution is in CT to develop a new protocol, while the problem is no such design in CT1/CT4 for non-UE related connection, and not sure if there will be enough TU to develop a completely new protocol in CT WGs within 5G-A timeframe.

Furthermore, RAN3 impact is also expected, e.g., the data connection establishment procedure carried out on CP signalling interaction such as non-UE associated NGAP signalling.

Considering to be more future-proof for 6G, we prefer sensing signalling over a new LCS-UPP-like sensing UP protocol designed by SA/CT, since this new sensing UP protocol can easily extend to also support the sensing signalling between UE and SF in a unified manner in 6G.

- RAN3 study RAN3 impact for candidate solutions to transfer sensing signalling via direct connection between RAN node and SF, in coordination with SA2.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

UAV use cases

- The use cases of UAV sensing target include 1/ to detect the UAV existence (e.g., for intrusion detection), and 2/ to track the UAV flying (e.g., for collision detection and warning).

- The requirements of UAV detection and UAV tracking are quite different from each other, which may lead to different design on the network architecture and its procedures and signalling.

- RAN3 assume that the UAV use cases at least include UAV detection. The support of UAV tracking is up to further progress of RAN1/SA2.

How to transfer sensing signalling between RAN and SF

- Significant signalling overhead for sensing measurement reporting is foreseen, especially when full channel state information, delay/doppler/angle profiles, or point cloud information are reported.

- If relying on AMF to forward sensing signalling over CP, AMF would be a bottleneck for heavy signalling transfer between RAN node and SF.

- Based on SA2 Rel-20 ISAC TR 23.700-14, it seems that common view of SA2 is that sensing data and the associated information is transported via direct connection between RAN node and SF in the core network.

- RAN3 study RAN3 impact for candidate solutions to transfer sensing signalling via direct connection between RAN node and SF, in coordination with SA2.

4. Reference

[1] RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR, AT&T

[2] 3GPP TR 23.700-14: “Study on Integrated Sensing and Communication; Stage 2”, V1.0.0 (2025-09)