3GPP TSG-RAN WG3 Meeting #129bis	R3-256730

Prague, Czech Republic, 13-17 October, 2025

Agenda item:	9.2.12

Source: 	Huawei

Title: 	Discussion on RLC enhancement for XR

Document for:	Discussion and Decision

# 1. Introduction

In Release 19, RAN3 specified enhancement in F1-U interface to support the RLC enhancement including avoiding unnecessary retransmission and the timely retransmission/polling. In this contribution, we will discuss some clean-up issues regarding RLC enhancements and give our views.

# 2. Discussion

### 2.1 Avoid unnecessary retransmission

To avoid DL unnecessary RLC retransmission, RAN3 has enhanced F1-U to enable the gNB-DU to stop transmission or retransmission of the outdated SDUs, as captured in the TS 38.425 [1].

------ Excerption from TS 38.425------ The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs. The corresponding node may stop transmission or retransmission of the NR PDCP PDU as indicated in the DL discard NR PDCP PDU SN, or stop transmission or retransmission of one or a number of blocks of downlink NR PDCP PDUs, as specified in TS 38.322 [7].

As the highlighted sentences stated, the gNB-DU handles the indicated DL discard NR PDCP PDU SN differently when performing the legacy discard behaviour and the newly defined transmission/retransmission stop behaviour. When the gNB-DU decides to discard as legacy, it discards all PDCP PDUs up to and including the DL discard NR PDCP PDU SN, while it only stops transmission/retransmission for the DL discard NR PDCP PDU SN in the newly defined behaviour. This was due to some concerns on PSI based discard. When DL PSI based discard is enabled, it may appear that the later packets with higher SN expire before the earlier packets with lower SN, so that the gNB-DU cannot treat all packets up to the highest discarded SN as outdated. However, this issue exists for both the legacy behaviour and the new behaviour. Actually, when such case appears, the gNB-CU should not use DL discard NR PDCP PDU SN to indicate the outdated packets, it can indicate the packets via multiple blocks instead.

Thus, there is no need to capture the gNB-DU’s understanding on the indicated DL discard NR PDCP PDU SN in different way for the legacy discard behaviour and the newly defined behaviour for stop (re-)transmission. To make the specification more concise, the procedure text can be further simplified as following.

------ Excerption from TS 38.425 ------ The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs. The corresponding node may stop transmission or retransmission of the NR PDCP PDUs indicated to be discarded, as specified in TS 38.322 [7].

Proposal 1: The gNB-CU is able to indicate the discarded PDCP PDUs properly when PSI based discard is enabled. The F1-U procedure text can be simplified as “The corresponding node may stop transmission or retransmission of the NR PDCP PDUs indicated to be discarded”.

### 2.2 Timely retransmission

To support DL timely retransmission, RAN3 has agreed to introduce F1-U indications to indicate the SN of the PDCP PDUs for which the remaining time falls below the threshold for remaining-time-based polling. The following content in TS 38.425 shows the agreed changes.

------ Excerption from TS 38.425------ #### 5.5.2.1 DL USER DATA (PDU Type 0) This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU. The following shows the respective DL USER DATA frame. NOTE 1: All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP. Bits; Number of Octets. 7; 6; 5; 4; 3; 2; 1; 0. PDU Type (=0); Spare; DL Discard Blocks; DL Flush; Report polling; 1. New IE Flag; BSSI; TTNBI; Request OutofSeq Report; Report Delivered; User data existence flag; Assistance Info. Report Polling Flag; Retransmission flag; 1. NR-U Sequence Number; 3. DL discard NR PDCP PDU SN; 0 or 3. DL discard Number of blocks; 0 or 1. DL discard NR PDCP PDU SN start (first block); 0 or 3. Discarded Block size (first block); 0 or 1. …. DL discard NR PDCP PDU SN start (last block); 0 or 3. Discarded Block size (last block); 0 or 1. DL report NR PDCP PDU SN; 0 or 3. BSSize; 0 or 3. TTNB; 0 or 2. New IE Flag 7(E); New IE Flag 6; New IE Flag 5; New IE Flag 4; New IE Flag 3; New IE Flag 2; New IE Flag 1; New IE Flag 0; 0 or 1 New IE Flags Octet. DL retransmission NR PDCP PDU SN; 0 or 3. DL retransmission Number of blocks; 0 or 1. DL retransmission NR PDCP PDU SN start (first block); 0 or 3. Retransmission Block size (first block); 0 or 1. …. DL retransmission NR PDCP PDU SN start (last block); 0 or 3. Retransmission Block size (last block); 0 or 1. DL polling NR PDCP PDU SN; 0 or 3. DL polling Number of blocks; 0 or 1. DL polling NR PDCP PDU SN start (first block); 0 or 3. Polling Block size (first block); 0 or 1. …. DL polling NR PDCP PDU SN start (last block); 0 or 3. Polling Block size (last block); 0 or 1. Padding; 0-3. Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

The highlighted parts are used to indicate the gNB-DU to consider remaining-time-based polling for these PDCP PDUs. For the polling, we think the information over F1-U interface can be simplified, because the gNB-DU does not need to know the specific SNs of the PDUs triggering polling. According to the latest RLC running CR agreed by RAN2 in R2-2505402 [2], the RLC entity includes a poll in the AMD PDU if the condition for remaining-time-based polling has been met, where the poll field is only 1 bit with no specific SN information.

------ Excerption from R2-2505402 ------ Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); or - if an indication is received from upper layer (e.g., PDCP) that the condition for remaining-time-based RLC polling has been met for an RLC SDU: - include a poll in the AMD PDU as described below.

------ Excerption from TS 38.322[3] ------ #### 6.2.3.7 Polling bit (P) field Length: 1 bit. The P field indicates whether or not the transmitting side of an AM RLC entity requests a STATUS report from its peer AM RLC entity. The interpretation of the P field is provided in Table 6.2.3.7-1. Table 6.2.3.7-1: P field interpretation Value; Description. 0; Status report not requested. 1; Status report is requested.

Thus, for the F1-U indication for DL remaining-time-based RLC polling, we think there is no need to transfer all the PDCP PDU SNs. To minimize the overhead, one bit is enough to indicate to the gNB-DU that the condition for remaining-time-based polling is met, i.e., there is at least one SDU which has been submitted to the gNB-DU but the successful delivery has not been confirmed and its remaining time is below the threshold. Then the gNB-DU can decide to send the poll over the air interface if there is any transmitted but unacknowledged SDU. Corresponding CR for the change can be found in R3-256731 [4].

Proposal 2: RAN3 to use one-bit F1-U indication to indicate the condition for remaining-time-based polling is met.

# 3. Conclusion

In this contribution, we analyzed the RAN3 impacts to support the RLC enhancements discussed in R19. The following proposals were made:

Proposal 1: The gNB-CU is able to indicate the discarded PDCP PDUs properly when PSI based discard is enabled. The F1-U procedure text can be simplified as “The corresponding node may stop transmission or retransmission of the NR PDCP PDUs indicated to be discarded”.

Proposal 2: RAN3 to use one-bit F1-U indication to indicate the condition for remaining-time-based polling is met.

# 4. Reference

[1]. 3GPP TS 38.425: NG-RAN; NR user plane protocol (Release 19)

[2]. R2-2505402, vivo, Introduction of R19 XR enhancements for RLC spec.

[3]. 3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[4]. R3-256731, Huawei, Correction on RLC enhancement for XR.