3GPP TSG-RAN WG3 Meeting #129bis			R3-256736

Prague, Czech Republic, 13th-17th October 2025

Agenda item:	10.4.

Source:	Fujitsu

Title:	Initial views on 6G RAN internal functions split

Document for:	Discussion

Background

In 6G WG SID [1], the following objectives are captured.

Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, our initial view on RAN internal function split is provided.

Discussion of RAN internal functions and split

RAN function split

RAN3 and RAN2 will study “RAN internal functional split, interfaces, protocol stacks and procedures” as mentioned in 6G SID [1] and it was agreed to capture requirements for RAN architecture and migration to TR 38.914 [2] in RAN#109 meeting. According to the requirements, 6G RAN should support open interface (IF) for multi-vendor interoperability. Based on this, RAN3 should consider maintaining the current 5G inter-operable open IFs with potential enhancements.

In 5G NR, there are two split IFs in the gNB, i.e., E1 and F1 IFs. For the IF between 6GR CU-CP and CU-UP (E1-like IF), general necessity has already been justified based on the requirement of C-plane and U-plane separation approved in RAN#109. Separation of CU-CP and CU-UP provides MNOs with benefit from separate dimensioning of the network elements, pooling gains of the deployed capacity leading to improved resource utilization and minimization of disruption of service when fault occurs in the system. Further, the distributed split RAN architecture with separation of CP and UP encourages MNOs to select cloud-based implementation of network and that allows more flexibility and reduction of deployment costs. Thus, E1-like IF can be beneficial in 6G NW, which is assumed to be cloudified RAN systems. As an additional aspect, since the discussion on CP and UP separation is related with radio protocol discussion in RAN2, discussion on the design of E1-like IF should be coordinated with RAN2.

For the IF between 6GR CU and DU (F1-like IF), multiple companies asked to discuss if the IF is actually useful. There is a demand of multi-vendor interoperability of the IF in commercial market, which is also supported by the test specification development in O-RAN alliance. As O-RAN alliance will consider 6G enhancements of their specification based on the current open IFs, the F1-like IF is better to be kept in terms of the coordination with external standards organization.

Regarding the IF between 6GR gNBs (Xn-like IF), as the inter-base station (BS) coordination is essential functions e.g., for mobility and various self-optimization methods, Xn-like IF should be also supported in 6G. Thus, all of the open IFs already defined in 5G NR can be considered as necessary IFs in 6G RAN.

Proposal 1: 6G RAN architecture should support the existing RAN functional splits in 5G NR.

Potential enhancement on RAN architecture

While keeping the current open IFs, 6G RAN should provide better flexibility and scalability to allow operators to further reduce their OPEX/CAPEX compared with 5G. For that purpose, RAN3 should study potential enhancements on RAN architecture.

As an example, the CU-DU connection topology can be discussed. Figure 1 (a) illustrates the existing topology of the CU-DU connection. A single CU can connect to multiple DUs and the CU can control the DUs by a centralized manner. In the topology, coordination between multiple CUs is often required e.g., for inter-CU LTM, one CU needs to transfer necessary information (e.g., candidate cell list) from its own DU to other DUs via corresponding CUs. This means that inter-CU LTM needs more network signalling compared with intra-CU LTM.

On the other hand, as illustrated in Figure 1 (b), when a single DU can be connected to multiple CUs, due to increase of connections between CU and DU, coordination between multiple CUs is required less frequently. Considering the example of LTM, a UE is more likely to remain in the source CU when it changes the DU. Therefore, NW and UEs have more opportunities to conduct intra-CU LTM instead of inter-CU LTM, which means more efficient LTM operation can be achieved.

As another potential use case of the structure that a single DU connects with multiple CUs, RAN sharing can be considered. As shown in Figure 2 (a), in the 5G specification, MOCN based RAN sharing is specified as one of the architectures. In MOCN, both CU and DU are shared among different operators. Policy of each MNO can be provided from the core network to the shared CU. In this deployment, configurability of the service requirement for UEs is limited to information such as QoS requirement like 5QI. On the other hand, in case of the DU level sharing (Figure 2 (b)), since MNO owns their CU, it is possible for CU to e.g., select an appropriate DU (among dedicated DUs and shared DUs) to send data to a UE based on e.g., measurement report from the UE. It also provides seamless mobility between shared and non-shared DUs compared with MOCN (where only NG based mobility is possible). Thus, more flexible topology between CU and DU should be discussed further.

Figure 1 topology of CU-DU connection

Figure 2 RAN sharing topology

Proposal 2: RAN3 should study potential enhancements (e.g., CU-DU connection topology), taking the 5G RAN IF specification as the starting point.

Service-based RAN

Service-based RAN, where RAN functionalities are mutually connected via service-based IF (SBI) could be beneficial since it could provide scalability in the cloudified CU and DU deployment, leading to CAPEX reduction.

Service-based RAN can also be beneficial for new functionalities (e.g., AI/ML, sensing). Such functionalities can be defined as independent functions in service-based RAN, which allows MNOs to flexibly select features they deploy. In addition, since the data related to the feature can reside on corresponding function, accessibility to the data from core NW function can be enhanced. Thus, potential benefit of service-based RAN architecture should be studied first.

Proposal 3: RAN3 should study potential benefit of service-based RAN architecture.

Conclusion

From the above discussion, we propose the followings:

Proposal 1: 6G RAN architecture should support the existing RAN functional splits in 5G NR.

Proposal 2: RAN3 should study potential enhancements (e.g., CU-DU connection topology), taking the 5G RAN IF specification as the starting point.

Proposal 3: RAN3 should study potential benefit of service-based RAN architecture.

- References

[1] RP-252912 Revised SID: Study on 6G Radio;

[2] RP-252882 3GPP TR 38.914.