3GPP TSG-RAN WG3 Meeting #129 bis	             R3-256851

Prague, Czech, 13th – 17th Oct 2025

Agenda Item:	9.2.11

Source:	CMCC

Title:	Discussion on NW side data collection

Document for:	Discussion

# Introduction

RAN2 provides the following information based on agreements achieved in RAN2 in Rel 19 [1]:

For NW side data collection on AI/ML based beam management, the agreements present in Annex have been achieved in Rel-19 for which RAN2 assumes those agreements may have RAN1 and RAN3 impact. For RAN1 to take into account: - The resource configuration for logged NW side data collection is provided with CSI-ResourceConfig in CSI-MeasConfig - The resource configuration does not have separate resources for Set A and Set B - RAN2 assumes there is no separate CSI resource capability is introduced specifically for logged NW side data collection, that is, the legacy CSI resource capability (e.g. beamManagementSSB-CSI-RS) would be reused. - The list of logging configurations is provided as a new list configurations within CSI-MeasConfig - the data logging procedure based on receiving measurement quantities from lower layer is captured in RRC spec. - Logging periodicity, which is different from CSI resource periodicity, is configurable for NW side data collection. - The L3 event (i.e. Event A1 or Event A2) configuration is contained in the logging configuration to support the event based data logging. - For event based data logging, UE shall perform and log the L1 measurement if the entering condition of configured L3 event is met; UE should not perform and log the L1 measurement if the leaving condition of configured L3 event is met. - From RAN2 point of view, it is sufficient to collect the L1-RSRP and/or Beam ID for the logging contents. - Only periodic CSI resources are used for NW side data collection. For RAN3 to take into account: - The list of logging configurations is provided as a new list configurations within CSI-MeasConfig. - the data logging procedure based on receiving measurement quantities from lower layer is captured in RRC spec. - The L3 event (i.e. Event A1 or Event A2) configuration is contained in the logging configuration to support the event based data logging. - The logged data is reported via UEInformationRequest/UEInformationResponse. - The UE can send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The NW side data collection configuration should be released if UE reports the lower power indication to the NW via UAI.

# Discussions

Based on [1], the feature is specified in RRC as follows:

- Logging configuration is provided under CSI-MeasConfig; only periodic CSI resources are used; no separate Set-A/Set-B resources; legacy CSI capability is reused.

- Event-based logging uses L3 events A1/A2 with hysteresis/threshold/TTT; UE logs L1 measurements on entering condition and not on leaving condition.

- Minimum logged content is L1-RSRP and/or Beam ID.

- Reporting uses UEInformationRequest/UEInformationResponse; UE Assistance Information (UAI) may indicate data availability, trigger reason, and low power; data-collection configuration is released on low-power indication and when the UE transitions to IDLE/INACTIVE or re-establishes (incl. RLF).

- Multiplexing of SON/MDT with AI/ML logged data in the same UE information response is not supported.

- Data forwarding to OAM or source gNB after HO is out of scope.

The agreement that "Data forwarding to OAM or source gNB after HO is not in RAN2 scope" is noted and it is understood that this functionality is not planned for Release 19.

In line with this, the expected network behaviour regarding the AI/ML logged data after a handover:

- The logged data collected in the source gNB is not forwarded to the target gNB during handover.

- The UE discards the logged data upon successful handover completion (as per the RAN2 agreement for inter-RAT HO, which RAN3 assumes applies to intra-RAT HO for consistency).

Therefore, if the network wishes to obtain the logged data, the UEInformationRequest must be sent by the source gNB before the handover execution is triggered. RAN3 will not specify any procedures in Release 19 for transferring this logged data between gNBs (via Xn) or to the OAM after a handover has occurred.

RAN3 will not specify any procedures in Release 19 for transferring the AI/ML logged data between gNBs (via Xn) or to the OAM after a handover has occurred.

RAN3 confirms no impact on existing handover procedures beyond the data discard behaviour already agreed by RAN2.

From RAN3’s perspective, since network-side data collection is specified at the RRC level and logged data is reported via UEInformationRequest/UEInformationResponse, the data always terminates at the CU-CP.

- If AI model training and inference are executed at the CU, no F1 interface impact arises, because the CU directly consumes the logged data.

- If AI model training/inference are executed at the DU, then the CU would need to forward logged data received in UEInformationResponse to the DU. In this case, new F1 procedures or extensions would be required to support transfer of logged data from CU → DU.

The impact on the F1 interface depends on deployment: when AI/ML runs at the CU no new signalling is needed, whereas DU-based AI/ML would require extensions for data forwarding.

# Conclusions

In this submission, we have discussion on network side data collection. The following proposals are made:

- RAN3 will not specify any procedures in Release 19 for transferring the AI/ML logged data between gNBs (via Xn) or to the OAM after a handover has occurred.

RAN3 confirms no impact on existing handover procedures beyond the data discard behaviour already agreed by RAN2.

The impact on the F1 interface depends on deployment: when AI/ML runs at the CU no new signalling is needed, whereas DU-based AI/ML would require extensions for data forwarding.

# References

R3-256511, LS to RAN1 and RAN3 on NW side data collection