3GPP TSG-RAN WG3 Meeting #129bis	R3-257195

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.1

Source:	Nokia

Title:	Response to R3-256747, R3-257070

Document for:	Response

# 1	Introduction

We here provide response to R3-256747 [1] and R3-257070 [2] and provide analysis of introduction of state management for Trace Sessions following SA5’s LS [3].  This corresponds to the scope of the TraceJob IOC. State management for trace sessions does not as such require impact on RAN3-defined network interfaces, and we also discuss this aspect in the present paper.

# 2	Discussion

The trace mechanism is specified in TS 32.422. For the present analysis, limited to the 5GS, we consider the following steps for activation of trace relative to a UE:

- Step 1: Start a Trace Session: This includes instantiation of a TraceJob object (details in TS 28.623), including allocation of Trace Reference (TR). This is done by OAM.

- Step 2: Corresponding SA5-defined Trace Session activation signalling between OAM and CN, or between OAM and NG-RAN.

- Step 3: Start one or more Trace Recording Sessions for the Trace Session. This includes selecting UE(s) and allocate a Trace Recording Session Reference (TRSR) per Trace Recording Session. In case of signalling-based activation this step is done by the CN which will use the TraceTarget information within the TraceJob IOC (traceTargetType, traceTargetValueList) to select the context of the UE that is targeted by the OAM system. In case of management-based activation, this step is done by the NG-RAN.

- Step 4: Corresponding CT4-defined and/or RAN3-defined Trace Recording Session activation signalling (e.g. NGAP Trace Activation IE).

- Step 5: Collect requested trace or measurements for the UE, or from the UE (may require e.g. specific RRC configuration).

Similarly, we consider in this analysis the following steps for Trace Session deactivation:

- Step 11: OAM decides to deactivate the Trace Session

- Step 12: Corresponding SA5-defined Trace Session deactivation signalling between OAM and CN, or between OAM and NG-RAN.

- Step 13: Stop ongoing Trace Recording Sessions corresponding to the Trace Session. In case of signalling-based trace this step is done by the CN. In case of management-based trace, this step is done by the NG-RAN.

- Step 14: Corresponding CT4-defined and/or RAN3-defined Trace Recording Session deactivation signalling (e.g. NGAP TRACE STOP message, or similar on other RAN3-defined interfaces).

- Step 15: Stop collecting trace or measurements.

SA5 indicates in [3] that the scenario justifying the new Trace Session states (administrativeState, operationalState) relates to temporary halt of trace jobs (Trace Sessions), in order to e.g. preserve the job configurations during temporary stoppages or reduce operational overhead. From NG-RAN perspective we believe that the essential part of reduction of operational overhead is to preserve information related to the Trace Recording Sessions associated to the suspended (temporarily halted) Trace Session, and in particular the mapping between TRSR and UE id which is used in the TCE for correlation of received Trace Records. Preservation of e.g. measurements configured at the UE would in our view not represent a benefit – rather it seems beneficial that involved entities, including UEs, will not collect or report data corresponding to suspended Trace Session.

Observation 1: It is beneficial to preserve mapping between TRSR and UE id for Trace Recording Sessions corresponding to suspended Trace sessions.

Observation 2: It is beneficial that involved entities, including UEs, will not collect or report data for Trace Recording Sessions corresponding to suspended Trace Sessions.

Based on this, we believe the following steps would be used for Trace Session suspension:

- Step A: OAM decides to suspend the Trace Session and updates the TraceJob object

- Step B: Corresponding SA5-defined Trace Session suspension signalling between OAM and CN, or between OAM and NG-RAN

- Step C: Suspend ongoing Trace Recording Sessions corresponding to the Trace Session. In case of signalling-based trace this step is done by the CN. In case of management-based trace, this step is done by the NG-RAN.

- Step D: Reuse legacy CT4-defined and/or RAN3-defined Trace Recording Session deactivation signalling (e.g. NGAP TRACE STOP message, or similar on other RAN3-defined interfaces).

- Step F: Stop collecting trace or measurements (same as legacy).

Trace session resume would then become:

- Step G: OAM decides to resume the Trace Session

- Step H: Corresponding SA5-defined Trace Session resume signalling between OAM and CN, or between OAM and NG-RAN.

- Step I: Resume ongoing Trace Recording Sessions corresponding to the Trace Session. In case of signalling-based trace this step is done by the CN. In case of management-based trace, this step is done by the NG-RAN.

- Step J: Reuse legacy CT4-defined and/or RAN3-defined Trace Recording Session activation signalling (e.g. NGAP TRACE START message, or similar on other RAN3-defined interfaces).

- Step K: Start collecting trace or measurements (same as legacy).

This solution would provide the benefits indicated in observations 1 and 2 above. From RAN3 point of view we can therefore reply to SA5 that “RAN3 doesn’t foresee stage 3 impact in their specifications”.

Proposal 1: Reply to SA5 that RAN3 doesn’t foresee stage 3 impact in their specifications relative to support of state management for TraceJob IOC (administrativeState and operationalState attributes).

Concerning logged MDT we reach same conclusion as [1]. Furthermore, relative to feasibility, there is no stop mechanism for logged MDT sessions already configured at the UE. When a UE enters RRC_CONNECTED indicating availability of logged measurements, the gNB can request reporting of this information for transparent forwarding to the TCE. If such reporting is not requested, the UE will indicate availability of logged measurements each time it is served by a new gNB (the log expires in the UE after 48 hours). There is no information available in the gNB that would prevent such request to be made, and there is no mechanism enabling the gNB to request the UE to discard logged measurements and measurement configuration. Suspension of logged MDT is therefore not feasible.

Proposal 2: Reply to SA5 that suspension of logged MDT is not feasible.

# 3	Conclusion

We have made the following observations and proposal:

Observation 1: It is beneficial to preserve mapping between TRSR and UE id for Trace Recording Sessions corresponding to suspended Trace sessions.

Observation 2: It is beneficial that involved entities, including UEs, will not collect or report data for c Trace Recording Sessions corresponding to suspended Trace Sessions.

Proposal 1: Reply to SA5 that RAN3 doesn’t foresee stage 3 impact in their specifications relative to support of state management for TraceJob IOC (administrativeState and operationalState attributes).

Proposal 2: Reply to SA5 that suspension of logged MDT is not feasible.

# References

[1]		R3-256747, Temporary suspension of trace production, ZTE

[2]		R3-257070, Discussion on Trace Suspension feasibility, Ericsson

[3]		R3-256519 - S5-253909, LS on temporary suspension of trace production, SA5#162

# Annex – Draft reply LS to SA5 cc RAN2

3GPP TSG RAN WG3 Meeting #129bis	R3-25xxxx

Prague, Czech Republic, 13 – 17 October, 2025

Title:	[Draft] Reply LS on temporary suspension of trace production

Response to:	R3-256519 / S5-253909

Release:	Rel-19

Work Item:	TraceQoE_OAM

Source:	Nokia [to be RAN3]

To:	SA5

Cc:	RAN2

Contact Person:

#### Name:	Håkon Helmers

E-mail Address:	hakon.helmers@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall Description

RAN3 would like to thank SA5 for their LS on temporary suspension of trace production.

In the LS, SA5 provides motivation to introduce administrativeState and operationalState attributes in the TraceJob IOC, e.g. operators currently must delete and recreate trace jobs to pause/resume them which causes operational inefficiencies. SA5 asks RAN3 to evaluate the feasibility of adding such attributes in the TraceJob IOC.

RAN3 has discussed enabling support of suspension/resumption of trace in RAN. RAN3 assumes the described operational inefficiencies are resolved if mapping between TRSR and UE id for Trace Recording Sessions corresponding to suspended Trace sessions is preserved. RAN3 also believes it is beneficial that involved entities, including UEs, will not collect or report data for Trace Recording Sessions corresponding to suspended Trace Sessions. For trace and immediate MDT, RAN3 foresees the use of legacy network signalling for trace deactivation in trace suspension scenario and trace activation in trace resumption scenario. For trace and immediate MDT it is therefore possible to support state management for TraceJob IOC (administrativeState and operationalState attributes) without stage 3 impact in RAN3’s specification.

On the other side, there is no network signalling framework for deactivation of logged MDT. RAN3 would therefore like to inform SA5 that suspension of logged MDT is not feasible.

- Actions

To SA5

ACTION: RAN3 kindly asks SA5 to take the above information into account.

- Date of Next TSG-RAN3 Meetings

TSG-RAN WG3 Meeting RAN3#130			17 - 21 November 2025		Dallas, USA

TSG-RAN WG3 Meeting RAN3#131			09 - 13 February 2026			Göteborg, Sweden