3GPP TSG-RAN WG3 Meeting #130	R3-258116

Dallas, US, 17-21 Nov, 2025

Title: 	(TP for pCR) Network architecture for ISAC

Source: 	Huawei

Agenda item:	13.2

Document Type:	pCR

# 1. Introduction

The new study item on Study on Integrated Sensing and Communication (ISAC) for NR was agreed in [1]. For network architecture, the previous RAN3-129bis meeting made the following agreements, and the baseline pCR was agreed in [2].

Capture logical architecture for ISAC RAN3 focus on sensing protocol in coordination with SA2 To continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions.

This contribution provides our further views on the network architecture for gNB-based mono-static for UAV use case. The companion paper on the sensing procedures and signalling can be found in [3].

# 2. Discussion

## 2.1 Previous SA2 agreements on the system architecture

SA2 already initiated the study on ISAC, and captured the key issues as well as potential solutions in TR 23.700-14 [4]. Especially the key issue #1 is to study the System Architecture to Support Sensing. So far overall forty solutions were discussed in SA2, where multiples solutions proposed the RAN/CN network architecture covering the following two connection options between the gNB and the Sensing Function (Note that the exact terminology of sensing function would be finally decided by SA2).

- Option 1: Direct connection

- Option 2: Indirect connection by AMF

The latest previous SA2-171 meeting agreed the following principles.

### 7.1.1 Agreed Principles for KI#1: System Architecture to Support Sensing To support KI#1 System Architecture to Support Sensing, the following principles are concluded: Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary. Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result. Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE). NOTE 4: Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase. NOTE 5: Other KI's interim conclusions will be aligned with KI#1 conclusions.

From the above, SA2 agreed that the SF may contain the SCF and SPF functionality, and the gNB is the only entity as the sensing entity. Regarding the direct/indirect connection, there were no solid conclusion yet.

- Observation 1:  In SA2 key issue#1 on the system architecture for sensing, SA2 agreed that the SF may contain the SCF and SPF functionality, and the gNB is the only entity as the sensing entity. Regarding the direct/indirect connection, there were no solid conclusion yet.

## 2.2 Network architecture for gNB-based mono-static sensing

In the RAN3 agreed BL pCR [2], the logical architecture is given as follows, where the Nx interface is between the gNB and the SF, which needs further discussion and decision.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture. Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF. Figure 7.1 Logical architecture for ISAC This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF. Editor’s Note: The details of "Nx" interface need further discussion and decision.

So far, the SF is referring to nowhere in the BL pCR. In TS 23.700-14, SA2 already defines the Sensing Function as follows.

Sensing Function: Indicating the logical function which is involved to support Sensing Service. NOTE 2: The Sensing Function cannot be a Sensing Entity.

For SF, RAN3 can add the abbreviations of SF, and refer the SF definition to TR 23.700-14.

For SF (Sensing Function), RAN3 to add the reference to TR 23.700-14, and the abbreviation in the ISAC pCR.

About the direct or indirect connection between the gNB and the SF, it is expected that SA2 can make final conclusion in Nov meeting. From RAN3 perspective, it is better to leave the network architecture conclusion (including the direct connection and indirect connection) to SA2. This is not only beneficial to avoid any potential parallel work, but also allows RAN3 to concentrate on the RAN own aspects. While RAN3 focuses on the protocol stack between the gNB and the SF.

The direct connection and indirect connection between the gNB and the Sensing Function is left to SA2 conclusion.

For the interface between the gNB and SF, the Nx interface is captured in the BL pCR [2]. It needs to discuss whether Nx interface is the NG interface or a new interface.

First, the R19 Ambient IoT already went through very thorough analysis during the SI and WI phase, and finally concluded that the NG interface is reused. We consider that the ISAC architecture can take the same approach. This is not only very simple, but also avoid lengthy and worthless effort to redefine interface setup/configuration update etc procedures which are already defined over the NG interfaces. Hence RAN3 can update the Nx to NG in the logical architecture in the ISAC BL pCR, so that the NG interface is the interface between the gNB and SF.

RAN3 to update the Nx to NG in the logical architecture, so that the NG interface is the interface between the gNB and SF.

For the CP between the gNB and the SF, the sensing specific control plane procedures are transported via the NG-C interface. Below Figure depicts the control-plane protocol stack.

Figure 7.2 depicts the control-plane Protocol stack between the gNB and SF respectively. Figure 7.2. Control-plane Protocol Stack for sensing

RAN3 to capture the CP protocol stack, where the NGAP is used to transport the sensing signaling procedures.

RAN3 also needs to discuss whether to use CP or UP protocol to transfer the sensing report to the SF when the gNB accepts the sensing request from the SF. The decision is dependent on multiple factors, including the sensing data size, sensing report periodicity, gNB complexity, sensing measurements format etc. We consider that UP report is the feasible approach, while the detailed analysis could be found in [3]. For the UP between the gNB and the SF, the sensing specific data is transported via the NG-U interface. Below Figure depicts the user-plane protocol stack for ISAC.

Figure 7.3. User-plane Protocol Stack for sensing

RAN3 to capture the UP-protocol stack, where the GTP-U is to deliver the sensing data, if UP report is agreed, between the gNB and the SF.

For the RAN split architecture for ISAC, we consider that there is no need to study it during the study item. First this is not covered in the RAN SID [1], second, it is not important considering the fact that the RAN split architecture can be studied during the normative phase when the whole RAN/CN signalling procedures are clear, or can be left out totally.

No need to study the ISAC for RAN split architecture during the study item.

# 3. Conclusion

Based on the discussion in this paper, we make the following observations and proposals.

- SA2 progress on the system architecture

- Observation 1:  In SA2 key issue#1 on the system architecture for sensing, SA2 agreed that the SF may contain the SCF and SPF functionality, and the gNB is the only entity as the sensing entity. Regarding the direct/indirect connection, there were no solid conclusion yet.

- Network architecture for gNB-based mono-static sensing

- For SF (Sensing Function), RAN3 to add the reference to TR 23.700-14, and the abbreviation in the ISAC pCR.

- The direct connection and indirect connection between the gNB and the Sensing Function is left to SA2 conclusion.

- RAN3 to update the Nx to NG in the logical architecture, so that the NG interface is the interface between the gNB and SF.

- RAN3 to capture the CP protocol stack, where the NGAP is used to transport the sensing signaling procedures.

- RAN3 to capture the UP-protocol stack, where the GTP-U is to deliver the sensing data, if UP report is agreed, between the gNB and the SF.

- No need to study the ISAC for RAN split architecture during the study item.

The Annex 5 provides the TP on the network architecture for the sensing.

# 4. Reference

- RP-251861 New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- R3-258088	(pCR to TR 38.765) Study on Integrated Sensing and Communication (ISAC) for NR

- R3-258117	(TP for pCR) RAN-CN procedures and signaling for ISAC	Huawei

- TR 23.700-14 Study on Integrated Sensing and Communication

# 5. Annex – TP to ISAC pCR against R3-257332

CHANGES START

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-252819: "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

[3]	3GPP TR 23.700-14: "Study on Integrated Sensing and Communication".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Sensing Function: as defined in TR 23.700-14 [3].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Sensing Function 	SF

# 7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the NG interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Figure 7.2 depicts the control-plane Protocol stack between the gNB and SF.

Figure 7.2. Control-plane Protocol Stack for sensing

Figure 7.3 depicts the user-plane Protocol stack between the gNB and SF.

Figure 7.3. User-plane Protocol Stack for sensing

CHANGES END